- 14-51.043 Customized Place Name Signs.

(1) No place name signs within limited access facilities will be permitted for custom treatment, other than the addition of a pictograph to the standard white-on-green sign.

(2) On limited access rights-of-way, no customized treatment may be constructed to stand individually, or as an assembly, such that it resembles a place name sign, nor shall a place name be included as part of any landscape structure or design within limited access rights of way.

(3) Customized treatment of county, municipal, sovereign lands, and unincorporated area place name signs shall be considered only on non-limited access facilities.

(4) The preferred location of all customized place name signs is off the state highway system right of way, where increased lateral clearance can be used. In such instances, the property owner’s permission is required. Custom place name signs may be permitted by the county for sovereign lands and unincorporated areas that are identified on the Official Florida Transportation Map, only where they will be located outside non-limited access facilities right of way. Place name signs located off the state highway system right of way shall conform to Section 479.16, F.S.

(5) The Department will not assume responsibility for vegetation management.

(6) The custom place name sign and structure or other treatment shall be located at or in absolute proximity to, the geographical boundary of the county, municipality, sovereign nation or unincorporated area in the approach direction only.

(7) The proposed installation shall not interfere in any manner with other traffic control devices in the area.

(8) Existing standard geographic boundary guide signs, and/or nonofficial signs or structures, at or near the location shall be removed.

(9) All custom place name signs installed under these provisions are traffic control devices and shall be made of retro-reflective material or be lighted. Supporting structures shall be designed, constructed, and installed to meet the Department’s clear zone and safety criteria including breakaway features, when located within the state right of way or located where there is limited lateral clearance from the travel lanes. The design shall be signed and sealed by a Professional Engineer registered in the State of Florida.

(10) Sign size and lettering shall be appropriate for driver readability without decreasing speed.

(11) Sign information shall be limited to the name of the county, municipality, sovereign nation, or eligible unincorporated area, a pictograph, the words “Welcome To,” and where applicable, a regional designation or phrase approved by local resolution. The “regional designation or phrase” is the only opportunity to chose and indicate an association with any one community designation or initiative program, including national and local grant, planning, or development. Highway signing is intended for traffic control only.

(12) The sign and structure shall be completely devoid of any commercial advertising, sponsorship, or the name of any political candidate, and be of such design and color as to be in good taste and aesthetically pleasing.

(13) In urban areas, the primary location for custom place name signs shall be along the roadside behind curb and gutter sections. Medians will only be considered if all other approach, right roadside locations, either on or off state highway rights of way, are not physically possible. Median spaces shall not be used to accommodate design size or aesthetic preference.

(14) Installations in any median shall meet the Department’s appropriate clear zone and safety criteria. Signs shall not be installed in both the median and roadside at a given location.

(15) Displays shall be fixed. Flashing lights, colored lights, or changeable messages shall not be used. However, customized treatment, including interior or exterior illumination, is allowable. In the absence of lighting, all sign face components shall be retro-reflective.

(16) Upon approval of a customized place name sign request, the Department and the local government shall execute an agreement providing for the local government to install and maintain the customized sign/sign supports and all landscaping and shrubbery associated with the installation, as well as to defray the cost of any electrical energy necessary for operation of the sign display. The agreement shall clearly indicate that the Department reserves the right to have the installation modified or removed from the state highway right of way.

(17) Unincorporated areas that are not on the Official Florida Transportation Map are not eligible for custom place name signs.

Rulemaking Authority 316.0745 FS. Law Implemented 316.0745 FS. History–New 3-27-05, Amended 11-24-11.