62-550.828 Ground Water Rule.

(1) The requirements contained in the July 1, 2011, edition of 40 C.F.R. Part 141, Subpart S (Sections 141.400 through 141.405) (http://www.flrules.org/Gateway/Reference.asp?No=Ref-00914), are adopted and incorporated herein by reference and are enforceable under this rule, except the following regulations are not adopted or incorporated in this rule: 40 C.F.R. §§141.400(c)(5), 141.402(e)(2), 141.402(f), and 141.403(a)(7)(iii).

(2) In addition to the public notification requirements in Part IV of Chapter 62-560, F.A.C., the following public notification requirements are adopted and incorporated herein by reference and are enforceable under this rule:

(a) In the July 1, 2011, edition of 40 C.F.R. §141.202 (http://www.flrules.org/Gateway/Reference.asp?No=Ref-00916), the Tier 1 public notice requirements pertaining to detection of E. coli, enterococci, or coliphage in source water samples required under 40 C.F.R. §§141.402(a) and 141.402(b) (July 1, 2011), which are incorporated by reference in subsection 62-550.828(1), F.A.C.

(b) In the July 1, 2011, edition of 40 C.F.R. §141.203 (http://www.flrules.org/Gateway/Reference.asp?No=Ref-00919), the Tier 2 public notice requirements pertaining to failure to take corrective action or failure to maintain at least 4-log treatment of viruses before or at the first customer under 40 C.F.R. §141.403(a) (July 1, 2011), which is incorporated by reference in subsection 62-550.828(1), F.A.C.

- TABLE 1

- MAXIMUM CONTAMINANT LEVELS FOR INORGANIC COMPOUNDS

| FEDERAL CONTAMINANT ID NUMBER | CONTAMINANT | MCL (mg/L) |
| --- | --- | --- |
| 1074 | Antimony | 0.006 |
| 1005 | Arsenic | 0.05 through 12/31/2004 0.010 on and after 01/01/2005 |
| 1094 | Asbestos | 7 MFL |
| 1010 | Barium | 2 |
| 1075 | Beryllium | 0.004 |
| 1015 | Cadmium | 0.005 |
| 1020 | Chromium | 0.1 |
| 1024 | Cyanide (as free Cyanide) | 0.2 |
| 1025 | Fluoride | 4.0 |
| 1030 | Lead | 0.015 |
| 1035 | Mercury | 0.002 |
| 1036 | Nickel | 0.1 |
| 1040 | Nitrate | 10 (as N) |
| 1041 | Nitrite | 1 (as N) |
|  | Total Nitrate and Nitrite | 10 (as N) |
| 1045 | Selenium | 0.05 |
| 1052 | Sodium | 160 |
| 1085 | Thallium | 0.002 |

Abbreviations Used:  MCL = maximum contaminant level;

MFL = million fibers per liter (longer than 10 micrometers);

mg/L = milligrams per liter.

- TABLE 2

- MAXIMUM RESIDUAL DISINFECTANT LEVELS

| FEDERAL CONTAMINANT ID NUMBER | DISINFECTANT RESIDUAL | MRDL (mg/L) |
| --- | --- | --- |
| 1012 | Chlorine | 4.0 (as Cl2) |
| 1006 | Chloramines | 4.0 (as Cl2) |
| 1008 | Chlorine Dioxide | 0.8 (as ClO2) |

- Abbreviations Used:	mg/L = milligrams per liter;

MRDL = maximum residual disinfectant level.

- TABLE 4

- MAXIMUM CONTAMINANT LEVELS FOR VOLATILE ORGANIC CONTAMINANTS

| FEDERAL CONTAMINANT ID NUMBER | CONTAMINANT & (CAS NUMBER) | MCL (mg/L) |
| --- | --- | --- |
| 2977 | 1,1-Dichloroethylene (75-35-4) | 0.007 |
| 2981 | 1,1,1-Trichloroethane (71-55-6) | 0.2 |
| 2985 | 1,1,2-Tricholoroethane (79-00-5) | 0.005 |
| 2980 | 1,2-Dichloroethane (107-06-2) | 0.003 |
| 2983 | 1,2-Dichloropropane (78-87-5) | 0.005 |
| 2378 | 1,2,4-Tricholorobenzene (120-82-1) | 0.07 |
| 2990 | Benzene (71-43-2) | 0.001 |
| 2982 | Carbon tetrachloride (56-23-5) | 0.003 |
| 2380 | cis-1,2-Dichloroethylene (156-59-2) | 0.07 |
| 2964 | Dichloromethane (75-09-2) | 0.005 |
| 2992 | Ethylbenzene (100-41-4) | 0.7 |
| 2989 | Monochlorobenzene (108-90-7) | 0.1 |
| 2968 | o-Dichlorobenzene (95-50-1) | 0.6 |
| 2969 | para-Dichlorobenzene (106-46-7) | 0.075 |
| 2996 | Styrene (100-42-5) | 0.1 |
| 2987 | Tetrachloroethylene (127-18-4) | 0.003 |
| 2991 | Toluene (108-88-3) | 1 |
| 2979 | trans-1,2-Dichloroethylene (156-60-5) | 0.1 |
| 2984 | Trichloroethylene (79-01-6) | 0.003 |
| 2976 | Vinyl chloride (75-01-4) | 0.001 |
| 2955 | Xylenes (total) (1330-20-7) | 10 |

Abbreviations used:	CAS Number = Chemical Abstract System Number;

MCL = maximum contaminant level;

mg/L = milligrams per liter.

- TABLE 5

- MAXIMUM CONTAMINANT LEVELS FOR SYNTHETIC ORGANIC CONTAMINANTS

| FEDERAL CONTAMINANT ID NUMBER | CONTAMINANT & (CAS NUMBER) | MCL (mg/L) | Regulatory Detection Limit (mg/L) |
| --- | --- | --- | --- |
| 2063 | 2,3,7,8-TCDD (Dioxin) (1746-01-6) | 3 X 10-8 | 5 x 10-9 |
| 2105 | 2,4-D (94-75-7) | 0.07 | 0.0001 |
| 2110 | 2,4,5-TP (Silvex) (93-72-1) | 0.05 | 0.0002 |
| 2051 | Alachlor (15972-60-8) | 0.002 | 0.0002 |
| 2050 | Atrazine (1912-24-9) | 0.003 | 0.0001 |
| 2306 | Benzo(a)pyrene (50-32-8) | 0.0002 | 0.00002 |
| 2046 | Carbofuran (1563-66-2) | 0.04 | 0.0009 |
| 2959 | Chlordane (57-74-9) | 0.002 | 0.0002 |
| 2031 | Dalapon (75-99-0) | 0.2 | 0.001 |
| 2035 | Di(2-ethylhexyl)adipate (103-23-1) | 0.4 | 0.0006 |
| 2039 | Di(2-ethylhexyl)phthalate (117-81-7) | 0.006 | 0.0006 |
| 2931 | Dibromochloropropane (DBCP) (96-12-8) | 0.0002 | 0.00002 |
| 2041 | Dinoseb (88-85-7) | 0.007 | 0.0002 |
| 2032 | Diquat (85-00-7) | 0.02 | 0.0004 |
| 2033 | Endothall (145-73-3) | 0.1 | 0.009 |
| 2005 | Endrin (72-20-8) | 0.002 | 0.00001 |
| 2946 | Ethylene dibromide (EDB) (106-93-4) | 0.00002 | 0.00001 |
| 2034 | Glyphosate (1071-83-6) | 0.7 | 0.006 |
| 2065 | Heptachlor (76-44-8) | 0.0004 | 0.00004 |
| 2067 | Heptachlor epoxide (1024-57-3) | 0.0002 | 0.00002 |
| 2274 | Hexachlorobenzene (118-74-1) | 0.001 | 0.0001 |
| 2042 | Hexachlorocyclopentadiene (77-47-4) | 0.05 | 0.0001 |
| 2010 | Lindane (58-89-9) | 0.0002 | 0.00002 |
| 2015 | Methoxychlor (72-43-5) | 0.04 | 0.0001 |
| 2036 | Oxamyl (vydate) (23135-22-0) | 0.2 | 0.002 |
| 2326 | Pentachlorophenol (87-86-5) | 0.001 | 0.00004 |
| 2040 | Picloram (1918-02-1) | 0.5 | 0.0001 |
| 2383 | Polychlorinated biphenyls (PCBs) | 0.0005 | 0.0001 |
| 2037 | Simazine (122-34-9) | 0.004 | 0.00007 |
| 2020 | Toxaphene (8001-35-2) | 0.003 | 0.001 |

Abbreviations used:	CAS Number = Chemical Abstract System Number;

MCL = maximum contaminant level;

mg/L = milligrams per liter.

NOTE: The Detection Limits listed in this table become effective January 22, 2004.

- TABLE 6

- SECONDARY DRINKING WATER STANDARDS

| FEDERAL CONTAMINANT ID NUMBER | CONTAMINANT | SMCL (mg/L)* |
| --- | --- | --- |
| 1002 | - Aluminum | 0.2 |
| 1017 | Chloride | 250 |
| 1022 | Copper | 1 |
| 1025 | Fluoride | 2.0 |
| 1028 | Iron | 0.3 |
| 1032 | Manganese | 0.05 |
| 1050 | - Silver | 0.1 |
| 1055 | Sulfate | 250 |
| 1095 | Zinc | 5 |
| 1905 | Color | 15 color units |
| 1920 | Odor** | 3 (threshold odor number) |
| 1925 | pH | 6.5 - 8.5 |
| 1930 | - Total Dissolved Solids | 500 |
| 2905 | - Foaming Agents | 0.5 |

- Abbreviations Used:

- SMCL = maximum contaminant level;

- mg/L = milligrams per liter.

*	Except color, odor, and pH.

**	For purpose of compliance with ground water quality secondary standards, as referenced in Chapter 62-520, F.A.C., levels of ethylbenzene exceeding 30 micrograms per liter, toluene exceeding 40 micrograms per liter, or xylenes exceeding 20 micrograms per liter shall be considered equivalent to exceeding the drinking water secondary standard for odor.

- TABLE 7: MONITORING FREQUENCIES AND LOCATIONS (6/24/2014)

See 40 C.F.R. Part 141, Subpart V, which is incorporated by reference in Rule 62-550.822, F.A.C., for Stage 2 disinfection byproducts monitoring frequencies and locations.

| - CONTAMINANT OR DISINFECTANT RESIDUAL GROUP | APPLICABILITY | INITIAL OR ROUTINE MONITORING |  | TRIGGER THAT INCREASES MONITORING | INCREASED MONITORING |  | TRIGGER THAT REDUCES MONITORING | REDUCED MONITORING |  | MONITORING LOCATION(S) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GW SYSTEMS | SUBPART H SYSTEMS |  | GW SYSTEMS | SUBPART H SYSTEMS |  | GW SYSTEMS | SUBPART H SYSTEMS |  |
| ASBESTOS RULE 62-550.511 | CWSs, NTNCWSs | 1 SAMPLE EVERY 9 YEARS |  | SAMPLE > MCL | 1 SAMPLE QUARTERLY |  | SYSTEM NOT SUSCEPTIBLE | NO SAMPLING REQUIRED |  | NOTE 1 |
| NITRATE & NITRITE RULES 62-550.500(5) & 62-550.512 | CWSs, NTNCWSs | 1 SAMPLE ANNUALLY | 1 SAMPLE QUARTERLY | GW SYS. WITH SAMPLE  50% OF MCL | 1 SAMPLE QUARTERLY | --- | SUBPART H SYS. WITH EACH OF 4 MOST RECENT QUARTERLY SAMPLES  50% OF MCL | --- | 1 SAMPLE ANNUALLY DURING QUARTER WITH HIGHEST RESULT | EVERY ENTRY POINT TO DIST. SYS. DURING NORMAL OPERATING CONDITIONS |
|  | TWSs | 1 SAMPLE ANNUALLY |  | NITRATE SAMPLE  MCL OR NITRITE SAMPLE  50% OF MCL | 1 SAMPLE QUARTERLY |  | --- | --- |  |  |
| INORGANICS RULES 62-550.500(5) & 62-550.513 | CWSs, NTNCWSs | 1 SAMPLE EVERY 3 YEARS | 1 SAMPLE ANNUALLY | SAMPLE > MCL | 1 SAMPLE QUARTERLY |  | FOUR CONSECUTIVE QUARTERS < MCL | SEE ROUTINE MONITORING |  | EVERY ENTRY POINT TO DIST. SYS. DURING NORMAL OPERATING CONDITIONS |
| CHLORINE & CHLORAMINES RULES 62-550.514(1) & 62-550.821 | CWSs/NTNCWSs ADDING CHLORINE OR CHLORAMINES | MULTIPLE SAMPLES MONTHLY; SEE NOTE 2 |  | --- | --- |  | --- | --- |  | NOTE 2 |
| CHLORINE DIOXIDE RULES 62-550.514(1) & 62-550.821 | CWSs/NTNCWSs/TWSs ADDING CHLORINE DIOXIDE | 1 SAMPLE DAILY |  | SAMPLE > MCL | ADDITIONAL 3-SAMPLE SET THE FOLLOWING DAY |  | --- | --- |  | NOTE 3 |
| TOTAL TRIHALOMETHANES & HALOACETIC ACIDS (FIVE) - STAGE 1 MCLs RULES 62-550.514(2) & 62-550.821 | CWSs/NTNCWSs ADDING A DISINFECTANT & SERVING  10,000 PERSONS | 1 SAMPLE PER TREATMENT PLANT QUARTERLY | 4 SAMPLES PER TREATMENT PLANT QUARTERLY | --- | --- |  | GW SYS. WITH ANNUAL AVG. TTHM & HAA5  50% OF MCL; SUBPART H SYS. WITH ANNUAL AVG. SOURCE-WATER TOC  4.0 MG/L & ANNUAL AVG. TTHM & HAA5  50% OF MCL | 1 SAMPLE PER TREATMENT PLANT ANNUALLY DURING MONTH OF WARMEST WATER TEMP. | 1 SAMPLE PER TREATMENT PLANT QUARTERLY | NOTE 4 |
|  | CWSs/NTNCWSs ADDING A DISINFECTANT & SERVING 500 to 9,999 PERSONS | 1 SAMPLE PER TREATMENT PLANT ANNUALLY DURING MONTH OF WARMEST WATER TEMP. | 1 SAMPLE PER TREATMENT PLANT QUARTERLY | GW SYS. WITH AVG. OF ANNUAL SAMPLES > MCL | 1 SAMPLE PER TREATMENT PLANT QUARTERLY | --- | GW SYS. WITH ANNUAL AVG. TTHM & HAA5  50% OF MCL FOR 2 CONSECUTIVE YEARS OR  25% OF MCL FOR 1 YEAR; SUBPART H SYS. WITH ANNUAL AVG. SOURCE-WATER TOC  4.0 MG/L & ANNUAL AVG. TTHM & HAA5  50% OF MCL | 1 SAMPLE PER TREATMENT PLANT EVERY 3 YEARS DURING MONTH OF WARMEST WATER TEMP. | 1 SAMPLE PER TREATMENT PLANT ANNUALLY DURING MONTH OF WARMEST WATER TEMP. |  |
|  | CWSs/NTNCWSs ADDING A DISINFECTANT & SERVING < 500 PERSONS | 1 SAMPLE PER TREATMENT ANNUALLY DURING MONTH OF WARMEST WATER TEMP. |  | AVG. OF ANNUAL SAMPLES > MCL | 1 SAMPLE PER TREATMENT PLANT QUARTERLY |  | GW SYS. WITH ANNUAL AVG. TTHM & HAA5  50% OF MCL FOR 2 CONSECUTIVE YEARS OR  25% OF MCL FOR 1 YEAR |  | --- |  |
| CHLORITE - STAGE 1 MCL RULES 62-550.514(2) & 62-550.821 | CWSs/NTNCWSs ADDING CHLORINE DIOXIDE | 1 SAMPLE DAILY |  | SAMPLE > MCL | ADDITIONAL 3-SAMPLE SET THE FOLLOWING DAY |  | --- | --- |  | NOTE 5 |
|  |  | 3-SAMPLE SET MONTHLY |  | --- | --- |  | NO INDIVIDUAL ENTRY-POINT OR DIST. SYS. SAMPLE > MCL FOR 1 YEAR | 3-SAMPLE SET QUARTERLY |  |  |
| BROMATE - STAGE 1 MCL RULES 62-550.514(2)(b) & 62-550.821 | CWSs/NTNCWSs ADDING OZONE | 1 SAMPLE PER TREATMENT PLANT MONTHLY |  | --- | --- |  | ANNUAL AVG. SOURCE-WATER BROMIDE  0.05 MG/L BASED UPON MONTHLY MEASUREMENTS | 1 SAMPLE PER TREATMENT PLANT QUARTERLY |  | ENTRANCE TO DIST. SYS. UNDER NORMAL OPERATING CONDITIONS |
| VOLATILE ORGANICS RULES 62-550.500(5) & 62-550.515 | CWSs, NTNCWSs | 4 CONSECUTIVE QUARTERLY SAMPLES EVERY 3 YEARS OR, IF AUTHORIZED, 1 SAMPLE ANNUALLY; SEE NOTE 6 |  | DETECTION OF ANY VOC AT  0.0005 MG/L | 1 SAMPLE QUARTERLY |  | GW SYS. WITH NO DETECTION OF ANY VOC DURING 3 YEARS OF ANNUAL SAMPLING | 1 SAMPLE EVERY 3 YEARS | --- | NOTE 7 |
| SYNTHETIC ORGANICS RULES 62-550.500(5) & 62-550.516 | CWSs/NTNCWSs SERVING  3,300 PERSONS | 4 CONSECUTIVE QUARTERLY SAMPLES EVERY 3 YEARS |  | DETECTION OF ANY SOC | 1 SAMPLE QUARTERLY |  | NO DETECTION OF ANY SOC DURING INITIAL COMPLIANCE PERIOD | 2 QUARTERLY SAMPLES IN THE SAME YEAR EVERY 3 YEARS |  | NOTE 7 |
|  | CWSs/NTNCWSs SERVING  3,300 PERSONS |  |  |  |  |  |  | 1 SAMPLE EVERY 3 YEARS |  |  |
| MICROBIOLOGICAL CONTAMINANTS RULE 62-550.518 | CWSs, NTNCWSs, TWSs SERVING > 1,000 PERSONS | MULTIPLE SAMPLES MONTHLY; SEE NOTE 8 |  | TOTAL COLIFORM POSITIVE SAMPLE | NOTE 9 |  | --- | --- |  | SITES REFLECTING WATER THROUGHOUT DIST. SYS. |
|  | TWSs SERVING  1,000 PERSONS | 2 SAMPLES QUARTERLY | 2 SAMPLES MONTHLY |  |  |  |  |  |  |  |
| SECONDARY CONTAMINANTS RULES 62-550.500(5) & 62-550.520 | CWSs | 1 SAMPLE EVERY 3 YEARS |  | --- | --- |  | --- | --- |  | EVERY ENTRY POINT TO DIST. SYS. DURING NORMAL OPERATING CONDITIONS |
| GROSS ALPHA, RADIUM-226, RADIUM-228, & URANIUM RULE 62-550.519(1) | CWSs | INITIAL MONITORING IS 4 CONSECUTIVE QUARTERLY SAMPLES ROUTINE MONITORING IS 1 SAMPLE EVERY 3 YEARS - NOTE 10 |  | SAMPLE > MCL OR IF MONITORING ONCE EVERY 6 YEARS, A SAMPLE RESULT > 1/2 MCL OR IF MONITORING ONCE EVERY 9 YEARS, A SAMPLE RESULT ≥ DETECTION LIMIT | - 1 SAMPLE QUARTERLY WHEN PREVIOUS SAMPLE RESULT IS > MCL OR IF SAMPLING EVERY 9 YEARS AND THE SAMPLE IS  MCL BUT > 1/2 MCL, SAMPLE EVERY 6 YEARS; OR IF SAMPLE IS ≥ DETECTION LIMIT BUT  1/2 MCL OR IF SAMPLING EVERY 6 YEARS AND THE SAMPLE IS > 1/2 MCL BUT  MCL, SAMPLE EVERY 3 YEARS |  | AVERAGE OF INITIAL MONITORING SAMPLES OR LAST REDUCED MONITORING SAMPLE < DETECTION LIMIT | - 1 SAMPLE EVERY 9 YEARS |  | EVERY ENTRY POINT TO DIST. SYS. DURING NORMAL OPERATING CONDITIONS |
|  |  |  |  |  |  |  | DETECTION LIMIT  AVERAGE OF INITIAL MONITORING SAMPLES OR LAST REDUCED MONITORING SAMPLE  1/2 MCL | - 1 SAMPLE EVERY 6 YEARS |  |  |
|  | NTNCs | ROUTINE MONITORING IS 1 SAMPLE EVERY 6 YEARS NOTE 10 |  | SAMPLE > MCL OR IF MONITORING ONCE EVERY 9 YEARS, A SAMPLE RESULT > ½ MCL | - 1 SAMPLE QUARTERLY WHEN PREVIOUS SAMPLE - RESULT IS > MCL OR IF SAMPLING EVERY 9 YEARS AND THE SAMPLE IS > ½ MCL BUT  MCL, SAMPLE EVERY 6 YEARS |  | AVERAGE OF QUARTERLY MONITORING SAMPLES OR LAST MONITORING SAMPLE < DETECTION LIMIT | - NO FURTHER MONITORING REQUIRED |  |  |
|  |  |  |  |  |  |  | AVERAGE OF INITIAL MONITORING SAMPLES OR LAST REDUCED MONITORING SAMPLE  1/2 MCL AND > DETECTION LIMIT | 1 SAMPLE - EVERY 9 YEARS |  |  |
| BETA PARTICLE & PHOTON RADIOACTIVITY RULE 62-550.519(2) | CWSs DESIGNATED AS VULNERABLE | 1 SAMPLE QUARTERLY FOR GROSS BETA & 1 SAMPLE ANNUALLY FOR TRITIUM & STRONTIUM-90 |  | - SAMPLE > MCL | - 1 SAMPLE MONTHLY FOR GROSS BETA, TRITIUM, & STRONTIUM-90 |  | ANNUAL AVERAGE OF GROSS BETA MINUS POTASSIUM-40  50 pCi/L | - 1 SAMPLE EVERY 3 YEARS FOR GROSS BETA, TRITIUM, & STRONTIUM-90 |  | EVERY ENTRY POINT TO DIST. SYS. DURING NORMAL OPERATING CONDITIONS |
|  | CWSs DESIGNATED AS UTILIZING WATERS CONTAMINATED BY EFFLUENTS FROM NUCLEAR FACILITIES | QUARTERLY FOR GROSS BETA & IODINE-131 & ANNUALLY FOR TRITIUM & STRONTIUM-90; SEE NOTE 11 |  | QUARTERLY RESULT FOR GROSS BETA OR IODINE-131 > MCL; ANNUAL RESULT FOR TRITIUM OR STRONTIUM-90 > MCL | - MONTHLY FOR GROSS BETA, IODINE-131, TRITIUM, & STRONTIUM-90 |  | ANNUAL AVERAGE OF GROSS BETA MINUS POTASSIUM-40  15 pCi/L | - EVERY 3 YEARS FOR GROSS BETA, IODINE-131, TRITIUM, & STRONTIUM-90 |  |  |

- TABLE 8: INITIAL OR ROUTINE MONITORING SCHEDULE (6/24/2014)

REFERENCE SUBSECTION 62-550.500(3), F.A.C.

Under initial or routine monitoring, public water systems shall take required samples during the time period specified below. See 40 C.F.R. Part 141, Subpart V, which is incorporated by reference in Rule 62-550.822, F.A.C., for the Stage 2 disinfection byproducts routine monitoring schedule.

| CONTAMINANT OR DISINFECTANT RESIDUAL GROUP |  | COMMUNITY WATER SYSTEMS SERVING MORE THAN 3,300 PEOPLE | COMMUNITY WATER SYSTEMS SERVING 3,300 OR FEWER PEOPLE | NON-TRANSIENT NON-COMMUNITY WATER SYSTEMS | TRANSIENT NON-COMMUNITY WATER SYSTEMS |
| --- | --- | --- | --- | --- | --- |
| ASBESTOS RULES 62-550.500(3) & 62-550.511 |  | FIRST YEAR OF EACH NINE-YEAR COMPLIANCE CYCLE | SECOND YEAR OF EACH NINE-YEAR COMPLIANCE CYCLE | THIRD YEAR OF EACH NINE-YEAR COMPLIANCE CYCLE | NOT REQUIRED |
| NITRATES AND NITRITES RULES 62-550.500(3) & 62-550.512 | GROUND WATER SYSTEMS | ANNUALLY |  |  |  |
|  | SUBPART H SYSTEMS | QUARTERLY |  |  | ANNUALLY |
| INORGANICS RULES 62-550.500(3) & 62-550.513 | GROUND WATER SYSTEMS | FIRST YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | SECOND YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | THIRD YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | NOT REQUIRED |
|  | SUBPART H SYSTEMS | ANNUALLY |  |  | NOT REQUIRED |
| CHLORINE & CHLORAMINES RULES 62-550.514(1) & 62-550.821 |  | MONTHLY FOR SYSTEMS ADDING CHLORINE OR CHLORAMINES |  |  | NOT REQUIRED |
| CHLORINE DIOXIDE RULES 62-550.514(1) & 62-550.821 |  | DAILY FOR SYSTEMS ADDING CHLORINE DIOXIDE |  |  |  |
| TOTAL TRIHALOMETHANES & HALOACETIC ACIDS (FIVE) – STAGE 1 MCL RULES 62-550.500(3), 550.514(2), & 62-550.821 | GROUND WATER SYSTEMS | QUARTERLY FOR SYSTEMS ADDING A DISINFECTANT & SERVING  10,000 PEOPLE & ANNUALLY DURING MONTH OF WARMEST WATER TEMPERATURE FOR SYSTEMS ADDING A DISINFECTANT & SERVING  10,000 PEOPLE | ANNUALLY DURING MONTH OF WARMEST WATER TEMPERATURE FOR SYSTEMS ADDING A DISINFECTANT | QUARTERLY FOR SYSTEMS ADDING A DISINFECTANT & SERVING  10,000 PEOPLE & ANNUALLY DURING MONTH OF WARMEST WATER TEMPERATURE FOR SYSTEMS ADDING A DISINFECTANT & SERVING  10,000 PEOPLE | NOT REQUIRED |
|  | SUBPART H SYSTEMS | QUARTERLY FOR SYSTEMS ADDING A DISINFECTANT | QUARTERLY FOR SYSTEMS ADDING A DISINFECTANT & SERVING  500 PEOPLE & ANNUALLY DURING MONTH OF WARMEST WATER TEMPERATURE FOR SYSTEMS ADDING A DISINFECTANT & SERVING  500 PEOPLE |  | NOT REQUIRED |
| CHLORITE – STAGE 1 MCL RULES 62-550.514(2) & 62-550.821 |  | DAILY AT ENTRANCE TO DISTRIBUTION SYSTEM & MONTHLY IN DISTRIBUTION SYSTEM FOR SYSTEMS ADDING CHLORINE DIOXIDE |  |  | NOT REQUIRED |
| BROMATE – STAGE 1 MCL RULES 62-550.514(2) & 62-550.821 |  | MONTHLY FOR SYSTEMS ADDING OZONE |  |  | NOT REQUIRED |
| VOLATILE ORGANICS RULES 62-550.500(3) & 62-550.515 |  | QUARTERLY OR, IF AUTHORIZED, ANNUALLY DURING THE FIRST YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | QUARTERLY OR, IF AUTHORIZED, ANNUALLY DURING THE SECOND YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | QUARTERLY OR, IF AUTHORIZED, ANNUALLY DURING THE THIRD YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | NOT REQUIRED |
| SYNTHETIC ORGANICS RULES 62-550.500(3) & 62-550.516 |  | QUARTERLY DURING THE FIRST YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD. UNLESS REDUCED MONITORING IS AUTHORIZED | QUARTERLY DURING THE SECOND YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD. UNLESS REDUCED MONITORING IS AUTHORIZED | QUARTERLY DURING THE THIRD YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD UNLESS REDUCED MONITORING IS AUTHORIZED | NOT REQUIRED |
| MICROBIOLOGICAL CONTAMINANTS RULES 62-550.500(3) & 62-550.518 | GROUND WATER SYSTEMS | MONTHLY |  |  | MONTHLY FOR SYSTEMS SERVING  1,000 PEOPLE & QUARTERLY FOR SYSTEMS SERVING  1,000 PEOPLE |
|  | SUBPART H SYSTEMS | MONTHLY |  |  |  |
| GROSS ALPHA, RADIUM-226, RADIUM-228 & URANIUM RULES 62-550.500(3) & 62-550.519 |  | ROUTINE MONITORING OCCURS DURING FIRST YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD UNLESS REDUCED MONITORING IS APPROVED | ROUTINE MONITORING OCCURS DURING SECOND YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD UNLESS REDUCED MONITORING IS APPROVED | ROUTINE MONITORING OCCURS DURING THIRD YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD UNLESS REDUCED MONITORING IS APPROVED NOT REQUIRED | NOT REQUIRED |
| BETA PARTICLE & PHOTON RADIOACTIVITY RULES 62-550.500(3) & 62-550.519 |  | QUARTERLY/ANNUALLY FOR SYSTEMS DESIGNATED AS VULNERABLE OR UTILIZING WATERS CONTAMINATED BY EFFLUENTS FROM NUCLEAR FACILITIES |  | NOT REQUIRED | NOT REQUIRED |
| SECONDARY CONTAMINANTS RULES 62-550.500(3) & 62-550.520 |  | FIRST YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | SECOND YEAR OF EACH THREE-YEAR COMPLIANCE PERIOD | NOT REQUIRED | NOT REQUIRED |

NOTE 1: Systems susceptible to asbestos contamination due solely to corrosion of asbestos-cement pipe shall sample at a tap served by asbestos-cement pipe and under conditions where asbestos contamination is most likely to occur. Systems susceptible to asbestos contamination due solely to source water shall monitor at every entry point to the distribution system during normal operating conditions. Systems susceptible to asbestos contamination due to both source water and corrosion of asbestos-cement pipe shall sample at a tap served by asbestos-cement pipe and under conditions where asbestos contamination is most likely to occur.

NOTE 2: Systems shall measure the residual disinfectant level at the same locations in the distribution system where, and at the same time when, total coliforms are sampled.

NOTE 3: Systems shall take routine daily samples at the entrance to the distribution system. Systems shall take additional three-sample sets in the distribution system at the following locations:

(a) If chloramines are used to maintain a disinfectant residual in the distribution system or if chlorine is used to maintain a disinfectant residual in the distribution system and there are no disinfection points after the entrance to the distribution system (i.e., no booster chlorination), the system shall take three samples as close to the first customer as possible at intervals of at least six hours.

(b) If chlorine is used to maintain a disinfectant residual in the distribution system and there are one or more disinfectant addition points after the entrance to the distribution system (i.e., booster chlorination), the system shall take one sample at each of the following locations: as close to the first customer as possible, in a location representative of average residence time, and as close to the end of the distribution system as possible.

NOTE 4: Systems taking one sample shall take the sample at a location reflecting maximum residence time in the distribution system. Systems taking more than one sample shall take at least 25% of the samples at locations representing maximum residence time of the water in the distribution system and shall take the remaining samples at locations representing at least average residence time in the distribution system and representing the entire distribution system, taking into account number of persons served, different sources of water, and different treatment methods.

NOTE 5: Systems shall take routine daily samples at the entrance to the distribution system. Systems shall take routine monthly or additional three-set samples in the distribution system; each three-set sample shall consist of one sample at each of the following locations: a location as close to the first customer as possible, a location representative of average residence time, and a location reflecting maximum residence time in the distribution system.

NOTE 6: For initial base point monitoring, systems shall take four consecutive quarterly samples during the first three-year compliance period. If a system does not detect any VOC, it shall take one sample annually beginning with the next three-year compliance period.

NOTE 7: During the first quarter of initial base point monitoring, GW systems shall take a minimum of one sample that is representative of each well. Under all other circumstances, systems shall sample at every entry point to the distribution system during normal operating conditions.

NOTE 8: The minimum number of samples shall be as set forth in Rule 62-550.518(2), F.A.C.

NOTE 9: Systems shall conduct repeat monitoring in accordance with Rule 62-550.518(7), F.A.C., and systems that routinely collect fewer than five samples per month shall collect at least five samples during the next month the system provides water to the public.

NOTE 10: The Department shall waive the final two quarters of initial monitoring for a sampling point if the results of the samples from the previous two quarters are below the regulatory detection limit. Additionally, under the conditions described in Rule 62-550.519(1)(c), F.A.C., historical data may be used to satisfy initial monitoring requirements.

NOTE 11: Quarterly monitoring for gross beta shall be based on the analysis of monthly samples or the analysis of a composite of three monthly samples. For  iodine-131, a composite of five consecutive daily samples shall be analyzed once each quarter. Annual monitoring for tritium and strontium-90 shall be conducted by means of the analysis of a composite of four consecutive quarterly samples or analysis of four quarterly samples.

Rulemaking Authority 403.8055, 403.861(9) FS. Law Implemented 403.852(12), 403.853(1), (3), (4), (7), 403.861(16), (17) FS. History–New 12-30-11.