- 62-304.710 Loxahatchee Basin TMDLs.

(1) Southwest Fork Loxahatchee. The TMDL for the Southwest Fork Loxahatchee is 43 counts/100mL for fecal coliform, and is allocated as follows:

(2) The wasteload allocation (WLA) for the Loxahatchee Environmental Control District (Permit #FL0034649) is its National Pollutant Discharge Elimination System (NPDES) permit conditions;

(3) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2007 period, will require a 91 percent reduction of sources contributing to exceedances of the criteria;

(4) The load allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2007 period, will require a 91 percent reduction of sources contributing to exceedances of the criteria; andm

(5) The Margin of Safety is implicit.

(6) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class II criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 5-14-12.