- 62-303.200 Definitions.

As used in this chapter:

(1) “Biological Health Assessment” shall mean one of the following aquatic community-based biological evaluations:  Stream Condition Index (SCI), Lake Vegetation Index (LVI), or Shannon-Weaver Diversity Index.

(2) “BioRecon” shall mean a biological assessment that measures stream health in predominantly freshwaters using benthic macroinvertebrates, performed and calculated using the Standard Operating Procedures (SOP) for the BioRecon in the document titled BRN 1000: Biological Reconnaissance Field Method (DEP-SOP-003/11 BRN 1000), dated 10-24-11, which is incorporated by reference herein. Copies of the SOP may be obtained from the Department’s internet site at http://www.dep.state.fl.us/water/wqssp/swq-docs.htm or by writing to the Florida Department of Environmental Protection, Standards and Assessment Section, 2600 Blair Stone Road, MS 6511, Tallahassee, FL 32399-2400.

(3) “Clean techniques” shall mean those applicable field sampling procedures and analytical methods referenced in “Method 1669: Sampling Ambient Water for Trace Metals at EPA Water Quality Criteria Levels, July 1996, USEPA, Office of Water, Engineering and Analysis Division, Washington, D.C.,” which is incorporated by reference. Copies of the procedures and methods may be obtained from the Department’s internet site at http://www.dep.state.fl.us/water/wqssp/swq-docs.htm or by writing to the Florida Department of Environmental Protection, Standards and Assessment Section, 2600 Blair Stone Road, MS 6511, Tallahassee, FL 32399-2400.

(4) “Department” or “DEP” shall mean the Florida Department of Environmental Protection.

(5) “Designated use” shall mean the present and future most beneficial use of a body of water as designated by the Environmental Regulation Commission by means of the classification system contained in Chapter 62-302, F.A.C.

(6) “Estuary” shall mean predominantly marine regions of interaction between rivers and nearshore ocean waters, where tidal action and river flow mix fresh and salt water. Such areas include bays, mouths of rivers, and lagoons.

(7) “Impaired water” shall mean a waterbody or waterbody segment that does not meet its applicable water quality standards as set forth in Chapters 62-302 and 62-4, F.A.C., as determined by the methodology in Part IV of this chapter, due in whole or in part to discharges of pollutants from point or nonpoint sources.

(8) “Lake” shall mean a lentic fresh waterbody with a relatively long water residence time and an open water area that is free from emergent vegetation under typical hydrologic and climatic conditions. Aquatic plants, as defined in subsection 62-340.200(1), F.A.C., may be present in the open water. Lakes do not include springs, wetlands, or streams (except portions of streams that exhibit lake-like characteristics, such as long water residence time, increased width, or predominance of biological taxa typically found in non-flowing conditions).

(9) “Lake Vegetation Index (LVI)” shall mean a Biological Health Assessment that measures lake biological health in predominantly freshwaters using aquatic and wetland plants, performed and calculated using the Standard Operating Procedures for the LVI in the document titled LVI 1000: Lake Vegetation Index Methods (DEP-SOP-003/11 LVI 1000) and the methodology in Sampling and Use of the Lake Vegetation Index (LVI) for Assessing Lake Plant Communities in Florida: A Primer (DEP-SAS-002/11), both dated 10-24-11, which are incorporated by reference herein.  Copies of the documents may be obtained from the Department’s internet site at http://www.dep.state.fl.us/water/wqssp/swq-docs.htm or by writing to the Florida Department of Environmental Protection, Standards and Assessment Section, 2600 Blair Stone Road, MS 6511, Tallahassee, FL 32399-2400.

(10) “Nuisance species” shall mean species of flora or fauna whose noxious characteristics or presence in sufficient number, biomass, or areal extent may reasonably be expected to prevent, or unreasonably interfere with, a designated use of those waters.

(11) “Open coastal waters” shall mean all gulf or ocean waters that are not classified as estuaries or open ocean waters.

(12) “Nutrient” shall mean total nitrogen (TN), total phosphorus (TP), or their organic or inorganic forms.

(13) “Nutrient response variable” shall mean a biological variable, such as chlorophyll a, biomass, or structure of the phytoplankton, periphyton or vascular plant community, that responds to nutrient load or concentration in a predictable and measurable manner. For purposes of interpreting paragraph 62-302.530(47)(b), F.A.C., Dissolved oxygen (DO) shall also be considered a nutrient response variable if it is demonstrated for the waterbody that DO conditions result in biological imbalance and the DO responds to a nutrient load or concentration in a predictable and measurable manner.

(14) “Nutrient Watershed Region” shall mean a drainage area over which the nutrient thresholds in paragraph 62-302.531(2)(c), F.A.C., apply.

(a) The Panhandle West region consists of the Perdido Bay Watershed, Pensacola Bay Watershed, Choctawhatchee Bay Watershed, St. Andrew Bay Watershed, and Apalachicola Bay Watershed.

(b) The Panhandle East region consists of the Apalachee Bay Watershed, and Econfina/Steinhatchee Coastal Drainage Area.

(c) The North Central region consists of the Suwannee River Watershed and an area in Alachua County stream to sink region affected by the Hawthorne Formation.

(d) The West Central region consists of the Peace, Myakka, Hillsborough, Alafia, Manatee, Little Manatee River Watersheds, Sarasota/Lemon Bay Watershed and small, direct Tampa Bay tributary watersheds south of the Hillsborough River Watershed.

(e) The Peninsula region consists of the Waccasassa Coastal Drainage Area, Withlacoochee Coastal Drainage Area, Crystal/Pithlachascotee Coastal Drainage Area, small, direct Tampa Bay tributary watersheds west of the Hillsborough River Watershed, small, direct Charlotte Harbor tributary watersheds south of the Peace River Watershed, Caloosahatchee River Watershed, Estero Bay Watershed, Imperial River Watershed, Kissimmee River/Lake Okeechobee Drainage Area, Loxahatchee/St. Lucie Watershed, Indian River Watershed, Daytona/St. Augustine Coastal Drainage Area, St. John’s River Watershed, Nassau Coastal Drainage Area, and St. Mary’s River Watershed.

(f) The South Florida region consists of those areas south of the Peninsula region, such as the Cocohatchee River Watershed, Naples Bay Watershed, Rookery Bay Watershed, Ten Thousand Islands Watershed, Lake Worth Lagoon Watershed, Southeast Coast – Biscayne Bay Watershed, Everglades Watershed, Florida Bay Watershed, and the Florida Keys.

A map of the Nutrient Watershed Regions is incorporated by reference herein and may be obtained from the Department’s internet site at http://www.dep.state.fl.us/water/wqssp/swq-docs.htm or by writing to the Florida Department of Environmental Protection, Standards and Assessment Section, 2600 Blair Stone Road, MS 6511, Tallahassee, FL 32399-2400.

(15) “Open ocean waters” means all surface waters extending seaward from the most seaward natural 90-foot (15-fathom) isobath. Contour lines may be determined from National Oceanic and Atmospheric Administration Charts.

(16) “Physical alterations” shall mean human-induced changes to the physical structure of the waterbody.

(17) “Planning list” shall mean the list of surface waters or segments for which assessments will be conducted to evaluate whether the water is impaired and a TMDL is needed, as provided in subsection 403.067(2), F.S.

(18) “Pollutant” shall be as defined in subsection 502(6) of the CWA. Characteristics of a discharge, including dissolved oxygen, pH, or temperature, shall also be defined as pollutants if they result or may result in the potentially harmful alteration of downstream waters.

(19) “Pollution” shall be as defined in subsection 502(19) of the CWA and subsection 403.031(2), F.S.

(20) “Predominantly fresh waters” shall mean surface waters in which the chloride concentration is less than 1,500 milligrams per liter or specific conductance is less than 4,580 µmhos/cm.

(21) “Predominantly marine waters” shall mean surface waters in which the chloride concentration is greater than or equal to 1,500 milligrams per liter or specific conductance is greater than or equal to 4,580 µmhos/cm.

(22) “Reference water” means a waterbody that exhibits a range of physical, chemical and biological characteristics approximating the natural background conditions of the same, or similar, type of waterbody within an ecologically similar region. A reference water may be representative of the water quality and structure and function of biological communities of natural background conditions even if there is evidence of limited human disturbance in the waterbody or watershed, as long as anthropogenic sources do not produce a significant measurable or predicted effect on the parameter of concern in the waterbody.

(23) “Secretary” shall mean the Secretary of the Florida Department of Environmental Protection.

(24) “Shannon-Weaver Diversity Index” shall mean:  negative summation (from i=1 to s) of (ni/N) log2 (ni/N) where s is the number of species in a sample, N is the total number of individuals in a sample, and ni is the total number of individuals in species i.

(25) “Spill” shall mean a short-term, unpermitted discharge to surface waters, not to include sanitary sewer overflows or chronic discharges from leaking wastewater collection systems.

(26) “Spring vent” shall mean a location where groundwater flows out of a natural, discernable opening in the ground onto the land surface or into a predominantly fresh surface water.

(27) “Stream” shall mean a predominantly fresh surface waterbody the flows in a defined channel with banks., Streams do not include wetlands or portions of streams that exhibit lake characteristics (e.g., long water residence time, increased width, and predominance of biological taxa typically found in non-flowing conditions).

(28) “Stream Condition Index (SCI)” shall mean a Biological Health Assessment that measures stream biological health in predominantly freshwaters using benthic macroinvertebrates, performed and calculated using the Standard Operating Procedures for the SCI in the document titled SCI 1000: Stream Condition Index Methods (DEP-SOP-003/11 SCI 1000) and the methodology in Sampling and Use of the Stream Condition Index (SCI) for Assessing Flowing Waters: A Primer (DEP-SAS-001/11), both dated 10-24-11, which are incorporated by reference herein. Copies of the documents may be obtained from the Department’s internet site at http://www.dep.state.fl.us/water/wqssp/swq-docs.htm or by writing to the Florida Department of Environmental Protection, Standards and Assessment Section, 2600 Blair Stone Road, MS 6511, Tallahassee, FL 32399-2400. For water quality standards purposes, the Stream Condition Index shall not apply in the South Florida Nutrient Watershed Region.

(29) “Study list” shall mean the list of surface waters or segments, as identified in Rule 62-303.390, F.A.C., that do not attain surface water quality standards, but the cause of nonattainment is unknown and requires further study to identify the cause of nonattainment, or exhibit a clear adverse trend in nutrients or nutrient response variables where a site specific numeric interpretation has not been established pursuant to paragraph 63-302.531(2)(a), F.A.C.

(30) “Surface water” means those waters of the State upon the surface of the earth to their landward extent, whether contained in bounds created naturally or artificially or diffused. Water from natural springs shall be classified as surface water when it exits from the spring onto the earth’s surface.

(31) “Total Maximum Daily Load” (TMDL) for an impaired waterbody or waterbody segment shall mean the sum of the individual wasteload allocations for point sources and the load allocations for nonpoint sources and natural background. Prior to determining individual wasteload allocations and load allocations, the maximum amount of a pollutant that a waterbody or waterbody segment can assimilate from all sources without exceeding water quality standards must first be calculated. A TMDL shall include either an implicit or explicit margin of safety and a consideration of seasonal variations.

(32) “Verified list” shall mean the list of impaired water bodies or segments for which TMDLs will be calculated, as provided in subsection 403.067(4), F.S., and which will be submitted to EPA pursuant to paragraph 303(d)(1) of the CWA.

(33) “Water quality criteria” shall mean elements of State water quality standards, expressed as constituent concentrations, levels, or narrative statements, representing a quality of water that supports the present and future most beneficial uses.

(34) “Water quality standards” shall mean standards composed of designated present and future most beneficial uses (classification of waters), the numerical and narrative criteria, including Site Specific Alternative Criteria, applied to the specific water uses or classification, the Florida antidegradation policy, and the moderating provisions, such as variances, mixing zone rule provisions, or exemptions.

(35) “Water segment” shall mean a portion of a waterbody that the Department will assess and evaluate for purposes of determining whether a TMDL will be required. Water segments previously evaluated as part of the Department’s 1998 305(b) Report are depicted in the map titled “Water Segments of Florida,” which is incorporated by reference.

(36) “Waters” shall be those surface waters described in Section 403.031(13), F.S.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 6-10-02, Amended 6-5-06, 12-11-06, 7-2-12.