- 62-303.330 Biological Assessment.

(1) Biological data must meet the requirements of subsections (3) and (8) in Rule 62-303.320, F.A.C.

(2) Biological Health Assessments used to evaluate predominantly fresh water streams and lakes under this rule shall include the Stream Condition Index (SCI) the Lake Vegetation Index (LVI), and the Shannon-Weaver Diversity Index. BioRecons can also be used to evaluate predominantly fresh water streams under this rule. Because these Biological Health Assessment procedures require specific training and expertise, persons conducting a BioRecon, SCI or LVI must comply with the quality assurance requirements of Chapter 62-160, F.A.C. (including adherence to Sampling and Use of the Stream Condition Index (SCI) for Assessing Flowing Waters: A Primer (DEP-SAS-001/11), which was incorporated by reference in subsection 62-303.200(29), F.A.C., and Sampling and Use of the Lake Vegetation Index (LVI) for Assessing Lake Plant Communities in Florida: A Primer (DEP-SAS-002/11), which was incorporated by reference in subsection 62-303.200(9), F.A.C.), attend at least eight hours of Department field training and pass a Department field audit that verifies the sampler follows the applicable SOPs, as set forth in Chapter 62-160, F.A.C., before their Biological Health Assessment data will be considered valid for use under this rule.

(3) A water segment shall be included on the planning list if it meets any of the following conditions:

(a) One of the two most recent Shannon-Weaver Diversity Index (subsection 62-302.530(10), F.A.C.) scores is less than 75 percent of the value from an appropriate control site.

(b) One of the two most recent Stream Condition Index scores is:

1. A score of < 35; or

2. A 20 point reduction from the historic maximum value if the historic maximum value SCI is above 64.

(c) One of the two most recent BioRecon scores is ≤ 4.

(d) One of the two most recent Lake Vegetation Index scores is:

1. A score < 43; or

2. A 20 point reduction from the historic maximum value if the historic maximum value LVI is above 78.

(4) The “historic maximum value” shall be the highest mean of any three consecutive, temporally independent Stream Condition Index (SCI) scores or Lake Vegetation Index (LVI) scores at the same location that are collected prior to the most recent sample being considered for evaluation with this provision. To qualify as temporally independent samples, each Biological Health Assessment shall be conducted at least three months apart. Biological Health Assessments collected at the same water segment less than three months apart shall be considered one sample, with the mean value used to represent the sampling period.

(5) Other information relevant to the biological health of the water segment, including toxicity tests and information about alterations in the type, nature, or function of a waterbody, shall also be considered when assessing aquatic life use support.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 6-10-02, Amended 12-11-06, 7-2-12.