62-303.390 The Study List.

(1) The Study List contains waters where evidence indicates nonattainment of water quality standards, but the Department does not have enough information to determine the causative pollutant(s) and therefore cannot determine the appropriate remedy, and waters where a site specific numeric interpretation has not been established pursuant to paragraph 63-302.531(2)(a), F.A.C., and there is a clear adverse trend in nutrients or nutrient response variables. Causes of nonattainment can include excess pollutant loading or concentrations, habitat or hydrologic alterations, or natural conditions. Waters that do not attain water quality standards due to natural conditions pursuant to paragraph 62-303.420(1)(b), F.A.C., shall not be added to the Study List. To conform to the expectations of Section 303(d) of the Federal Clean Water Act and federal regulations at 40 C.F.R. 130.7(b), waters and associated parameters indentified in the Study List will be submitted to EPA as water quality limited segments. However, pursuant to paragraph 403.067(2)(a), F.S., the Study List cannot be used in the administration or implementation of any regulatory program. A TMDL shall not be established for a waterbody placed on the Study List pursuant to subsection 62-303.390(2), F.A.C., until such time as it is placed on the verified list pursuant to Part IV of this Chapter.

(2) A Class I, II, or III water shall be placed on the study list if:

(a) For waters with a statistically-significant increasing trend in TN, TP, nitrate-nitrite, or chlorophyll a pursuant to subsection 62-303.351(5), 62-303.352(3), 62-303.353(4), or 62-303.354(3), F.A.C., the Department confirms there is:

1. A statistically-significant (at the 95 percent confidence level) temporal trend in the annual geometric means after controlling for or removing the effects of confounding variables, such as climatic and hydrologic cycles, seasonality, quality assurance issues, and changes in analytical methods or method detection limits; and

2. A reasonable expectation that the water will become impaired within 10 years, taking into consideration the current concentrations of nutrients or nutrient response variables and the slope of the trend.

(b) A waterbody segment does not achieve the Biological Health Assessment provisions in Rule 62-303.430, F.A.C., but a cause has not been identified;

(c) A waterbody segment is verified as not meeting the dissolved oxygen criterion pursuant to Part IV of this Chapter, but a cause has not been identified;

(d) A waterbody segment where pollution control mechanisms are in place or planned that meet the requirements of Rule 62-303.600, F.A.C., except that there is uncertainty when water quality standards will be attained and the waterbody segment requires additional study; or

(e) For streams meeting the definition in subsection 62-302.200(36), F.A.C., the nutrient thresholds in subparagraph 62-302.531(2)(c)2., F.A.C., are exceeded based on data from the last 7.5 years and insufficient Biological Health Assessment, chlorophyll a, or other response variable data are available to fully assess achievement of the nutrient provisions in paragraph 62-302.531(2)(c), F.A.C.  A TMDL shall not be established for the waterbody prior to the collection of additional response variable data and the conclusion of the next assessment cycle.

(3) Waters that fall under paragraph 62-303.390(2)(a), F.A.C., and do not have a site specific numeric interpretation of the narrative pursuant to paragraph 62-302.351(2)(a), F.A.C., shall be removed from the Study List upon development of a site-specific interpretation of the narrative nutrient criteria for the waterbody. Those waters subject to a site specific interpretation of the narrative that meet the provisions of subparagraph 62-303.390(2)(a)1., F.A.C., will be reevaluated by the Department to determine whether adjustments are necessary to provide for the attainment and maintenance of water quality standards in downstream waterbodies.

(4) For waters that fall under paragraph 62-303.390(2)(b), F.A.C., above, a stressor identification study shall be conducted to identify the causative pollutant(s) or other factor(s) responsible for nonattainment. A stressor identification study includes collection and analysis of physical, chemical, and biological data necessary to determine the causative pollutant(s) or other factor(s) causing nonattainment.

(5) It is the Department’s goal to collect the additional data needed for waters on the Study List as part of its watershed management approach, with the data collected during either the same cycle that the water is initially listed on the study list or during the subsequent cycle.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 7-2-12.