- 62-303.420 Aquatic Life-Based Water Quality Criteria Assessment.

(1) The Department shall reexamine the data used in Rule 62-303.320, F.A.C., to determine whether water quality criteria are met.

(a) If values exceeding the criteria are not due to pollutant discharges or reflect natural background conditions, including seasonal or other natural variations, the water shall not be listed on the verified list. In such cases, the Department shall note for the record why the water was not listed and provide the basis for its determination that the exceedances were not due to pollutant discharges.

(b) If the Department has information suggesting that the values not meeting the dissolved oxygen (DO) criterion are due to natural background conditions, it is the Department’s intent to support that conclusion through the use of Biological Health Assessment procedures referenced in Rule 62-303.330, F.A.C. The waterbody or segment shall not be included on the verified list for DO if two or more temporally independent Biological Health Assessments indicate the waterbody supports the protection and maintenance of a healthy, well-balanced population of fish and wildlife. In addition, the Biological Health Assessments shall be conducted in the same waterbody segment, or for streams, in the adjacent downstream waterbody segment where the water quality samples were taken.  These Biological Health Assessments shall be conducted on the same day or after the water quality samples were collected.

(2) If the water was listed on the planning list and there were insufficient data from the last five years preceding the planning list assessment to meet the data distribution requirements of subsection 62-303.320(4), F.A.C., and to meet a minimum sample size for verification of twenty samples, additional data will be collected as needed to provide a minimum sample size of twenty. Once these additional data are collected, the Department shall re-evaluate the data using the approach outlined in subsection 62-303.320(1), F.A.C., but using Table 3, and place waters on the verified list when 10% or more of the samples do not meet the applicable criteria, with a minimum of a 90% confidence level using a binomial distribution. The Department shall limit the analysis to data collected during the five years preceding the planning list assessment and the additional data collected pursuant to this paragraph. For sample sizes greater than 500, the Department shall calculate the number of samples not meeting the criterion that are needed for the given sample size using the binomial distribution.

Table 3: Verified List

| Minimum number of samples not meeting an applicable water quality criterion needed to put a water on the Verified list with at least 90% confidence. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sample sizes |  | Are listed if they have at least this # of samples that do not meet a criterion |  | Sample sizes |  | Are listed if they have at least this # of samples that do not meet a criterion |
| From | To |  |  | From | To |  |
| 20 | 25 | 5 |  | 254 | 262 | 33 |
| 26 | 32 | 6 |  | 263 | 270 | 34 |
| 33 | 40 | 7 |  | 271 | 279 | 35 |
| 41 | 47 | 8 |  | 280 | 288 | 36 |
| 48 | 55 | 9 |  | 289 | 297 | 37 |
| 56 | 63 | 10 |  | 298 | 306 | 38 |
| 64 | 71 | 11 |  | 307 | 315 | 39 |
| 72 | 79 | 12 |  | 316 | 324 | 40 |
| 80 | 88 | 13 |  | 325 | 333 | 41 |
| 89 | 96 | 14 |  | 334 | 343 | 42 |
| 97 | 104 | 15 |  | 344 | 352 | 43 |
| 105 | 113 | 16 |  | 353 | 361 | 44 |
| 114 | 121 | 17 |  | 362 | 370 | 45 |
| 122 | 130 | 18 |  | 371 | 379 | 46 |
| 131 | 138 | 19 |  | 380 | 388 | 47 |
| 139 | 147 | 20 |  | 389 | 397 | 48 |
| 148 | 156 | 21 |  | 398 | 406 | 49 |
| 157 | 164 | 22 |  | 407 | 415 | 50 |
| 165 | 173 | 23 |  | 416 | 424 | 51 |
| 174 | 182 | 24 |  | 425 | 434 | 52 |
| 183 | 191 | 25 |  | 435 | 443 | 53 |
| 192 | 199 | 26 |  | 444 | 452 | 54 |
| 200 | 208 | 27 |  | 453 | 461 | 55 |
| 209 | 217 | 28 |  | 462 | 470 | 56 |
| 218 | 226 | 29 |  | 471 | 479 | 57 |
| 227 | 235 | 30 |  | 480 | 489 | 58 |
| 236 | 244 | 31 |  | 490 | 498 | 59 |
| 245 | 253 | 32 |  | 499 | 500 | 60 |

(3) If the waterbody was placed on the planning list based on worst case values used to represent multiple samples taken during a four day period, the Department shall evaluate whether the worst case value should be excluded from the analysis pursuant to subsections (4) and (5). If the worst case value should not be used, the Department shall then re-evaluate the data following the methodology in subsection 62-303.420(2), F.A.C., using the more representative worst case value or, if all valid values are below acutely toxic levels, the median value.

(4) If the waterbody was listed on the planning list based on samples that do not meet water quality criteria for metals, the metals data shall be excluded if it is determined that the quality assurance requirements of subsection 62-303.320(8), F.A.C., were not met or that the sample was not collected and analyzed using clean techniques, if the use of clean techniques is appropriate. The Department shall re-evaluate the remaining valid data using the methodology in subsection 62-303.420(2), F.A.C., excluding any data that cannot be validated.

(5) Values that exceed possible physical or chemical measurement constraints (pH greater than 14, for example) or that represent data transcription errors, outliers the Department determines are not valid measures of water quality, water quality criteria exceedances due solely to violations of specific effluent limitations contained in state permits authorizing discharges to surface waters, water quality criteria exceedances within permitted mixing zones for those parameters for which the mixing zones are in effect, and water quality data collected following contaminant spills, discharges due to upsets or bypasses from permitted facilities, or rainfall in excess of the 25-year, 24-hour storm, shall be excluded from the assessment carried out under this rule. However, the Department shall note for the record that the data were excluded and explain why they were excluded.

(6) Once the additional data review is completed pursuant to subsections (1) through (5), the Department shall re-evaluate the data and shall include waters on the verified list that meet the criteria in subsection 62-303.420(2) or paragraph 62-303.320(6)(b), F.A.C.

(7) Notwithstanding the requirements of subsection (2), water segments shall also be included on the verified list if, based on representative data collected and analyzed in accordance with Chapter 62-160, F.A.C.:

(a) There are less than twenty samples, but there are five or more samples that do not meet an applicable water quality criterion based on data from at least five temporally independent sampling events, or

(b) Scientifically credible and compelling information regarding the magnitude, frequency, or duration of samples that do not meet an applicable water quality criterion provides overwhelming evidence of impairment.  Any determinations to list waters based on this provision shall be documented, and the documentation shall include the basis for the decision.

(c) For any water chemistry data used to list waters under this paragraph, the Department shall include in the administrative record all of the applicable data quality assessment elements listed in Table 2 of the Department’s Guidance Document “Data Quality Assessment Elements for Identification of Impaired Surface Waters” (DEP EAS 01-01, April 2001).

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.021(11), 403.062, 403.067 FS. History–New 6-10-02, Amended 12-11-06, 7-2-12.