- 25-7.070 Sealing Meters.

All meters tested for installation shall be sealed at the time of the test by the meter personnel performing the test. The seal shall be of a type that will ensure detection of tampering. Those utilities using a compression type lead seal shall have as a sealing tool a device furnished with a die, which shall bear the initials of the utility. Utilities using a snap tin type seal shall have the seal stamped in a similar manner.

Rulemaking Authority 366.05(1), 350.127(2), 368.03, 368.05(2) FS. Law Implemented 366.05(3), 368.03, 368.05 FS. History–New 2-13-84, Formerly 25-7.70, Amended 10-11-12.