9B-72.070 Product Evaluation and Quality Assurance for Optional Statewide Approval.

(1) Method 1. Products specifically addressed in the code through performance criteria and standardized testing or comparative or rational analysis methods, which cannot be approved through the plan review and inspection process, shall demonstrate compliance with the Code through one of the following:

(a) A certification mark or listing from an approved certification agency indicating that the product is in compliance with the Code. Indication of compliance shall be by certification to standards adopted by the Code.

(b) A test report from an approved testing laboratory, which identifies which products are covered by the test report and provides verifiable documentation indicating the product tested complies with the Code.

(c) An evaluation report from an approved product evaluation entity that covers the subject product and, based upon testing or comparative or rational analysis, or a combination thereof, indicates that the product was evaluated to be in compliance with the Code and that the product is, for the purpose intended, at least equivalent to that required by the Code.

(d) An evaluation report from a Florida Registered Architect or a Florida Professional Engineer developed and signed and sealed, based upon testing or comparative or rational analysis, or a combination thereof and indicates that the product was evaluated to be in compliance with the Code and that the product is, for the purpose intended, at least equivalent to that required by the Code.

(2) Method 2. Products for which there are no specific standardized tests or comparative or rational analysis methods of evaluation established as required by the Code shall demonstrate compliance with the intent of the Code through one of the following:

(a) An evaluation report from an approved product evaluation entity and provides verifiable documentation indicating the product complies with the intent of the Code.

(b) An evaluation report from a Florida Registered Architect or a Florida Professional Engineer based on testing or comparative or rational analysis or combination thereof, which provides verifiable documentation indicating the product complies with the intent of the Code.

(3) Products listed in subsections 9B-72.060(1)-(8), F.A.C., shall be manufactured under a quality assurance program audited by an approved quality assurance entity.

(4) Evaluation Report and Test Report Documentation Requirements. All reports and documentation required in subsections 9B-72.070(1) and (2), F.A.C., shall contain the information listed below. Design drawings submitted for permitting purposes are not to be construed to be an evaluation report and do not require this information.

(a) Name and address of the manufacturer, evaluation entity, engineer or architect or testing laboratory.

(b) Statement of compliance with the appropriate section or standard of the Code.

(c) Description of the product, construction method or building system.

(d) Technical documentation supporting the compliance statement.

(e) Installation requirements.

(f) Limitations and conditions of use.

(g) Certification of independence in conformance with Rule 9B-72.110, F.A.C.

(h) Name, title and signature of person authorized to sign on behalf of entity or signature, registration number and seal in the case of architects and engineers.

(5) The following documents shall be submitted for approval of listed products or products bearing a certification mark or an approved certification agency as complying with the standards established by the Code:

(a) Name and address of manufacturer and certification agency;

(b) Manufacturer’s product model number or identifier and product description;

(c) Indication of the standard the product was tested to;

(d) Performance level of the product and conditions or limitations of use.

Specific Authority 553.842(1) FS. Law Implemented 553.842(6) FS. History–New 5-5-02, Amended 9-4-03.