- 62-304.335 Perdido Bay Basin TMDLs.

(1) Fecal Coliform TMDL for Elevenmile Creek (US 90). The Total Maximum Daily Load is 400 counts/100 ml and is allocated as follows:

(a) The Wasteload Allocation (WLA) for wastewater point sources subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Permitting Program is to meet the Class III water quality criteria for fecal coliform in Chapter 62-302, F.A.C.;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1972 to 2006 period, will require a 63 percent reduction at sources contributing to exceedances of the criteria;

(c) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1972 to 2006 period, will require a 63 percent reduction at sources contributing to exceedances of the criteria;

(d) The Margin of Safety is implicit;

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(2) Fecal Coliform TMDL for Elevenmile Creek (State Road 297A). The Total Maximum Daily Load is 400 counts/100 ml and is allocated as follows:

(a) The WLA for wastewater point sources subject to the Department’s National Pollutant Discharge Elimination System Permitting Program is to meet the Class III water quality criteria for fecal coliform in Chapter 62-302, F.A.C.;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1972 to 2006 period, will require a 66 percent reduction at sources contributing to exceedances of the criteria;

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1972 to 2006 period, will require a 66 percent reduction at sources contributing to exceedances of the criteria;

(d) The Margin of Safety is implicit;

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(3) Fecal Coliform TMDL for Tenmile Creek. The Total Maximum Daily Load for Fecal Coliforms for Tenmile Creek is 400 counts/100 ml and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1972 to 2006 period, will require a 43 percent reduction at sources contributing to exceedances of the criteria;

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1972 to 2006 period, will require a 43 percent reduction at sources contributing to exceedances of the criteria;

(d) The Margin of Safety is implicit;

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(4) Fecal Coliform TMDL for Brushy Creek. The TMDL for Brushy Creek is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is not applicable;

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2004 to 2011 period, will require a 64 percent reduction of sources contributing to exceedances of the criteria; and,

(d) The Margin of Safety is implicit.

(e) While the LA for fecal coliform has been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from all anthropogenic sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 6-3-08, Amended 11-14-12.