- 62-304.435 Upper East Coast Basin TMDLs.

(1) Spruce Creek (Freshwater Segment). The Total Maximum Daily Load for the freshwater segment of Spruce Creek is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation (WLA) for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1997 to 2005 period, will require a 53 percent reduction at sources contributing to exceedances of the criteria,

(b) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1997 to 2005 period, will require a 53 percent reduction at sources contributing to exceedances of the criteria,

(c) The Margin of Safety is implicit,

(d) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(2) Spruce Creek (Marine Segment). The Total Maximum Daily Load for the marine segment of Spruce Creek is based on achieving the Class 3 marine minimum dissolved oxygen criterion of 4.0 mg/L, and is allocated as follows:

(a) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 25 percent reduction of current anthropogenic 5 day biochemical oxygen demand (BOD5) loading, and a 27 percent reduction of current anthropogenic total phosphorus (TP) loading based on measured concentrations from the 1992 to 2005 period,

(b) The LA for nonpoint sources is a 25 percent reduction of current anthropogenic 5 day biochemical oxygen demand (BOD5) loading, and a 27 percent reduction of current anthropogenic total phosphorus (TP) loading based on measured concentrations from the 1992 to 2005 period,

(c) The Margin of Safety is implicit.

(3) Fecal Coliform TMDL for Pellicer Creek. The TMDL for Pellicer Creek is 43 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is not applicable,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from 2004, 2005, and 2009, will require a 94 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA for fecal coliform has been expressed as the percent reductions needed to attain the applicable Class II criteria, it is the combined reductions from all anthropogenic sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 6-3-08, Amended 11-14-12.