- 62-304.330 Pensacola Bay Basin TMDLs.

(1) Fecal Coliform TMDL for Bayou Chico, Jones Creek, Jackson Creek, Bayou Chico Beach and Sanders Beach. The Total Maximum Daily Load is 400 counts/100 ml and is allocated as follows:

(a) A Wasteload Allocation (WLA) for wastewater point sources is not applicable.

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1998 to 2005 period, will require a 61 percent reduction at sources contributing to exceedances of the criteria.

(c) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1998 to 2005 period, will require a 61 percent reduction at sources contributing to exceedances of the criteria.

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(2) The Blackwater River (Tidal) Fecal Coliform TMDL. The fecal coliform Total Maximum Daily Load for the Blackwater River (Tidal) is 400 counts/100 mL, and is allocated as follows:

(a) The WLA for the Milton Wastewater Treatment Facility (Permit Number FL0021903) is that the facility must meet its permit limits for fecal coliform.

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 7 percent reduction at sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 7 percent reduction at sources contributing to exceedances of the criteria,

(d) The Margin of Safety is implicit, and

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal coliform concentrations. However, it is not the intent of these TMDLs to abate natural background conditions.

(3) The East Bay River (Marine Portion) Fecal Coliform TMDL. The TMDL for the East Bay River (Marine Portion) is 43 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 92 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 92 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class II criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(4) The Escambia River Fecal Coliform TMDL. The TMDL for the Escambia River is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 5 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 5 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(5) Texar Bayou Fecal Coliform TMDL. The TMDL for Texar Bayou is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 49 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 49 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(6) Carpenter Creek Fecal Coliform TMDL. The TMDL for Carpenter Creek is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from 2006 and 2012, will require a 28 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from 2006 and 2012, will require a 28 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(7) Turkey Creek Fecal Coliform TMDL. The TMDL for Turkey Creek is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is not applicable,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from 2006, 2007, and 2009, will require a 73 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(8) Yellow River Fecal Coliform TMDL. The TMDL for the Yellow River is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 60 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2003 to 2011 period, will require a 60 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(9) Judges Bayou (freshwater). The TMDL for the freshwater sections of Judges Bayou is a 74 percent reduction in total nitrogen (TN) to address a dissolved oxygen impairment, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the TN target which, based on reference stream concentrations for the 2003-2010 period, will require a 74 percent reduction of TN for sources contributing to the dissolved oxygen impairment, and

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the TN target which, based on reference stream concentrations from the 2003 to 2010 period, will require a 74 percent reduction of TN for sources contributing to the dissolved oxygen impairment.

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN has been expressed as the percent reductions needed to attain the applicable Class III dissolved oxygen criterion, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream TN concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 6-3-08, Amended 11-14-12, 2-27-13.