DEPARTMENT OF ENVIRONMENTAL PROTECTION

RULE NO.: 	RULE TITLE: 
62-302.530	Table: Surface Water Quality Criteria
PURPOSE AND EFFECT: The proposed rule amendments revise the human health-based surface water quality criteria to account for updated fish consumption rates among Floridians. Criteria were derived using a probabilistic model.
SUMMARY: The Department proposes to amend Rule 62-302.530, F.A.C., to revise the surface water quality criteria that are human health-based. These revised criteria are proposed to reflect more recent fish consumption data and were derived using a probabilistic model.
OTHER RULES INCORPORATING THIS RULE: Chapter 62-302, F.A.C., is referenced by the following rules: 18-2.021, 62-4.160, 62-4.241, 62-4.242, 62-4.244, 62-4.246, 62-25.001, 62-25.025, 62-25.080, 62-40.120, 62-40.210, 62-45.070, 62-45.170, 62-110.106, 62-113.200, 62-301.100, 62-302.200, 62-302.300, 62-302.400, 62-302.500, 62-302.520, 62-302.530, 62-302.531, 62-302.532, 62-302.540, 62-302.700, 62-302.800, 62-303.100, 62-303.200, 62-303.320, 62-303.330, 62-303.351, 62-303.352, 62-303.353, 62-303.354, 62-303.370, 62-303.390, 62-303.400, 62-303.430, 62-303.450, 62-303.720, 62-304.310, 62-304.335, 62-312.400, 62-330.100, 62-330.200, 62-330.405, 62-330.410, 62-330.412, 62-330.630, 62-340.700, 62-346.050, 62-346.051, 62-346.301, 62-348.200, 62-520.200, 62-520.520, 62-528.610, 62-528.630, 62-600.120. 62-600.200, 62-600.300, 62-600.400, 62-600.430, 62-600.440, 62-600.500, 62-600.520, 62-610.200, 62-610.300, 62-610.310, 62-610.554, 62-610.555, 62-610.650, 62-610.670, 62-610.810, 62-610.820, 62-610.830, 62-610.850, 62-610.860, 62-611.110, 62-611.200, 62-611.450, 62-611.500, 62-611.600, 62-611.650, 62-611.700, 62-620.320, 62-620.620, 62-621.303, 62-624.800, 62-625.400, 62-640.400, 62-650.300, 62-660.300, 62-673.340, 62-673.610, 62-701.200, 62-709.500, 62-711.540, 62-761.200, 62-762.201, 62-770.200, 62-771.100, 62-777.150, 62-777.170, 62-780.200, 62-782.200, 62-785.200, 62B-49.008, 62B-49.012, 62C-16.0051, F.A.C. Rule 62-302.530 is referenced by the following rules: 62-4.244, 62-302.200, 62-302.400, 62-302.500, 62-302.530, 62-302.531, 62-302.532, 62-302.800, 62-303.200, 62-303.320, 62-303.330, 62-303.370, 62-303.430, 62-303.450, 62-303.720, 62-611.500, 62-611.600, 62B-49.008, F.A.C.
EFFECT ON THOSE OTHER RULES: The proposed amendments revise the surface water quality criteria that are human health-based. These amendments will change the surface water quality criteria that industry, local government, agriculture and other surface water dischargers must meet to protect human health.
SUMMARY OF STATEMENT OF ESTIMATED REGULATORY COSTS AND LEGISLATIVE RATIFICATION: 
The Agency has determined that this will not have an adverse impact on small business or likely increase directly or indirectly regulatory costs in excess of $200,000 in the aggregate within one year after the implementation of the rule. A SERC has been prepared by the agency. The Agency prepared a SERC and a detailed cost estimate of costs that might be incurred by various entities with discharges to Florida waters, and concluded that there would be no predicted costs for additional impairments resulting from the adoption of the proposed human health-based criteria and that facilities would not be anticipated to have compliance issues or associated costs due to the Department’s proposed criteria.

The Agency has determined that the proposed rule is not expected to require legislative ratification based on the statement of estimated regulatory costs or if no SERC is required, the information expressly relied upon and described herein: The Department prepared a detailed cost estimate upon which it relied to determine whether legislative ratification would be required.
Any person who wishes to provide information regarding a statement of estimated regulatory costs, or provide a proposal for a lower cost regulatory alternative must do so in writing within 21 days of this notice.
RULEMAKING AUTHORITY: 403.061, 403.062, 403.087, 403.504, 403.704, 403.804 FS.
LAW IMPLEMENTED: 403.021, 403.061, 403.087, 403.088, 403.141, 403.161, 403.182, 403.502, 403.702, 403.708 FS.
A HEARING WILL BE HELD AT THE DATE, TIME AND PLACE SHOWN BELOW: 
DATE AND TIME: Tuesday, April 23, 2013, 9:00 a.m.
PLACE: Florida Department of Environmental Protection, Marjory Stoneman Douglas Building, Conference Room A, 3900 Commonwealth Boulevard, Tallahassee, Florida
Pursuant to the provisions of the Americans with Disabilities Act, any person requiring special accommodations to participate in this workshop/meeting is asked to advise the agency at least 5 days before the workshop/meeting by contacting: Eric Shaw at (850)245-8429 or through the address or e-mail identified below. If you are hearing or speech impaired, please contact the agency using the Florida Relay Service, 1(800)955-8771 (TDD) or 1(800)955-8770 (Voice).
THE PERSON TO BE CONTACTED REGARDING THE PROPOSED RULE IS: Eric Shaw, Department of Environmental Protection, Bureau of Assessment and Restoration Support, MS 6511, 2600 Blair Stone Road, Tallahassee, FL 32399-2400, (850)245-8429 or e-mail: eric.shaw@dep.state.fl.us

THE FULL TEXT OF THE PROPOSED RULE IS:

- 62-302.530 Table: Surface Water Quality Criteria.

The following table contains both numeric and narrative surface water quality criteria to be applied except within zones of mixing. The left-hand column of the Table is a list of constituents for which a surface water criterion exists. The headings for the water quality classifications are found at the top of the Table, and the classification descriptions for the headings are specified in subsection 62-302.400(1), F.A.C. Applicable criteria lie within the Table. The individual criteria should be read in conjunction with other provisions in water quality standards, including Rule 62-302.500, F.A.C. The criteria contained in Rule 62-302.500, F.A.C., also apply to all waters unless alternative or more stringent criteria are specified in Rule 62-302.530, F.A.C.  Unless otherwise stated, all criteria express the maximum not to be exceeded at any time. In some cases, there are separate or additional limits, which apply independently of the maximum not to be exceeded at any time. For example, annual average (denoted as “annual avg.” in the Table) means the maximum concentration at average annual flow conditions (see subsection 62-302.200(2), F.A.C.). Numeric interpretations of the narrative nutrient criterion in paragraph 62-302.530 (47)(b), F.A.C., shall be expressed as spatial averages and applied over a spatial area consistent with their derivation.  In applying the water quality standards, the Department shall take into account the variability occurring in nature and shall recognize the statistical variability inherent in sampling and testing procedures. The Department’s assessment methodology, set forth in Chapter 62-303, F.A.C., accounts for such natural and statistical variability when used to assess ambient waters pursuant to sections 305(b) and 303(d) of the Federal Clean Water Act.

| Parameter | Units | Class I | Class II | Class III and Class III-Limited (see Note 4) |  |  |  | Class IV |  | Class V |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Predominantly Fresh Waters | Predominantly Marine Waters |  |  |  |  |  |  |  |
| (1) Acrolein | Micrograms/L | ≤ 0.97 monthly avg.; 3 max | ≤ 1.4 monthly avg. | ≤ 1.4 monthly avg.; 3 max |  | ≤ 1.4 monthly avg. |  |  |  |  |  |  |
| (2) Acrylonitrile | Micrograms/L | ≤ 0.068 annual avg. | ≤ 0.41 annual avg. | ≤ 0.41 annual avg. |  | ≤ 0.41 annual avg. |  |  |  |  |  |  |
| (1) through (3) renumbered (3) through (5) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (6) (4) Antimony | Micrograms/L | ≤ 3.7 monthly avg. ≤14.0 | ≤ 270 monthly avg. ≤ 4,300 | ≤ 270 monthly avg. ≤ 4,300 |  | ≤ 270 monthly avg. ≤ 4,300 |  |  |  |  |  |  |
| (5) through (7) renumbered (7) through (9) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (10) (8) Benzene | Micrograms/L | ≤ 2.9 annual avg. ≤ 1.18 | ≤ 86 ≤ 71.28 annual avg. | ≤ 86 ≤ 71.28 annual avg. |  | ≤ 86 ≤ 71.28 annual avg. |  |  |  |  |  |  |
| (11) Benzidine | Micrograms/L | ≤ 0.00012 annual avg. | ≤ 0.00033 annual avg. | ≤ 0.00033 annual avg. |  | ≤ 0.00033 annual avg. |  |  |  |  |  |  |
| (12) (9) Beryllium | Micrograms/L | ≤ 4.0 ≤ 0.0077 annual avg. See Note (2). | ≤ 36 monthly avg. ≤ 0.13 annual avg. See Note (2). | ≤ 36 monthly avg. ≤ 0.13 annual avg. See Note (2). |  | ≤ 36 monthly avg. ≤ 0.13 annual avg. See Note (2). | ≤ 100 in waters with a hardness in mg/L of CaCO3 of less than 250 and shall not exceed 500 in harder waters |  |  |  |  |  |
| (10) renumbered (13) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (14) Bis (2-Chloro-ethyl) Ether | Micrograms/L | ≤ 0.039 annual avg. | ≤ 0.88 annual avg. | ≤ 0.88 annual avg. |  | ≤ 0.88 annual avg. |  |  |  |  |  |  |
| (15) Bis (2-Chloro-isopropyl) Ether | Micrograms/L | ≤ 185 monthly avg. | ≤ 9,500 monthly avg. | ≤ 9,500 monthly avg. |  | ≤ 9,500 monthly avg. |  |  |  |  |  |  |
| (11) through (15) renumbered (16) through (20) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (16) Carbon tetrachloride | Micrograms/L | ≤ 0.25 annual avg.; 3.0 max | ≤ 4.42 annual avg. | ≤ 4.42 annual avg. |  | ≤ 4.42 annual avg. |  |  |  |  |  |  |
| (17) through (18) renumbered (21) through (22) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (23) Chloro-benzene | Micrograms/L | ≤ 89 monthly avg.; See Note (2). | ≤ 1,140 monthly avg. See Note (2). | ≤ 1,140 monthly avg. See Note (2). |  | ≤ 1,140 monthly avg. See Note (2). |  |  |  |  |  |  |
| (24) 2-Chloro-napththalene | Micrograms/L | ≤ 160 monthly avg. See Note (2). | ≤ 230 monthly avg. See Note (2). | ≤ 230 monthly avg. See Note (2). |  | ≤ 230 monthly avg. See Note (2). |  |  |  |  |  |  |
| (19) through (26) renumbered (25) through (32) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (33) 1,2-Dichloro-benzene | Micrograms/L | ≤ 320 monthly avg. See Note (2). | ≤ 950 monthly avg. See Note (2). | ≤ 950 monthly avg. See Note (2). |  | ≤ 950 monthly avg. See Note (2). |  |  |  |  |  |  |
| (34) 3,3’-Dichloro-benzidine | Micrograms/L | ≤ 0.028 annual avg. | ≤ 0.040 annual avg. | ≤ 0.040 annual avg. |  | ≤ 0.040 annual avg. |  |  |  |  |  |  |
| (35) 1,2-Dichloro-ethane | Micrograms/L | ≤ 0.48 annual avg. | ≤ 61 annual avg. | ≤ 61 annual avg. |  | ≤ 61 annual avg. |  |  |  |  |  |  |
| (36) (27) 1,1-Dichloroethylene (1,1-dichloroethene) | Micrograms/L | < 0.057 annual avg.; < 7.0 max See Note (2). | ≤ 5,300 monthly avg. < 3.2 annual avg. See Note (2). | ≤ 5,300 monthly avg. < 3.2 annual avg. See Note (2). |  | ≤ 5,300monthly avg. < 3.2 annual avg. See Note (2). |  |  |  |  |  |  |
| (37) 1,2 (trans) dichloroethylene | Micrograms/L | ≤ 93 monthly avg.; | ≤ 7,450 monthly avg. | ≤ 7,450 monthly avg. |  | ≤ 7,450 monthly avg. |  |  |  |  |  |  |
| (28) Dichloro-methane (methylene chloride) | Micrograms/L | ≤4.65 annual avg. | < 1,580 annual avg. | < 1,580 annual avg. |  | < 1,580 annual avg. |  |  |  |  |  |  |
| (38) 1,2-Dichloro-propane | Micrograms/L | ≤ 1.2 annual avg. | ≤ 45 annual avg. | ≤ 45 annual avg. |  | ≤ 45 annual avg. |  |  |  |  |  |  |
| (39) 1,3-Dichloro-propene | Micrograms/L | ≤ 0.44 annual avg. | ≤ 35 annual avg. | ≤ 35 annual avg. |  | ≤ 35 annual avg. |  |  |  |  |  |  |
| (40) (29) 2,4-Dinitrotoluene | Micrograms/L | ≤ 0.14 < 0.11 annual avg. | ≤ 5.6 < 9.1 annual avg. | ≤ 5.6 < 9.1 annual avg. |  | ≤ 5.6 < 9.1 annual avg. |  |  |  |  |  |  |
| (41) 1,2-Diphenyl-hydrazine | Micrograms/L | ≤ 0.048 annual avg. | ≤ 0.34 annual avg. | ≤ 0.34 annual avg. |  | ≤ 0.34 annual avg. |  |  |  |  |  |  |
| (30) through (31) renumbered (42) through (43) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (44) Ethylbenzene | Micrograms/L | ≤ 390 monthly avg.; See Note (2). | ≤ 1,600 monthly avg. See Note (2). | ≤ 1,600 monthly avg. See Note (2). |  | ≤ 1,600 monthly avg. See Note (2). |  |  |  |  |  |  |
| (32) through (34) renumbered (45) through (47) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (48) (35) (a) Halomethanes (Total trihalomethanes) (total of bromoform, dibromochloro-methane chlorodibromo-methane, bromodichloro-methane dichlorobromo – methane, and chloroform). Individual halomethanes shall not exceed (b)1. to (b) 8. 5. below. | Micrograms/L | ≤ 80 |  |  |  |  |  |  |  |  |  |  |
| (48) (35) (b) 1. Halomethanes (individual): Bromoform | Micrograms/L | ≤ 9.5 ≤ 4.3 annual avg. | ≤ 390 < 360 annual avg. | ≤ 390 < 360 annual avg. |  | ≤ 390 < 360 annual avg. |  |  |  |  |  |  |
| (48) (b) 2. Halomethanes (individual): Carbon tetrachloride (tetrachloro-methane) | Micrograms/L | ≤ 0.56 annual avg.; | ≤ 5.1 annual avg. | ≤ 5.1 annual avg. |  | ≤ 5.1 annual avg. |  |  |  |  |  |  |
| (48) (35) (b) 3. 2. Halomethanes (individual): Dibromochloro-methane Chlorodibromo-methane | Micrograms/L | ≤ 1.0 ≤ 0.41 annual avg. | ≤ 41 < 34 annual avg. | ≤ 41 < 34 annual avg. |  | ≤ 41 < 34 annual avg. |  |  |  |  |  |  |
| (48) (35) (b) 4. 3. Halomethanes (individual): Chloroform | Micrograms/L | ≤ 46 monthly avg. ≤5.67 annual avg. See Note (2). | ≤ 1,600 monthly avg. < 470.8 annual avg. See Note (2). | ≤ 1,600 monthly avg. < 470.8 annual avg. See Note (2). |  | ≤ 1,600 monthly avg. < 470.8 annual avg. See Note (2). |  |  |  |  |  |  |
| (48) (35) (b) 5. 4. Halomethanes (individual): Chloromethane (methyl chloride) | Micrograms/L | ≤ 5.67 annual avg. | ≤ 470.8 annual avg. | ≤ 470.8 annual avg. |  | ≤ 470.8 annual avg. |  |  |  |  |  |  |
| (48) (35) (b) 6. 5. Halomethanes (individual): Bromodichloro-methane Dichlorobromo-methane | Micrograms/L | ≤ 1.2 ≤ 0.27 annual avg. | ≤ 51 ≤ 22 annual avg. | ≤ 51 ≤ 22 annual avg. |  | ≤ 51 ≤ 22 annual avg. |  |  |  |  |  |  |
| (48) (b) 7. Halomethanes (individual): Dichloromethane (methylene chloride) | Micrograms/L | ≤ 5.0 | ≤ 3,700 annual avg. | ≤ 3,700 annual avg. |  | ≤ 3,700 annual avg. |  |  |  |  |  |  |
| (48) (b) 8. Halomethanes (individual): Methyl Bromide | Micrograms/L | ≤ 92 monthly avg. See Note (2). | ≤ 3,100 monthly avg. See Note (2). | ≤ 3,100 monthly avg. See Note (2). |  | ≤ 3,100 monthly avg. See Note (2). |  |  |  |  |  |  |
| (49) (36) Hexachloro-butadiene | Micrograms/L | ≤ 0.88 ≤ 0.45 annual avg. | ≤ 4.1 ≤ 49.7 annual avg. | ≤ 4.1 ≤ 49.7 annual avg. |  | ≤ 4.1 ≤ 49.7 annual avg. |  |  |  |  |  |  |
| (50) Hexachloro-cyclopentadiene | Micrograms/L | ≤ 27 monthly avg. See Note (2). | ≤ 810 monthly avg. See Note (2). | ≤ 810 monthly avg. See Note (2). |  | ≤ 810 monthly avg. See Note (2). |  |  |  |  |  |  |
| (51) Hexachloro-ethane | Micrograms/L | ≤ 0.70 annual avg. | ≤ 1.9 annual avg. | ≤ 1.9 annual avg. |  | ≤ 1.9 annual avg. |  |  |  |  |  |  |
| (37) through (38) renumbered (52) through (53) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (54) Isophorone | Micrograms/L | ≤ 45 annual avg. | ≤ 1,600 annual avg. | ≤ 1,600 annual avg. |  | ≤ 1,600 annual avg. |  |  |  |  |  |  |
| (39) through (45) renumbered (55) through (61) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (62) Nitrobenzene | Micrograms/L | ≤ 9.2 monthly avg. See Note (2). | ≤ 410 monthly avg. See Note (2). | ≤ 410 monthly avg. See Note (2). |  | ≤ 410 monthly avg. See Note (2). |  |  |  |  |  |  |
| (63) N-Nitroso-dimethylamine | Micrograms/L | ≤ 0.00087 annual avg. | ≤ 5.0 annual avg. | ≤ 5.0 annual avg. |  | ≤ 5.0 annual avg. |  |  |  |  |  |  |
| (64) N-Nitrosodi-n-propylamine | Micrograms/L | ≤ 0.0063 annual avg. | ≤ 0.84 annual avg. | ≤ 0.84 annual avg. |  | ≤ 0.84 annual avg. |  |  |  |  |  |  |
| (65) N-Nitrosodi-phenylamine | Micrograms/L | ≤ 4.8 annual avg. | ≤ 10 annual avg. | ≤ 10 annual avg. |  | ≤ 10 annual avg. |  |  |  |  |  |  |
| (46) through (49) renumbered (66) through (69) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (70) (50) Pesticides and Herbicides |  |  |  |  |  |  |  |  |  |  |  |  |
| (70) (50) (a) 2,4,5-TP | Micrograms/L | ≤ 10 |  |  |  |  |  |  |  |  |  |  |
| (70) (50) (b) 2-4-D | Micrograms/L | ≤ 100 |  |  |  |  |  |  |  |  |  |  |
| (70) (50) (c) Aldrin | Micrograms/L | ≤ 0.000069 ≤ .00013 annual avg.; 3.0 max | ≤ 0.000071 < .00014 annual avg.; 1.3 max | ≤ 0.000071 < .00014 annual avg.; 3.0 max |  | ≤ 0.000071 < .00014 annual avg.; 1.3 max |  |  |  |  |  |  |
| (70) (d) (50) (d) Beta-hexachlorocyclohexane (b-BHC) | Micrograms/L | ≤ 0.013 ≤ 0.014 annual avg. | ≤ 0.029 < 0.046 annual avg. | ≤ 0.029 < 0.046 annual avg. |  | ≤ 0.029 < 0.046 annual avg. |  |  |  |  |  |  |
| (70) (e) (50) (e) Chlordane | Micrograms/L | ≤ 0.0011 ≤ 0.00058 annual avg.; 0.0043 max | ≤ 0.0011 < 0.00059 annual avg.; 0.004 max | ≤ 0.0011 < 0.00059 annual avg.; 0.0043 max |  | ≤ 0.0011 < 0.00059 annual avg.; 0.004 max |  |  |  |  |  |  |
| (70) (f) (50) (f) DDT | Micrograms/L | ≤ 0.00031 ≤ 0.00059 annual avg.; ≤ 0.001 max | ≤ 0.00031 ≤ 0.00059 annual avg.; ≤ 0.001 max | ≤ 0.00031 ≤ 0.00059 annual avg.; ≤ 0.001 max |  | ≤ 0.00031 ≤ 0.00059 annual avg.; ≤ 0.001 max |  |  |  |  |  |  |
| (70) (g) (50) (g) Demeton | Micrograms/L | ≤ 0.1 | ≤ 0.1 | ≤ 0.1 |  | ≤ 0.1 |  |  |  |  |  |  |
| (70) (h) (50) (h) Dieldrin | Micrograms/L | ≤ 0.000073 ≤ 0.00014 annual avg.; 0.056 0.0019 max | ≤ 0.000075 ≤ 0.00014 annual avg.; 0.0019 max | ≤ 0.000075 ≤ 0.00014 annual avg.; 0.056 0.0019 max |  | ≤ 0.000075 ≤ 0.00014 annual avg.; 0.0019 max |  |  |  |  |  |  |
| (70) (i) (50) (i) Endosulfan | Micrograms/L | ≤ 0.056 | ≤ 0.0087 | ≤ 0.056 |  | ≤ 0.0087 |  |  |  |  |  |  |
| (70) (j) Endosulfan sulfate | Micrograms/L | ≤ 0.056 | ≤ 0.0087 | ≤ 0.056 |  | ≤ 0.0087 |  |  |  |  |  |  |
| (70) (k) (50) (j) Endrin | Micrograms/L | ≤ 0.036 ≤ 0.0023 | ≤ 0.0023 | ≤ 0.036 ≤ 0.0023 |  | ≤ 0.0023 |  |  |  |  |  |  |
| (70) (l) (50) (k) Guthion | Micrograms/L | ≤ 0.01 | ≤ 0.01 | ≤ 0.01 |  | ≤ 0.01 |  |  |  |  |  |  |
| (70) (m) (50) (l) Heptachlor | Micrograms/L | ≤ 0.00011 ≤ 0.00021 annual avg.; 0.0038 max | ≤ 0.00011 ≤ 0.00021 annual avg.; 0.0036 max | ≤ 0.00011 ≤ 0.00021 annual avg.; 0.0038 max |  | ≤ 0.00011 ≤ 0.00021 annual avg.; 0.0036 max |  |  |  |  |  |  |
| (70) (n) Heptachlor epoxide | Micrograms/L | ≤ 0.000055 annual avg.; 0.0038 max | ≤ 0.000055 annual avg.; 0.0036 max | ≤ 0.000055 annual avg.; 0.0038 max |  | ≤ 0.000055 annual avg.; 0.0036 max |  |  |  |  |  |  |
| (70) (o) (50) (m) Lindane (G-benzene hexochloride | Micrograms/L | See Minimum criteria in paragraph 62-302.500(1)(d), F.A.C. < 0.019 annual avg.; 0.08 max | See Minimum criteria in paragraph 62-302.500(1)(d), F.A.C. < 0.063 annual avg.; 0.16 max | See Minimum criteria in paragraph 62-302.500(1)(d), F.A.C. < 0.063 annual avg.; 0.08 max |  | See Minimum criteria in paragraph 62-302.500(1)(d), F.A.C. < 0.063 annual avg.; 0.16 max |  |  |  |  |  |  |
| (70) (p) (50) (n) Malathion | Micrograms/L | ≤ 0.1 | ≤ 0.1 | ≤ 0.1 |  | ≤ 0.1 |  |  |  |  |  |  |
| (70) (q) (50) (o) Methoxychlor | Micrograms/L | ≤ 0.03 | ≤ 0.03 | ≤ 0.03 |  | ≤ 0.03 |  |  |  |  |  |  |
| (70) (r) (50) (p) Mirex | Micrograms/L | ≤ 0.001 | ≤ 0.001 | ≤ 0.001 |  | ≤ 0.001 |  |  |  |  |  |  |
| (70) (s) (50) (q) Parathion | Micrograms/L | ≤ 0.013 < 0.04 | ≤ 0.04 | ≤ 0.013 < 0.04 |  | ≤ 0.04 |  |  |  |  |  |  |
| (70) (t) (50) (r) Toxaphene | Micrograms/L | ≤ 0.0002 | ≤ 0.0002 | ≤ 0.0002 |  | ≤ 0.0002 |  |  |  |  |  |  |
| (51) renumbered (71) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (72) (52) (a) Phenolic Compounds: Total |  | Phenolic compounds other than those produced by the natural decay of plant material, listed or unlisted, shall not taint the flesh of edible fish or shellfish or produce objectionable taste or odor in a drinking water supply. |  |  |  |  |  |  |  |  |  |  |
| (72) (52) (b) Total Chlorinated Phenols and Chlorinated Cresols | Micrograms/L | 1. The total of all chlorinated phenols, and chlorinated cresols, except as set forth in (c) 1., (c)2., (c)5., (c)6., or (c)7. to (c) 4. below, shall not exceed 1.0 unless higher values are shown not to be chronically toxic. Such higher values shall be approved in writing by the Secretary. 2. The compounds listed in (c) 1. to (c) 6. below shall not exceed the limits specified for each compound. |  |  |  |  |  |  | 1. The total of the following Phenolic compounds shall not exceed 50: 1a) Chlorinated phenols; 2b) Chlorinated cresols; and 3c) 2,4-dinitrophenol. |  |  |  |
| (72) (52) (c) 1. Phenolic Compound: 2-chlorophenol | Micrograms/L | ≤ 13 monthly avg. ≤ 120 | ≤ 22 monthly avg. < 400 See Note (2). | ≤ 22 monthly avg. < 400 See Note (2). |  | ≤ 22 monthly avg. < 400 See Note (2). | < 400 See Note (2). |  |  |  |  |  |
| (72) (52) (c) 2. Phenolic Compound: 2,4-dichlorophenol | Micrograms/L | ≤ 11 monthly avg. ≤ 93 See Note (2). | ≤ 43 monthly avg. < 790 See Note (2). | ≤ 43 monthly avg. < 790 See Note (2). |  | ≤ 43 monthly avg. < 790 See Note (2). | < 790 See Note (2). |  |  |  |  |  |
| (72) (c) 3. Phenolic Compound: 2,4-dimethylphenol | Micrograms/L | ≤ 60 monthly avg. | ≤ 130 monthly avg. See Note (2). | ≤ 130 monthly avg. See Note (2). |  | ≤ 130 monthly avg. See Note (2). |  |  |  |  |  |  |
| (72) (c) 4. Phenolic Compound: 2-Methyl-4,6-Dinitrophenol | Micrograms/L | ≤ 1.8 monthly avg. | ≤ 42 monthly avg. | ≤ 42 monthly avg. |  | ≤ 42 monthly avg. |  |  |  |  |  |  |
| (72) (52) (c) 5. 3. Phenolic Compound: Pentachlorophenol | Micrograms/L | ≤ 30 max; ≤ 0.10 ≤ 0.28 annual avg.; ≤e(1.005[pH]-5.29) max | ≤ 1.5 annual avg. ≤ 7.9 max | ≤ 30 max; ≤ 1.5 ≤ 0.28 annual avg.; ≤e(1.005[pH]-5.29) max |  | ≤ 1.5 annual avg. ≤ 7.9 max | ≤ 30 |  |  |  |  |  |
| (72) (52) (c) 6. 4. Phenolic Compound: 2,4,6-trichlorophenol | Micrograms/L | ≤ 2.0 ≤ 2.1 annual avg. | ≤ 4.0 < 6.5 annual avg. | ≤ 4.0 < 6.5 annual avg. |  | ≤ 4.0 < 6.5 annual avg. | ≤ 6.5 annual avg. |  |  |  |  |  |
| (72) (52) (c) 7. 5. Phenolic Compound: 2,4-dinitrophenol | Micrograms/L Milligrams/L | ≤ 9.3 monthly avg. ≤ 0.0697 See Note (2). | ≤ 790 monthly avg. < 14.26 See Note (2). | ≤ 790 monthly avg. < 14.26 See Note (2). |  | ≤ 790 monthly avg. < 14.26 See Note (2). | ≤ 14.26 See Note (2). |  |  |  |  |  |
| (72) (52) (c) 8. 6. Phenolic Compound: Phenol | Milligrams/L | ≤ 0.3 See Note (2). | ≤ 0.3 See Note (2). | ≤ 0.3 See Note (2). |  | ≤ 0.3 See Note (2). | ≤ 0.3 |  |  |  |  | ≤ 0.3 |
| (53) renumbered (73) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (74) (54) Phthalate Esters | Micrograms/L | ≤ 3.0 |  | ≤ 3.0 |  |  |  |  |  |  |  |  |
| (74) (a) Butylbenzyl Phthalate | Micrograms/L | ≤ 210 monthly avg. See Note (2). | ≤ 240 monthly avg. See Note (2). | ≤ 240 monthly avg. See Note (2). |  | ≤ 240 monthly avg. See Note (2). |  |  |  |  |  |  |
| (74) (b) Diethyl Phthalate | Micrograms/L | ≤ 2,600 monthly avg. See Note (2). | ≤ 6,450 monthly avg. See Note (2). | ≤ 6,450 monthly avg. See Note (2). |  | ≤ 6,450 monthly avg. See Note (2). |  |  |  |  |  |  |
| (74) (c) Dimethyl Phthalate | Micrograms/L | ≤ 39,00 monthly avg. See Note (2). | ≤ 160,000 monthly avg. See Note (2). | ≤ 160,000 monthly avg. See Note (2). |  | ≤ 160,000 monthly avg. See Note (2). |  |  |  |  |  |  |
| (74) (d) Di-n-butyl Phthalate | Micrograms/L | ≤ 31 monthly avg. See Note (2). | ≤ 660 monthly avg. See Note (2). | ≤ 660 monthly avg. See Note (2). |  | ≤ 660 monthly avg. See Note (2). |  |  |  |  |  |  |
| (75) (55) Polychlorinated Biphenyls (PCBs) | Micrograms/L | ≤ 0.000090 ≤ 0.000044 annual avg.; 0.014 max | ≤ 0.000090 ≤ 0.000045 annual avg.; 0.03 max | ≤ 0.000090 ≤ 0.000045 annual avg.; 0.014 max |  | ≤ 0.000090 ≤ 0.000045 annual avg.; 0.03 max |  |  |  |  |  |  |
| (76) (56) (a) Polycyclic Aromatic Hydrocarbons (PAHs). Total of: Acenaphthylene; Benzo(a)anthracene; Benzo(a)pyrene; Benzo(b)fluoran-thene; Benzo-(ghi)perylene; Benzo(k)fluoranthene; Chrysene; Dibenzo-(a,h)anthracene; and Indeno(1,2,3-cd)pyrene; and Phenanthrene | Micrograms/L | ≤ 0.0051 ≤ 0.0028 annual avg. | ≤ 0.030 ≤ 0.031 annual avg. | ≤ 0.030 ≤ 0.031annual avg. |  | ≤ 0.030 ≤ 0.031 annual avg. |  |  |  |  |  |  |
| (76) (56) (b) 1. (Individual PAHs): Acenaphthene | Micrograms/L Milligrams/L | ≤ 110 monthly avg. < 1.2 See Note (2). | ≤ 150 monthly avg. < 2.7 See Note (2). | ≤ 150 monthly avg. < 2.7 See Note (2). |  | ≤ 150 monthly avg. < 2.7 See Note (2). |  |  |  |  |  |  |
| (76) (56) (b) 2. (Individual PAHs): Anthracene | Micrograms/L Milligrams/L | ≤ 1,200 monthly avg. < 9.6 See Note (2). | ≤ 5,900 monthly avg. < 110 See Note (2). | ≤ 5,900 monthly avg. < 110 See Note (2). |  | ≤ 5,900 monthly avg. < 110 See Note (2). |  |  |  |  |  |  |
| (76) (56) (b) 3. (Individual PAHs): Fluoranthene | Micrograms/L Milligrams/L | ≤ 16 monthly avg. < 0.3 See Note (2). | ≤ 17 monthly avg. < 0.370 See Note (2). | ≤ 17 monthly avg. < 0.370 See Note (2). |  | ≤ 17 monthly avg. < 0.370 See Note (2). |  |  |  |  |  |  |
| (76) (56) (b) 4. (Individual PAHs): Fluorene | Micrograms/L Milligrams/L | ≤ 160 monthly avg. < 1.3 See Note (2). | ≤ 790 monthly avg. < 14 See Note (2). | ≤ 790 monthly avg. < 14 See Note (2). |  | ≤ 790 monthly avg. < 14 See Note (2). |  |  |  |  |  |  |
| (76) (56) (b) 5. (Individual PAHs): Pyrene | Micrograms/L Milligrams/L | ≤ 120 monthly avg. < 0.96 See Note (2). | ≤ 590 monthly avg. < 11 See Note (2). | ≤ 590 monthly avg. < 11 See Note (2). |  | ≤ 590 monthly avg. < 11 See Note (2). |  |  |  |  |  |  |
| (57) through (58) renumbered (77) through (78) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (79) (59) Silver | Micrograms/L See Note (3). | ≤ 0.07 | See Minimum criteria in paragraph Section 62-302.500(1)(c), F.A.C. | ≤ 0.07 |  | See Minimum criteria in paragraph Section 62-302.500(1)(c), F.A.C. |  |  |  |  |  |  |
| (60) through (61) renumbered (80) through (81) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (82) (62) 1,1,2,2-Tetrachloroethane | Micrograms/L | ≤ 0.21 ≤ 0.17 annual avg. | ≤ 6.7 ≤ 10.8 annual avg. | ≤ 6.7 ≤ 10.8 annual avg. |  | ≤ 6.7 ≤10.8 annual avg. |  |  |  |  |  |  |
| (83) (63) Tetrachloroethene (Tetrachloro-ethylene (1,1,2,2-tetrachloroethene) | Micrograms/L | ≤ 5.0 ≤ 0.8 annual avg., ≤ 3.0 max | ≤ 100 ≤ 8.85 annual avg. | ≤ 100 ≤ 8.85 annual avg. |  | ≤ 100 ≤ 8.85 annual avg. |  |  |  |  |  |  |
| (84) (64) Thallium | Micrograms/L | ≤< 1.7 | ≤< 6.3 | ≤< 6.3 |  | ≤< 6.3 |  |  |  |  |  |  |
| (65) Thermal Criteria (See Section 62-302.520, F.A.C.) |  |  |  |  |  |  |  |  |  |  |  |  |
| (85) Toluene | Micrograms/L | ≤ 360 monthly avg. See Note (2). | ≤ 4,400 monthly avg. See Note (2). | ≤ 4,400 monthly avg. See Note (2). |  | ≤ 4,400 monthly avg. See Note (2). |  |  |  |  |  |  |
| (66) through (67) renumbered (86) through (87) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (88) 1,2,4-Tri-chlorobenzene | Micrograms/L | ≤ 28 monthly avg. | ≤ 52 monthly avg. | ≤ 52 monthly avg. |  | ≤ 52 monthly avg. |  |  |  |  |  |  |
| (89) 1,1,2-Tri-chloroethane | Micrograms/L | ≤ 0.76 annual avg. | ≤ 26 annual avg. | ≤ 26 annual avg. |  | ≤ 26 annual avg. |  |  |  |  |  |  |
| (90) (68) Trichloroethylene (trichloroethene) | Micrograms/L | ≤ 0.90 ≤ 2.7 annual avg., ≤ 3.0 max | ≤ 14 < 80.7 annual avg. | ≤ 14 < 80.7 annual avg. |  | ≤ 14 < 80.7 annual avg. |  |  |  |  |  |  |
| (69) renumbered (91) No change. |  |  |  |  |  |  |  |  |  |  |  |  |
| (92) Vinyl Chloride | Micrograms/L | ≤ 0.031 annual avg. | ≤ 4.1 annual avg. | ≤ 4.1 annual avg. |  | ≤ 4.1 annual avg. |  |  |  |  |  |  |
| (70) renumbered (93) No change. |  |  |  |  |  |  |  |  |  |  |  |  |

Rulemaking Authority 403.061, 403.062, 403.087, 403.504, 403.704, 403.804 FS. Law Implemented 403.021(11), 403.061, 403.087, 403.088, 403.141, 403.161, 403.182, 403.502, 403.702, 403.708 FS. History–New 1-28-90, Formerly 17-3.065, Amended 2-13-92, 6-17-92, Formerly 17-302.540, 17-302.550, 17-302.560, 17-302.570, 17-302.580, Amended 4-25-93, Formerly 17-302.530, Amended 1-23-95, 1-15-96, 5-15-02, 7-19-04, 12-7-06, 8-5-10, 7-3-12,________ .

NAME OF PERSON ORIGINATING PROPOSED RULE: Drew Bartlett
NAME OF AGENCY HEAD WHO APPROVED THE PROPOSED RULE: Herschel T. Vinyard Jr.
DATE PROPOSED RULE APPROVED BY AGENCY HEAD: March 22, 2013
DATE NOTICE OF PROPOSED RULE DEVELOPMENT PUBLISHED IN FAR: April 27, 2012