- 62-304.315 Chipola River Basin TMDLs.

(1) Otter Creek. The fecal coliform total maximum daily load (TMDL) for Otter Creek is 400 counts/100mL, and is allocated as follows:

(a) The Waste load Allocation (WLA) for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Stormwater Permitting Program is not applicable;

(c) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2008 period, will require a 35 percent reduction of sources contributing to exceedances of the criteria; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(2) Jackson Blue Spring and Merritts Mill Pond. The nitrate TMDL is an in-stream monthly mean concentration of 0.35 mg/L and is allocated as follows:

(a) The WLA for wastewater sources is not applicable.

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is not applicable.

(c) The Load Allocations for nonpoint sources are to address anthropogenic sources in the basin such that in-stream nitrate concentrations meet the TMDL target, which, based on the mean concentrations from the 2000-2011 period, will require a 90 percent reduction of nitrate.

(d) The Margin of Safety is implicit.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 10-15-09, Amended 5-7-13.