- 62-303.380 Drinking Water Use Support and Protection of Human Health.

(1) A Class I water shall be placed on the planning list for potential impairment of drinking water use support and the protection of human health if:

(a) There is a sufficient number of samples from the water segment that do not meet the applicable Class I water quality criteria for bacteriological quality based on the methodology described in Rule 62-303.320, F.A.C., with the exception that paragraph 62-303.320(4)(a), F.A.C., does not apply and samples collected on different days within any four day period will be assessed as individual samples, or

(b) A public water system demonstrates to the Department that either:

1. Treatment costs to meet applicable drinking water criteria have increased by at least 25% to treat contaminants that exceed Class I criteria or to treat blue-green algae or other nuisance algae in the source water, or

2. The system has changed to an alternative supply because of additional costs that would be required to treat their surface water source.

(c) The water segment includes a sampling location that has one or more monthly geometric mean values above the monthly geometric mean fecal coliform criterion during the planning period. To calculate a monthly geometric mean value for a sampling location, there shall be at least five samples collected within that month, with at least one sample from each full week of the month.

(2) When determining increased treatment costs described in paragraph (b), costs due solely to new, more stringent drinking water requirements, inflation, or increases in costs of materials shall not be included.

(3) A water shall be placed on the planning list for assessment of the threat to human health if:

(a) For human health-based criteria expressed as maximums, the water segment does not meet the applicable criteria based on the methodology described in Rule 62-303.320, F.A.C., or

(b) For human health-based criteria expressed as annual averages, the annual average concentration for any year of the assessment period exceeds the criteria. To be used to determine whether a water should be assessed further for human-health impacts, data must meet the requirements of subsections (2), (3), (6), and (7) in Rule 62-303.320, F.A.C.

(4) When evaluating a water segment for bacteriological quality under paragraph (1)(a), the criterion used for fecal coliforms shall be that the MPN or MF shall not exceed 400 counts per 100 ml.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 6-10-02, Amended 12-11-06, 9-4-07, 8-1-13.