14-40.003 Highway Landscape Projects.

(1) Department Authorization Required. No landscaping may be altered, removed, or installed on the Department’s right of way by any person without written authorization by the Department pursuant to this rule chapter. All requirements for restoring the Department’s right of way and highway landscape projects, where such restoration is made necessary by the construction or maintenance of utilities, are specified in the Utility Accommodation Manual, incorporated by reference under Rule 14-46.001, F.A.C. Requests to remove, cut, or trim vegetation that screens outdoor advertising signs for which outdoor advertising sign permits have been issued pursuant to Chapter 479, Florida Statutes, must be made in accordance with Part III of this rule chapter.

(2) Definitions.

(a) “Abutting Private Property Owner” means any person or entity having lawful control of land which adjoins, or is contiguous to, Department non limited access right of way.

(b) “Department” means the Florida Department of Transportation.

(c) “Highway Landscape Project” means any planned or actual landscape or landscaping on Department right of way, including construction or installation, planning, beautification, and maintenance thereof, by a local government entity, non governmental entity, or abutting private property owner.

(d) “Landscape” or “Landscaping” means any vegetation, mulches, irrigation systems, and any site amenities, such as, street furniture, decorative paving, fences, and lighting (excluding public utility street and area lighting).

(e) “Local Governmental Entity” means as defined in Section 11.45(1)(e), Florida Statutes.

(f) “Non Governmental Entity” means any person or organization, other than a governmental entity, who seeks approval of a highway landscape project.

(g) “Screen” or “Screening” means the planting or installation of any vegetation or other landscape material which will reach a height greater than the height of the bottom of the lowest sign face, as viewed from a height of 3.5 feet above the roadway surface at the edge of the travel lane closest to the outdoor advertising sign.

(3) Approval Criteria.

(a) Approval is based on review of a complete set of landscape plans. The District Landscape Manager can be consulted during preparation of landscape plans. The following plan preparation guidelines must be used:

1. Project data must be included on sheet 1 of the plans, and must include a location map with beginning and end of project mile posts, index of plans included in the set, state road number, local government and contact information, and name and address of the person and firm who prepared the plans.

2. Plans must be drawn to scale, exhibiting an accurate and legible representation of existing conditions (above and below ground), and all proposed work. Plans must show all dimensions necessary to demonstrate compliance with this rule. If there is a baseline survey or centerline of construction, station points must be used. If there is no baseline survey or centerline, dimensioning must be from a fixed point. All dimensions must be noted in English system measurements (inches, feet, yards, miles, etc.). Plans must be drawn at no less than 100 scale (1 inch = 100 feet) on 24 inch by 36 inch or smaller sheets (folded, not rolled). Computer generated plans must use 11 inch by 17 inch sheets. Plans must contain a graphic scale and north arrow with standard orientation on each plan sheet, and reference the state road number, section number, milepost, and local street names. Plans must also include curbs, edge of pavement, edge of travel lanes, guardrails, right of way fence and/or right of way lines, sidewalks, intersections, median breaks, driveways, bike lanes, transit facilities, surveying monuments, signs, vegetation management zones of permitted outdoor advertising signs, lighting, traffic signals, other traffic control devices, drainage features, limits of clear sight, set backs and clear zone limits, existing off site features and conditions which affect or are affected by the project, easements, above and below ground utilities, and all existing vegetation. Details and text must be large enough to be legible on all plan sheets.

3. Only blackline, blueline, or xerographic reproductions will be accepted for the review process.

4. All proposed landscaping must be identified on the plans. For all plants, give the following information in tabular form:

Common Name

Botanical Name, including variety or cultivar

Quantity

Size when installed (height, caliper, spread, container size, clear trunk, multi-trunk, or any other descriptive aspect of the desired plants)

Maximum maintained or typical mature height, spread, and trunk diameter of normal mature plant specimens measured 6 inches above the ground.

Specifications (written and/or graphic)

5. A comprehensive maintenance plan for all proposed landscaping must accompany the plans. This may be on a separate set of plans or documents. Special maintenance requirements for the plant establishment period must be noted. The intent of design elements must be included in a description of the project, accompanied by a written or graphic guide as to the maintenance which will be provided to the plants and other areas within the project limits. When the landscape project is to be maintained by the Department, a maintenance cost estimate based on scheduled maintenance activities must be an attachment to the plans. Maintenance details and specifications must include the following:

Mowing schedule and height of grass, along with physical depiction of the limits of the mowing that will be performed as part of the landscape project.

Fertilizing schedules, formulas, rates, and methods of application.

Weeding/edging schedule and method: chemical, mechanical, or manual.

Herbicide schedules, formulas, rates, methods of application, special instructions, and precautions.

Pruning schedule and methods. In order to have safe, healthy, and aesthetic plants, and to maintain limits of clear sight, special attention must be given to changes in the schedule due to the maturity and size as trees and shrubs grow.

Mulch materials, thickness, and replacement frequency.

Irrigation schedule, supply source, and method of application.

Special care required for any hardscape materials, lighting, signage, benches, or other site amenities.

Litter pick-up and removal schedule (prior to mowing cycle or as needed).

A work zone traffic control plan (if necessary) for installation and maintenance of the project.

An estimate of manpower and equipment required to achieve an acceptable level of maintenance.

6. As-built plans, or a summary of changes, are required for all landscape projects.

7. Plans prepared by or for Department highway landscape projects must be signed and sealed by a registered landscape architect.

(b) No planting or installation of vegetation or other landscape material for landscape projects, or issuance of permits for such planting or installation, including construction and beautification projects, is allowed on Department right of way which screens or which, when mature, will screen an outdoor advertising sign permitted under Chapter 479, F.S. This prohibition applies to outdoor advertising signs exempt from Department permitting requirements that are on the state highway system and located within incorporated municipalities. This prohibition applies to all landscape, construction, and beautification projects on Department right of way regardless of the source of funds for the project, except for landscape projects approved by the Department prior to the date of the original state sign permit for the sign. For purposes of this rule, a landscape, construction, or beautification project is approved when it is specifically identified in the Department’s five year work program, is a permitted landscape project, is part of an executed agreement between the Department and a local government, or has been approved in writing by the Department for installation at a later date by a local government.

1. Screening is prohibited within a permitted vegetation management zone, as defined in paragraph 14-40.030(1)(d), F.A.C.

2. Where there is no permitted vegetation management zone for an outdoor advertising sign, the landscape architect will notify the sign permittee that a highway landscape project is proposed within the view zone. The sign permittee will have 30 days to submit an Application for Vegetation Management that proposes a vegetation management zone (See Part III). The screening prohibition in subparagraph 1. above will apply upon approval of the Permit for Vegetation Management at Outdoor Advertising Sign. If an Application for Vegetation Management has not been submitted by the sign permittee within 30 days of notification, screening will be prohibited wherever the sign face is not screened within the view zone, beginning at Terminus A and extending along the outside travel lane edge in advance of the sign until the sign face is not screened for a total of 500 feet. Contact information for any permitted sign may be obtained by contacting the State Outdoor Advertising Administrator, Florida Department of Transportation, 605 Suwannee Street, MS 22, Tallahassee, Florida 32399-0450.

3. The limits of the screening prohibition may be adjusted pursuant to a written agreement between the sign permittee and a local governmental entity.

(c) A local governmental entity may request approval to alter, remove, or install landscaping on the Department’s right of way through submission of a landscape plan. After review by the Department, and the making of any necessary revisions by the local governmental entity, the Department will prepare a written agreement requiring the local governmental entity to properly construct and maintain the landscape project. The landscape plan will become Exhibit A to the agreement. If separate, the maintenance plan as described in subparagraph (2)(a)7. will become Exhibit B of the agreement. When the agreement is executed, and a Notice to Proceed is issued by the Department, the local governmental entity may proceed with the project.

(d) Non-governmental entities may seek approval to alter, remove, or install landscaping on the Department’s right of way through submission of a landscape plan, and a resolution from the appropriate local government that commits the local government to execution of an agreement to properly construct and maintain the landscape project as described in paragraph (c) above.

(e) An abutting private property owner is not required to comply with paragraph (3)(d) of this rule and may apply for a permit to alter or install landscape materials on the Department’s non limited access right of way directly abutting the owner’s property between the right of way line and the nearest edge of pavement through submission of a Permit for Landscaping on State Road Right of Way, Form 650-050-09, Rev. 01/06, which is incorporated herein by reference and is available at any Department Office or on the Department website at: www.dot.state.fl.us/emo. Abutting private property owners must submit for approval a landscape plan, maintenance plan, and work zone traffic control plan.

(f) Non-governmental entities or abutting private property owners seeking approval to install landscaping on the Turnpike right of way shall submit a completed Form 650-050-09 to the District Landscape Manager, Florida’s Turnpike Operations Center, P. O. Box 9828, Fort Lauderdale, Florida 33310, telephone (954) 975-4855. The application must be accompanied by a landscape plan, maintenance plan, and work zone traffic control plan.

(g) Approval will only be granted when it is determined that all plans meet the requirements of this rule. No permit will be issued to an abutting private property owner to provide visibility of such property through the cutting, trimming, or removal of trees, shrubs, or herbaceous plants.

(4) Government Approvals. If the proposed highway landscape project is to be located on an Interstate Highway facility, Federal Highway Administration (FHWA) review is required. Approval pursuant to this Part does not relieve the local governmental entity, non governmental entity, or abutting property owner of local or other jurisdictional requirements.

(5) Installation and Maintenance.

(a) All landscape installation or maintenance activities performed by a local governmental entity, non governmental entity, or abutting property owner on the Department’s right of way must be performed in conformity with the Manual on Uniform Traffic Control Devices (incorporated by reference under Rule 14-15.010, F.A.C.), Standard Specifications for Road and Bridge Construction, (incorporated by reference under Rule 14-85.004, F.A.C.), and the Roadway and Traffic Design Standards (incorporated by reference under Rule 14-85.004, F.A.C.).

(b) If an agreement exists between the Department and a local governmental entity for the maintenance of an existing median and grassed areas for the section of roadway for which a landscape project is proposed, and if the Department determines that such agreement obligates the local governmental entity to maintain the proposed project in accordance with the approved maintenance plan, the agreement will be applied to the maintenance of the landscape project. If the Department has previously agreed to provide funds for such maintenance, no increased compensation will be provided by the Department for maintenance of the landscape project.

(6) Donation of Landscape Projects. The Department will accept donations of plants, materials, installation, and maintenance for landscape projects on the State Highway System that meet the requirements of this rule. The donated landscape projects must substantially improve the appearance or manageability of the median or roadside. The agreement must stipulate that the sign and the landscape project may be removed by the Department for failure to meet the requirements of this rule chapter or the agreement. An agreement must be on file with the Department for the area in question before placement of the sign. Signs acknowledging donated landscape projects by an individual or entity may be erected on the right of way, when the donation includes installation and maintenance pursuant to an executed agreement. Such signs will remain in place for a term of five years, unless otherwise specified in the agreement. The sign must not contain commercial logos or trademarks. Signs will be placed at each end of the landscape project.

(a) Interstate Highways: The sign acknowledging donation of landscape projects on the Interstate Highway System will be provided by the Department. This sign will be similar to the Adopt-a-Highway Program sign used by the Department, except that the word “landscaping” will be substituted for the words “litter control” and the colors will be green lettering on white background. The signs shall be installed and maintained by the Department.

(b) Arterial Highways: The approved sign design for arterial highways is depicted in Figure 1. The sign panel will be 18 inches tall and 24 inches wide with white background and green lettering, using lettering shown in Figure 1 and fabricated with non-reflective materials. The sign must be mounted on 4 inch by 4 inch pressure treated posts, or break away posts that meet or exceed the requirements of the Roadway and Traffic Design Standards (incorporated by reference under Rule 14-85.004, F.A.C.). The top of signs will be a maximum of no more than two feet above grade. Installation of signs is contingent upon an agreement with the appropriate local governmental entity. The approved sign panel(s) must be provided and replaced by the local governmental entity. Signs will be placed according to the approved landscape plan.

Specific Authority 334.044(2), 337.2505 FS. Law Implemented 334.044(25), 335.167, 337.2505, 337.405, 339.24, 339.2405, 479.106 FS. History–New 9-22-92, Amended 1-19-99, 4-2-02, 5-22-05, 2-8-06.