14-94.003 Statewide Minimum Level of Service Standards.

(1) The Statewide Minimum LOS Standards are as follows:

| STATEWIDE MINIMUM LEVEL OF SERVICE STANDARDS FOR THE STATE HIGHWAY SYSTEM, ROADWAYS ON THE STRATEGIC INTERMODAL SYSTEM (SIS), ROADWAYS ON THE FLORIDA INTRASTATE HIGHWAY SYSTEM (FIHS) AND ROADWAY FACILITIES FUNDED IN ACCORDANCE WITH SECTION 339.2819, FLORIDA STATUTES, THE TRANSPORTATION REGIONAL INCENTIVE PROGRAM (TRIP) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | SIS AND FIHS FACILITIES |  | TRIP FUNDED FACILITIES AND OTHER STATE ROADS3 |  |
|  | Limited Access Highway4 (Freeway) | Controlled Access Highway4 | Other Multilane4 | Two-Lane4 |
| Rural Areas | B | B1 | B | C |
| Transitioning Urbanized Areas, Urban Areas, or Communities | C | C | C | C |
| Urbanized Areas Under 500,000 | C(D) | C | D | D |
| Urbanized Areas Over 500,000 | D(E) | D | D | D |
| Roadways Parallel to Exclusive Transit Facilities | E | E | E | E |
| Inside TCMAs | D(E)2 | E2 | --2 | --2 |
| Inside TCEAs2 and MMTDs2 | --2 | --2 | --2 | --2 |
| Level of service standards inside of parentheses apply to general use lanes only when exclusive through lanes exist. 1. For rural two-lane facilities, the standard is C. 2. Means the Department must be consulted as provided by Section 163.3180(5), (7), or (15), Florida Statutes, regarding level of service standards set on SIS or TRIP facilities impacted by TCMAs, MMTDs, or TCEAs respectively. 3. Means the level of service standards for non TRIP facilities may be set by local governments in accordance with Rule 9J-5.0055, F.A.C. 4. It is recognized that certain roadways (i.e., constrained roadways) will not be expanded by the addition of through lanes for physical, environmental, or policy reasons. In such instances, a variance to the level of service may be sought pursuant to Section 120.542, Florida Statutes. NOTE: Level of service letter designations are defined in the Department’s 2002 Quality/Level of Service Handbook. |  |  |  |  |

(2) Specific assumptions and restrictions that apply to these minimum LOS standards are:

(a) The minimum LOS standards represent the lowest acceptable operating conditions in the peak hour.

(b) Definitions and measurement criteria used for the minimum LOS standards can be found in the Transportation Research Board’s Highway Capacity Manual Special Report 2000.

(c) When calculating or evaluating level of service pursuant to this rule, all calculations and evaluations shall be based on the methodology contained in Transportation Research Board’s Highway Capacity Manual Special Report 2000, the Department’s 2002 Quality/Level of Service Handbook, or a methodology determined by the Department to be of comparable reliability. Any methodology superseded by the Highway Capacity Manual 2000, such as a methodology based on the 1997 Highway Capacity Manual or Circular 212, shall not be used.

(3) Minimum LOS Standards for SIS Connectors and TRIP Funded Facilities are:

(a) Minimum LOS Standards for SIS Highways.

1. Limited access SIS highways shall adhere to the limited access FIHS LOS standards.

2. Controlled access SIS highways shall adhere to the controlled access FIHS LOS standards.

3. These standards shall apply regardless whether the facility is FIHS, SHS, or under other jurisdiction.

(b) Minimum LOS Standards for SIS Connectors. The minimum LOS standard for SIS connectors shall be LOS D.

(c) Minimum LOS Standards for Regionally Significant Roadways Funded by the TRIP.

1. Regionally significant roadways utilizing TRIP funding shall adhere to the Other State Roads Standards in Chapter 14-94, F.A.C.

2. These LOS standards apply to the TRIP funded portions of the roadway facilities extending to their logical termini for LOS analysis.

Specific Authority 163.3180(10), 334.044(2) FS. Law Implemented 163.3180(10), 163.3184(4), 334.03, 334.044(10)(a), (12), (19), 339.155(2), 339.2819, 339.61-.64 FS. History–New 4-14-92, Amended 5-8-06.