- 62-330.437 General Permit for Installation of Fences.

A general permit is granted to install a fence in wetlands or other surface waters under all of the following conditions:

(1) The fence shall not be located in Outstanding Florida Waters, Aquatic Preserves, Outstanding National Resource Waters, Class II waters, or waters approved, conditionally approved, restricted, or conditionally restricted by the Department of Agriculture and Consumer Services for shellfish harvesting;

(2) The fence shall extend a distance of no more than 25 feet waterward into the open water of any navigable river, stream, canal, or tributary thereof, or no more than 20 percent of the width of the open water, whichever is less. “Open water” means any portion of the waterway that is devoid of emergent vegetation;

(3) In a lake, the fence shall extend no more than 15 feet waterward of the landward extent of any lake, including contiguous wetlands;

(4) The fence shall be constructed of wire attached to posts rising at least two feet above the mean high water or the ordinary high water elevation of the waterway; and,

(5) In navigable waters and all lakes, the top of the fence posts shall be marked with reflectors visible from all directions so as to prevent the fence from being a navigational hazard.

Rulemaking Authority 373.026(7), 373.043, 373.118(1), 373.406(5), 373.4131, 373.414(9), 373.418, 403.805(1) FS. Law Implemented 373.118(1), 373.406(5), 373.413, 373.4131, 373.414(9), 373.416, 373.418, 403.814(1) FS. History–New 10-3-95, Formerly 62-341.437, Amended 10-1-13.