- 62-330.496 General Permit for Dry Borrow Pits of Less than Five Acres.

A general permit is granted for the construction, operation, maintenance, reclamation, or abandonment of a minor borrow pit for excavation of sand and other soil materials provided that all of the following conditions are met:

(1) The area of excavation for the borrow pit shall be less than 5 acres, when measured at the natural land surface grade of the pit.

(2) The final excavated depth of the borrow pit shall not be any deeper than the seasonal high water level.

(3) The borrow pit shall be constructed entirely in uplands for the purpose of using the borrow materials as appropriately permitted, authorized or as exempted.

(4) Borrow pits authorized under this permit must meet the following conditions:

(a) Does not authorize the construction of or maintain any embankment above the natural land surface grade as a part of this work to construct the pit and remove the soil materials.

(b) No above-grade roads shall be constructed as access to the pit area.

(c) Does not impact wetlands.

(d) Does not penetrate any water table condition, surficial aquifer, aquatard or karst materials.

(e) Adequate internal side slopes shall be mainained to prevent erosion or sloughing.

(f) The pit area shall be protected at all times by adequate fencing and gating structures to limit access and provide for safety.

(g) Applicants are advised that a borrow pit that is operated as a mine, as defined in section 2 of Volume I, will be subject to the mine reclamation requirements under Part III of Chapter 378, F.S.

Rulemaking Authority 373.026(7), 373.043, 373.118(1), 373.406(5), 373.4131, 373.414(9), 373.418, 403.805(1) FS. Law Implemented  373.118(1), 373.406(5), 373.413, 373.4131, 373.414(9), 373.416, 373.418, 373.426 FS. History–New 10-1-13.