- 62-330.635 General Permit for Soil Remediation.

(1) A general permit is granted to conduct soil removal activities, including installation of wells and work in wetlands and other surface waters necessary to perform soil remediation as part of a Remedial Action Plan approved by the Department. This includes construction, alteration, operation, and removal of a temporary access road for access to conduct this work, subject to the following conditions.

(a) Removal of contaminated soil is limited to no more than a total of 5 acres of wetlands.

(b) Temporary fill and materials for equipment access shall be removed immediately following completion of the remediation work.

(c) Any wetland area affected by the work shall be restored to pre-construction wetland elevations within 30 days following completion of the work, using sediments consisting of the same soil textural material as the original pre-construction soil material that is also free of vegetated debris, rebar and any other solid waste materials.

(d) Any muck removed from wetlands for construction of temporary fill roads shall be stockpiled in uplands and used in restoring the affected area to wetland conditions and preconstruction wetland elevations, unless this material is required to be removed as part of the remediation plan.

(e) Within 7 days of completion of construction, all wetland areas shall be restored to pre-construction wetland elevations and re-vegetated with native wetland species endemic to adjoining, undisturbed wetlands or the underlying wetland community type historically occurring at the site. The restored wetland areas shall be maintained and planted as necessary to ensure that at least 33 percent cover of planted or naturally reestablished native wetland plant species is appropriate for the wetland community type within 18 months of completion of authorized work. Exotic invasive species, including but not limited to: Schinus terebinthifolius, Melaleuca quinquenervia, Casuarina spp., Lygodium spp., and nuisance species Typha spp., and Ludwigia peruviana shall be controlled at densities not exceeding the densities of these species in undisturbed portions of the wetland.

(f) In addition to compliance with the notice provisions of Rule 62-330.402, F.A.C., within 60 days following completion of construction, the permittee will notify the Agency by letter of the date construction activities were completed.

(g) All contaminated soils removed from the site shall be disposed of in an appropriate disposal facility, in accordance with the Remedial Action Plan approved by the Department.

Rulemaking Authority 373.026(7), 373.043, 373.118(1), 373.406(5), 373.4131, 373.414(9), 373.418, 403.805(1) FS. Law Implemented 373.118(1), 373.406(5), 373.413, 373.4131, 373.414(9), 373.418, 376.3071, 403.814(1) FS. History–New 10-1-13.