61-30.803 Standards of Practice, Electrical Systems.

(1) Electrical systems and components include the following:

(a) Service entrance conductors, drip loop, cables, and raceways;

(b) Main service equipment and main disconnects;

(c) Service grounding;

(d) Interior components of main service panels and sub panels;

(e) Conductors;

(f) Over current protection devices;

(g) Readily accessible installed lighting fixtures, switches, and receptacles;

(h) Ground fault circuit interrupters;

(i) Amperage and voltage rating of electrical service;

(j) Main disconnect(s);

(k) Methods or types of wiring;

(l) Smoke detectors;

(m) Carbon monoxide detectors;

(n) Arc fault circuit interrupters.

(2) The inspector shall inspect all of the visible and readily accessible electrical systems and components.

(3) The inspector is not required to inspect:

(a) Remote control devices;

(b) Security alarm systems and components;

(c) Low voltage wiring, systems and components, ancillary wiring and systems and components not a part of the primary electrical power distribution system;

(d) Generators, photovoltaic solar collectors or battery or electrical storage devices and associated equipment.

(4) The inspector is not required to:

(a) Measure amperage, voltage or impedance;

(b) Perform a load calculation;

(c) Insert any tool, probe, or device into any electrical component;

(d) Determine the accuracy of circuit labeling.

Rulemaking Authority 468.8325 FS. Law Implemented 468.8323, 468.832(1)(j) FS. History‒New 10-22-13.