15B-4.001 Specifications and Regulations for All Nonpublic School Buses in Operation on, or Purchased Before, July 1, 1969, Twenty-four (24) Passenger Capacity and Up.

(1) All motor vehicles, other than private passenger automobiles and school buses with a seating capacity of less than twenty-four (24) pupils, which are used primarily for the transportation of pupils to school, but which are not operated by or under the purview of the state, a political subdivision thereof, or under franchise issued by a municipality or the public service commission, shall comply with the following regulations effective July 1, 1969.

(a) Shall be equipped with all equipment regularly required of a motor vehicle of that size and weight.

(b) Shall pass Florida’s annual inspection by the Florida Highway Patrol.

(c) All school bus drivers shall pass an annual physical examination, and have posted in bus certificate to drive same.

(d) Shall be painted National School Bus chrome yellow with black trim. No other colors acceptable.

(e) Shall have semaphore extender stop arm painted red with the “STOP” in six (6) inch white letters.

(f) Each school bus shall be equipped with signal lamps mounted laterally as high as practicable, which shall be capable of displaying on the traffic side to the front two alternately flashing lights, one red and one amber, located at the same level, and these lights shall have sufficient intensity to be visible at five hundred (500) feet in normal sunlight. Each such light shall be a minimum of five and one fourth (5 1/4) inches in diameter with black background of one and one fourth (1 1/4) inches to three (3) inches wide and with a minimum hood of two and one half (2 1/2) inches. Such lights shall meet or exceed the standards of the Society of Automotive Engineers of New York.

(g) Shall have non-leaking exhaust system.

(h) Shall be equipped with first aid kit.

(i) Shall be equipped with fire extinguisher.

(j) Shall have glass on all windows (not broken).

(k) Lettering: “School Bus” shall be painted in eight (8) inch letters on both front and rear and correspond with series B, standard alphabet. Emergency exists shall be indicated by two (2) inch letters on inside and outside. Name of school shall be in six (6) inch letters on belt line.

(l) Shall have outside rear view mirror on left and right sides, and each mirror shall have not less than fifty (50) square inches overall viewing area.

(m) Shall have inside rear view mirror capable of giving driver a clear view of the bus interior.

(n) Seats shall be standard type of metal fully padded, or fiberglass securely anchored and in good repair with no rough or sharp edges.

(o) Shall have liability insurance to protect pupils being transported.

(p) Shall transport no more passengers than equipped to seat; not less than thirteen (13) inches seat area per passenger.

(q) Shall have adequate switches to operate semaphore stop arm and warning lights.

(r) All drivers will be required to have a chauffeurs license and abide by all the laws, rules and regulations pertaining to operating a school bus.

Specific Authority 316.288 FS. Law Implemented 316.288 FS. History–New 11-20-75, Transferred to 15C-6.01, 4-17-81, Transferred back to 15B-4.01, 11-6-81, Formerly 15B-4.01.