- 20-49.0051 Large Producers ‒ Inspections.

This section regarding the inspections shall be applicable to Large Producers of fresh squeezed citrus juices.

(1) All inspections and audits shall be performed by or under the authority of FDACS.

(2) All Large production establishments shall be inspected according to Sections 2.2.1 through 2.2.58, July 1996, and 3.2.7a through 3.2.7o, June 1996, of the Citrus Handbook of the Processed Citrus Branch, Fruit and Vegetable Division, United States Department of Agriculture, incorporated herein by reference.

(3) All Large Producers shall be subject to full-time inspection by FDACS or its agent.

(4) The following specific Good Manufacturing Practices, in addition to those contained in Chapter 5K-4, F.A.C., and 21 CFR 110, revised April 1, 2000, incorporated herein by reference, shall be applicable:

(a) All soil, debris, stems, leaves, etc. must be removed from the fruit.

(b) All whole and intact fruit shall be washed, sanitized and/or surface heat-treated immediately prior to extraction. If the whole and intact fruit is washed, sanitized and/or surface heat-treated, but is not immediately extracted, the fruit may be maintained in a sanitary storage room or container for forty-eight (48) hours prior to extraction, so long as the temperature of the room or container is maintained at 40° Farenheit or less.

(c) All fruit that has been in cold storage at a temperature of 40° Farenheit or less in excess of forty-eight (48) hours shall be resanitized and regraded. All soft or unwholesome fruit shall be discarded.

(d) All belts and rollers must be maintained free of soil, wax, dirt and extraneous material.

(e) The entire wash area shall, at all times, be maintained free of excess debris, pests and standing water.

(f) Grading must eliminate damaged, defective, soft or decayed fruit.

(g) Drops, fruit from the ground, may not be used in the production of fresh citrus juice.

(h) The processing and filling area shall be completely enclosed and meet the structural requirements for food processing areas as defined in Chapter 5K-4, F.A.C., and 21 CFR 110, revised April 1, 2000, incorporated herein by reference.

(i) All fruit contact surfaces must be cleaned and sanitized after production and prior to startup. Appropriate cleaning and sanitizing agents must be used as prescribed by the equipment manufacturer for the specific use.

(j) If product residues or buildup of organic matter remain on equipment, additional chemical treatment shall be used to remove such residues or buildup.

(k) All lubricants must be food grade only, as found in 21 CFR 178.3570, revised April 1, 2000, incorporated herein by reference.

(l) Back-siphonage protection devices must be provided on any water outlet where a hose can be connected.

(m) A contingency plan for in-line and surge tank juice during breakdowns must be in place. Cleaning and sanitizing procedures must be performed prior to restarting operation after extended breakdowns.

(n) All juice containers must, at all times, be sanitarily handled and protected from contamination. Containers must be covered when removed from protection if not used immediately.

(o) Certificates for a potable water supply shall be obtained from the Florida Department of Health (hereafter “DOH”) approved laboratory on an annual basis prior to the start of the season.

(p) As to personnel and sanitary establishments, Large Producers shall meet all applicable state and federal regulations with respect to cleanliness and disease and pest control.

(q) All Large Producers shall establish and maintain records that:

1. Identify the source of the fruit used in the juice production by date and variety; and

2. Identify microbiological test results to date of production, fruit source and juice type; and

3. Implement a corrective action plan for unsafe products.

Rulemaking Authority 601.10(7), 601.11 FS. Law Implemented 601.10(7), 601.11, 601.29, 601.33, 601.38 FS. History–New 9-12-01.