20-64.002 Canned Orange Juice.

Florida State Grades shall be identical with United States Standards for Grades of Orange Juice, Sections 52.1551 through 52.1559, effective January 10, 1983, with the following exceptions:

(1) Minimum acid requirement for “B” Grade, Style II (Sweetened) shall be the same as for “B” Grade, Style I (Unsweetened).

(2)(a) During the period August 1 through November 30 each year, the minimum Brix-acid ratio shall be:

1. 11.0 to 1 regardless of Brix for Unsweetened Grade A.

2. 10.0 to 1 for Unsweetened Grade B.

3. 15.0 to 1 regardless of Brix for Sweetened Grades A and B.

(b) During the period December 1 through the following July 31, the minimum Brix-acid ratio for unsweetened Grade A shall be 10.0 to 1.

(3) Product shall meet the requirements of Section 601.9905, Florida Statutes.

(4) All facilities in which the product is stored shall, except for periods not to exceed seven days in the aggregate, be maintained at temperatures not to exceed 50º F; provided, the product need not be refrigerated during processing nor during shipment. The provisions of this subsection shall not apply to product packed in metal or fiber containers.

(5) Temperature of the product as it emerges from the can coolers and prior to casing, shall not exceed 100º F. However, a tolerance of one in six or two in thirteen samples is permitted. A lot of product failing to meet the temperature requirement may be cooled for 24 hours and re-inspected for compliance, if during the initial inspection no more than three in six or six in thirteen samples exceed 100º F. and no sample exceeded 110º F.

Specific Authority 601.10(7), 601.11 FS. Law Implemented 601.10(7), 601.11, 601.48, 601.9905, 601.9914(1), (2) FS. History–Formerly 105-1.19(1)(a), Revised 1-1-75, Amended 12-12-82, 11-6-83, Formerly 20-64.02, Amended 10-28-97.