Notice of Proposed Rule

DEPARTMENT OF ENVIRONMENTAL PROTECTION

RULE NOS.:	RULE TITLES:

62-296.320	General Pollutant Emission Limiting Standards

62-296.401	Incinerators

62-296.402	Sulfuric Acid Plants

62-296.403	Phosphate Processing

62-296.404	Kraft (Sulfate) Pulp Mills and Tall Oil Plants

62-296.405	Fossil Fuel Steam Generators with More Than 250 Million Btu Per Hour Heat Input

62-296.406	Fossil Fuel Steam Generators with Less Than 250 Million Btu Per Hour Heat Input, New and Existing Emissions Units

62-296.408	Nitric Acid Plants

62-296.409	Sulfur Recovery Plants

62-296.410	Carbonaceous Fuel Burning Equipment

62-296.412	Dry Cleaning Facilities

62-296.414	Concrete Batching Plants

62-296.415	Soil Thermal Treatment Facilities

62-296.416	Waste-to-Energy Facilities

62-296.501	Can Coating

62-296.502	Coil Coating

62-296.503	Paper Coating

62-296.504	Fabric and Vinyl Coating

62-296.505	Metal Furniture Coating

62-296.506	Surface Coating of Large Appliances

62-296.507	Magnet Wire Coating

62-296.508	Petroleum Liquid Storage

62-296.510	Bulk Gasoline Terminals

62-296.511	Solvent Metal Cleaning

62-296.512	Cutback Asphalt

62-296.513	Surface Coating of Miscellaneous Metal Parts and Products

62-296.514	Surface Coating of Flat Wood Paneling

62-296.515	Graphic Arts Systems

62-296.516	Petroleum Liquid Storage Tanks with External Floating Roofs

62-296.570	Reasonably Available Control Technology (RACT) – Requirements for Major VOC- and NOx-Emitting Facilities

62-296.600	Reasonably Available Control Technology (RACT) – Lead

62-296.602	Primary Lead-Acid Battery Manufacturing Operations

62-296.603	Secondary Lead Smelting Operations

62-296.604	Electric Arc Furnace Equipped Secondary Steel Manufacturing Operations

62-296.701	Portland Cement Plants

62-296.702	Fossil Fuel Steam Generators

62-296.703	Carbonaceous Fuel Burners

62-296.704	Asphalt Concrete Plants

62-296.705	Phosphate Processing Operations

62-296.706	Glass Manufacturing Process

62-296.707	Electric Arc Furnaces

62-296.708	Sweat or Pot Furnaces

62-296.709	Lime Kilns

62-296.710	Smelt Dissolving Tanks

62-296.711	Materials Handling, Sizing, Screening, Crushing and Grinding Operations

62-296.712	Miscellaneous Manufacturing Process Operations

PURPOSE AND EFFECT: The purpose of the proposed rule (OGC No. 12-1180) is to amend the above-listed rules in Chapter 62-296, F.A.C., related to air emission standards for stationary sources, to correct erroneous cross references, remove or amend obsolete provisions, and clarify rule language.

SUMMARY: Multiple rules in Chapter 62-296, F.A.C., will be amended to correct erroneous cross references; remove or amend obsolete provisions; clarify rule language, particularly where needed as a result of the intended repeal of DEP Method 9; and to revise cross-references because of the intended repeal of Rule 62-297.401 and subsections of Rule 62-297.440.

OTHER RULES INCORPORATING THESE RULES: 62-160.110; 62-204.800; 62-210.200; 62-210.300; 62-210.310; 62-212.500; 62-213.400; 62-213.412; 62-213.440; 62-256.300; 62-256.700; 62-297.310; 62-297.450; 62-640.400; 62-701.320;62-701.530; 62-701.710; 62-701.730; 62-701.803; 62-709.300; 62-709.320; and 62-709.460, F.A.C.

EFFECT ON OTHER RULES: There will be no effect on other rules.

SUMMARY OF STATEMENT OF ESTIMATED REGULATORY COSTS AND LEGISLATIVE RATIFICATION:  The Agency has determined that this will not have an adverse impact on small business or likely increase directly or indirectly regulatory costs in excess of $200,000 in the aggregate within one year after the implementation of the rule. A SERC has not been prepared by the Agency.

The Agency has determined that the proposed rule is not expected to require legislative ratification based on the statement of estimated regulatory costs or if no SERC is required, the information expressly relied upon and described herein: The Department determined that the amendments to these rules are necessary to clarify and to remove unnecessary, redundant, and obsolete references related to air emission from stationary standards and will not increase regulatory costs for any entity.

Any person who wishes to provide information regarding a statement of estimated regulatory costs, or provide a proposal for a lower cost regulatory alternative must do so in writing within 21 days of this notice.

RULEMAKING AUTHORITY: 403.061, 403.716, 403.8055 FS.

LAW IMPLEMENTED: 403.021, 403.031, 403.061, 403.087, 403.716, 497.606 FS.

IF REQUESTED WITHIN 21 DAYS OF THE DATE OF THIS NOTICE, A HEARING WILL BE SCHEDULED AND ANNOUNCED IN THE FAR.

THE PERSON TO BE CONTACTED REGARDING THE PROPOSED RULE IS: Cindy Phillips, Florida Department of Environmental Protection, Division of Air Resource Management, 2600 Blair Stone Road, MS 5500, Tallahassee, Florida, 32399-2400, Telephone: (850)717-9098, E-mail: Cindy.Phillips@dep.state.fl.us

THE FULL TEXT OF THE PROPOSED RULE IS:

62-296.320 General Pollutant Emission Limiting Standards.

(1) through (3) No change.

(4) General Particulate Emission Limiting Standards. The following emission limiting standards shall apply to emissions units of particulate matter not subject to a particulate emission limit or opacity limit set forth in or established elsewhere in this chapter.

(a) Process Weight Table.

1. through 2. No change.

3. Particulate Matter Emissions Test Method and procedures. All particulate matter emissions tests performed pursuant to the requirements of this rule shall comply with the following provisions.

a. Emissions units incorporating a scrubber for control of particulate matter shall use the following test methods.

(i) Citrus Plants. The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. An acetone wash shall be used. The minimum sample volume shall be 32 dry standard cubic feet.

(ii) All Others. The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. An acetone wash shall be used.

b. Emissions units incorporating dry controls for control of particulate matter shall use the following test methods.

(i) Phosphate Processing. The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. An acetone wash shall be used. The minimum sample volume shall be 30 dry standard cubic feet.

(ii) All Others. The test method for particulate emissions shall be EPA Method 17, with an acetone wash and an average stack temperature below 275 degrees Fahrenheit, or EPA Method 5 with an acetone wash. EPA Method 17 is described at 40 C.F.R. Part 60, Appendix A-6; and EPA Method 5 is described at 40 C.F.R. Part 60, Appendix A-3.These test methods are adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

c. No change.

(b) General Visible Emissions Standard.

1. No person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity, the density of which is equal to or greater than that designated as Number 1 on the Ringelmann Chart (20 percent opacity).

2. through 3. No change.

4. All visible emissions tests performed pursuant to the requirements of this rule shall comply with the following provisions.

a. The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

b. No change.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.031, 403.061, 403.087 FS. History–Formerly 17-2.620, 17-296.320, Amended 1-1-96, Amended 3-13-96, 10-6-08,___________.

62-296.401 Incinerators.

(1) Small Incinerators. Any incinerator, other than a biological waste incinerator, human or animal crematory, or air curtain incinerator, with a charging rate of less than fifty (50) tons per day shall comply with the following requirements.

(a) Emission Limiting Standards. Visible emissions shall not exceed five percent (5%) opacity except that visible emissions not exceeding fifteen (15%) percent opacity are allowed for one six-minute period up to six (6) minutes in any one (1) -hour period.

(b) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this subsection shall comply with the following requirements.

1. The reference test method for visible emissions shall be EPA Method 9, as described at 40 CFR, Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

2.  No change.

(c) No change.

(2) Existing incinerators, other than those which are operated or utilized for the disposal or treatment of biological waste, with a charging rate equal to or greater than 50 tons per day.

(a) through (b) No change.

(c) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

1. The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet. EPA Method 3 or 3A, using Orsat analysis is required for percent excess air correction. EPA Methods 3 and 3A, are described at 40 C.F.R. Part 60, Appendix A-2, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

2. No change.

(d) No change.

(3) New incinerators, other than those which are operated or utilized for the disposal or treatment of biological waste, with a charging rate equal to or greater than 50 tons per day.

(a) through (b) No change.

(c) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

1. The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet. EPA Method 3 or 3A, using Orsat analysis is required for percent excess air correction. EPA Methods 3 and 3A, are described at 40 C.F.R. Part 60, Appendix A-2, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

2. No change.

(d) No change.

(4) Biological Waste Incinerators.

(a) No change.

(b) Emission Limiting Standards.

1. For any biological waste incinerator unit with a capacity less than fifty (50) tons per day, visible emissions shall not exceed five percent (5%) opacity, six (6) minute average, except that visible emissions not exceeding fifteen percent (15%) percent opacity shall be allowed for one six-minute period up to six (6) minutes in any one (1)-hour period.

2. through 5. No change.

(c) No change.

(d) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this subsection shall comply with the following requirements. All EPA reference test methods are described in 40 CFR Part 60, Appendicesx A-2 through A-8, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

1. through 6. No change.

(e) No change.

(f) Continuous Emissions Monitoring Requirements. Each owner or operator of a biological waste incinerator unit shall install, operate, and maintain, in accordance with the manufacturer’s instructions, continuous emission monitoring equipment at the exit of the secondary (or last) combustion chamber.

1. The monitors shall record the following operating parameters:

a. Secondary (or last) combustion chamber exit Ttemperature.

b. No change.

2. No change.

(5) Human Crematories.

(a) No change.

(b) Emission Limiting Standards.

1. Visible emissions shall not exceed 5% opacity, six-minute average, except that visible emissions not exceeding 15% opacity shall be allowed for one six-minute period up to six minutes in any one-hour period.

2. through 3. No change.

(c) through (e) No change.

(f) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this subsection shall comply with the following requirements. All EPA reference test methods are described in 40 CFR Part 60, Appendicesx A-2 through A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

1. through 5. No change.

(g) No change.

(h) Frequency of Testing.

1. through 2. No  change.

3. The owner or operator of any human crematory unit shall not be required to have performance tests conducted for carbon monoxide and particulate matter, except as provided at paragraph 62-297.310(7)(c)(b), F.A.C.

(i) No change.

(6) Animal Crematories.

(a) No change.

(b) Emission Limiting Standards.

1. Visible emissions shall not exceed five percent (5%) opacity, six (6) minute average, except that visible emissions not exceeding fifteen percent (15%) percent opacity shall be allowed for one six-minute period up to six (6) minutes in any one (1) -hour period.

2. through 3. No change.

(c) through (e) No change.

(f) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this subsection shall comply with the following requirements. All EPA reference test methods are described in 40 CFR Part 60, Appendicesx A-2 through A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

1. through 5. No change.

(g) No change.

(h) Frequency of Testing.

1. through 2. No change.

3. The owner or operator of any animal crematory unit with a capacity of less than 500 pounds per hour shall not be required to have performance tests conducted for carbon monoxide and particulate matter, except as provided at paragraph 62-297.310(7)(c)(b), F.A.C.

4. No change.

(i) No change.

(7) Air Curtain Incinerators.

(a) No change.

(b) Operating Requirements.

1. Outside of startup periods, visible emissions shall not exceed ten percent (10%) percent opacity, six (6) minute average. During startup periods, which shall not exceed the first thirty (30) minutes of operation, an opacity of up to thirty-five percent (35%), averaged over a six (6) minute period, shall be allowed. The general excess emissions rule, Rule 62-210.700, F.A.C., shall not apply.

2. through 10. No change.

(c) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this subsection shall comply with the following requirements.

1. The reference test method for visible emissions shall be EPA Method 9, as described at 40 CFR Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

2. through 3. No change.

(d) No change.

Rulemaking Authority 403.061, 403.716 FS. Law Implemented 403.031, 403.061, 403.087, 403.716, 497.606, 470.025 FS. History–Formerly 17-2.600(1), Amended 12-2-92, Formerly 17-296.401, Amended 11-23-94, 1-1-96, 3-13-96, 11-13-97, 1-10-07,__________.

62-296.402 Sulfuric Acid Plants.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPADEP Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for acid mist/sulfur dioxide shall be EPA Method 8, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 40 dry standard cubic feet.

(c) No change.

(4) through (5) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(2), 17-296.402, Amended 11-23-94, 1-1-96, 3-13-96,___________.

62-296.403 Phosphate Processing Fluorides Limits.

Fluorides (water soluble or gaseous atomic weight 19) expressed as pounds of fluoride per ton of phosphate materials input to the system expressed as tons of P2O5.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for fluoride emissions shall be EPA Method 13A or EPA Method 13B, as described at 40 C.F.R. Part 60, Appendix A-5, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(b) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(3), 17-296.403, Amended 11-23-94, 1-1-96, 3-13-96,___________.

62-296.404 Tall Oil Plants and Kraft (Sulfate) Pulp Mills and Tall Oil Plants.

The provisions of this rule that apply to tall oil plants within Kraft (Sulfate) Pulp Mills also apply to tall oil plants that are located in a separate facility. In the case of separate tall oil plants, phrases such as “the owner or operator of a kraft pulp mill” shall be construed to read “the owner or operator of a tall oil plant.”

(1) Visible Emissions.

(a) Kraft Recovery Furnaces Equipped with Dry Collectors – 45 percent opacity, six minute average, except:

1. Visible emissions of up to 60 percent opacity shall be allowed for one six-minute period during any one-hour period; or

2. If the emissions unit is equipped with a certified continuous emission monitoring device for measuring opacity, then the monitoring results shall be reported to the Department quarterly in the form of an excess emissions report, and visible emissions in excess of 45 percent opacity shall be allowed for up to six percent of the total number of possible contiguous periods of excess emissions in a quarter (excluding periods of startup, shutdown, or malfunction and periods when the emissions unit is not operating). The continuous emission monitoring device shall be certified, calibrated, and operated according to the procedures for opacity monitors contained in 40 C.F.R. Part 60, Subpart A, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

(b) through (c) No change.

(2) Particulate Matter.

(a) No change.

(b) Visible emission limits for kraft pulp mill emissions units equipped with wet scrubbers shall be effective only if the visible emission measurement can be made without being substantially affected by plume mixing or moisture condensation. If the Department determines that visible emissions exceed 20 percent opacity, a special compliance test may be required in accordance with subsection 62-297.340(2), F.A.C.

(3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Kraft Recovery Furnaces.

1. The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

2. The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. For EPA Method 5, the filter temperature must not exceed 320 degrees Fahrenheit. EPA Method 17, as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated by reference at Rule 62-204.800, F.A.C., may be used if stack temperature is less than 400 degrees Fahrenheit. An adjustment of 0.004 grains per dry standard cubic foot shall be added to the test results when using Method 17. A water wash shall be used with either method.

3. The test method for TRS shall be EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C, as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C pursuant to subsection 62-297.401(16), F.A.C., shall also be required for instrument certification and compliance testing.

(b) Lime Kilns and Calciners.

1. The particulate emissions test method for scrubber controlled emissions units shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. A water wash shall be used.

2. The particulate emissions test method for dry control emissions units shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. An acetone wash shall be used.

3. The test method for TRS shall be EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C, as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C pursuant to subsection 62-297.401(16), F.A.C., shall also be required for instrument certification and compliance testing.

(c) Smelt Dissolving Tank Vents.

1. The particulate emissions test method for scrubber controlled emissions units shall be EPA Method 5 as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. A water wash shall be used.

2. The particulate emissions test method for dry control emissions units shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. An acetone wash shall be used.

3. The test method for TRS shall be EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C, as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C pursuant to subsection 62-297.401(16), F.A.C., shall also be required for instrument certification and compliance testing.

(d) The TRS test method for tall oil plants shall be EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C, as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C pursuant to subsection 62-297.401(16), F.A.C., shall also be required for instrument certification and compliance testing.

(e) Other Combustion Devices used to Incinerate TRS.

1. The particulate emissions test method for scrubber controlled emissions units shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. A water wash shall be used.

2. The particulate emissions test method for dry control emissions units shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 32 dry standard cubic feet. An acetone wash shall be used.

3. The test method for TRS shall be EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C, as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. EPA Method 16 or EPA Method 16A or EPA Method 16B or EPA Method 16C pursuant to subsection 62-297.401(16), F.A.C., shall also be required for instrument certification and compliance testing.

(f) No change.

(5) Continuous Emissions Monitoring Requirements. Each owner or operator of a tall oil plant or kraft (sulfate) pulp mill or tall oil plant shall install continuous monitoring systems for monitoring total reduced sulfur (TRS) emissions, or the performance of total reduced sulfur air pollution control systems as specified in this subsection.

(a) No change.

(b) Continuous determination of total reduced sulfur emissions.

1. A total reduced sulfur continuous emissions monitoring system shall be installed, calibrated, certified and operated pursuant to all of the following provisions:

a. through c. No change.

d. The continuous emissions monitoring system shall be located, installed and certified pursuant to the provisions of 40 C.F.R. Part 60, Appendix B, Performance Specification 2 and Performance Specification 3, and 40 C.F.R. Part 60, Appendix B, Performance Specification 5, which are adopted and incorporated by reference at rule in subsection 62-204.800(7), F.A.C. The exception is that the phrase “or other approved alternative” in Section 3.2 of Performance Specification 5 is not adopted. For the purposes of emissions compliance testing and certification of continuous emissions monitoring systems, 40 C.F.R. Part 60, Appendix A, Reference EPA Method 16, and EPA Method 16A, EPA Method 16B, or EPA Method 16C as described at 40 C.F.R. Part 60, Appendix A-6, adopted and incorporated by reference at rule in subsection 62-204.800(7), F.A.C., shall be are to be used.

e. No change.

f. During any initial emissions compliance tests conducted pursuant to Rule 62-296.404, F.A.C., or within 30 days thereafter, and at such times as there is reason to believe the system does not conform to the performance specifications under this rule (for example, equipment repairs, replacements, excessive drift and such), the owner or operator of any affected emissions unit shall conduct continuous monitoring system performance evaluations and furnish the Department, within sixty days thereof, two copies of a written report of the results of such tests. The report may be submitted electronically to the Department as specified in Rule 62-210.370, F.A.C.  These continuous emissions monitoring systems performance evaluations shall be conducted in accordance with the requirements and procedures contained in sub-subparagraph 62-296.404(5)(b)1.d., F.A.C.

g. through h. No change.

2. No change.

(c) No change.

(d) The owner or operator of any tall oil plant or kraft pulp mill or tall oil plant shall provide the Department with a list of physical and chemical parameters for each regulated total reduced sulfur emissions unit that is not required to be equipped with a total reduced sulfur continuous monitor, which will be regularly monitored to demonstrate that the emissions unit is being operated in a manner that can reasonably be expected to result in compliance with the applicable total reduced sulfur emission limiting standards. The owner or operator shall provide information showing the correlation between the specific magnitudes of the specific surrogate parameters and the associated emissions of total reduced sulfur. The owner or operator shall recommend the frequency and method of monitoring for each parameter. The Department shall issue notice to the company pursuant to Chapter 62-103, F.A.C., that specifies the parameters that are to be monitored, the frequency of monitoring, and the parameter limits that must be maintained. The parameters, parameter limits and frequency of monitoring shall become a modification to the permit for each affected emissions unit. Excess emissions shall be deemed to occur if the parameters exceed the parameter limits specified in the permit. Such parameter limits may be in the form of the applicable total reduced sulfur emission standard, if an equation is used that estimates the 12-hour average total reduced sulfur emission rate based on the surrogate parameter values during each 12-hour averaging period; or the parameter limits may be in the form of specific parameter values that are not to be exceeded (or dropped below) more often than a specified period of time during each 12-hour averaging period.

(6) Quarterly Reporting Requirements. The owner or operator of any digester system, multiple effect evaporator system, condensate stripper system, tall oil plant, kraft recovery furnace, lime kiln, calciner or other emissions unit subject to the provisions of subsection 62-296.404(5), F.A.C. (Continuous Monitoring Requirements), shall submit a written total reduced sulfur emissions and surrogate parameter data report to the Department or local program, as specified in the facility’s permit, postmarked by the 30th day following the end of each calendar quarter. The report may be submitted electronically.

(a) through (c) No change.

(d) The owner or operator of any tall oil plant or kraft pulp mill or tall oil plant shall notify the Department or local program, as specified in the facility’s permit, in writing within fourteen days of the date on which periods of excess emissions exceed the percentages allowed by subparagraphs 62-296.404(6)(c)1. through 3., F.A.C. The notification may be submitted electronically.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(4),17-296.404, Amended 11-23-94, 1-1-96, 3-13-96,___________.

62-296.405 Fossil Fuel Steam Generators with More Than 250 Million Btu Per Hour Heat Input.

(1) Existing Emissions Units Emissions Limits.

(a) Visible emissions – 20 percent opacity except for either one six-minute period per one-hour period during which opacity shall not exceed 27 percent, or one two minute period per hour during which opacity shall not exceed 40 percent. The option selected shall be specified in the emissions unit’s construction and operation permits. Emissions units governed by this visible emission limit shall test for particulate emissions compliance annually and as otherwise required by Chapter 62-297, F.A.C. Emissions units electing to test for particulate matter emissions compliance quarterly shall be allowed visible emissions of 40 percent opacity. The results of such tests shall be submitted to the Department or local program, as specified in the facility’s permit. Upon demonstration that the particulate standard has been regularly complied with, the Secretary, upon petition by the applicant, shall reduce the frequency of particulate testing to no less than once annually.

(b) through (d) No change.

(e) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

1. The test method for visible emissions shall be EPA DEP Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. In lieu of Method 9 testing, a transmissometer utilizing a six 6-minute block average for opacity measurement may be used, provided such transmissometer is installed, certified, calibrated, operated and maintained in accordance with the provisions of 40 C.F.R. Part 75, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

2. The test methods for particulate emissions shall be EPA Methods 17, 5, 5B, or 5F, incorporated and adopted by reference in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet. EPA Method 5 may be used with filter temperature at no more than 320 degrees Fahrenheit. For EPA Method 17, stack temperature shall be less than 375 degrees Fahrenheit. The owner or operator may use EPA Method 5 to demonstrate compliance. EPA Method 3 or 3A with Orsat analysis shall be used when the oxygen base F-factor computed according to EPA Method 19 is used in lieu of heat input. Acetone wash shall be used with EPA Method 5 or 17. Methods 3 and 3A are described at 40 C.F.R. Part 60, Appendix A-2; EPA Methods 5, 5B, and 5F are described at 40 C.F.R. Part 60, Appendix A-3; EPA Method 17 is described at 40 C.F.R. Part 60, Appendix A-6; and EPA Method 19 is described at 40 C.F.R. Part 60, Appendix A-7; adopted and incorporated by reference at Rule 62-204.800, F.A.C.

3. The test methods for sulfur dioxide emissions shall be EPA Methods 6, 6A, 6B or 6C, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. Fuel sampling and analysis may be used as an alternate sampling procedure if such a procedure was incorporated in the operation permit for the emissions unit prior to April 23, 1985. Otherwise, fuel sampling and analysis may be used if the emissions unit obtains an alternate procedure under the provisions of Rule 62-297.620, F.A.C. Such alternate procedure shall become a condition of the emissions unit's permit. The Department will retain the authority to require EPA Method 6 or 6C if it has reason to believe that exceedances of the sulfur dioxide emissions limiting standard are occurring. Results of an approved fuel sampling and analysis program or continuous emissions monitoring program shall have the same effect as EPA Method 6 test results for purposes of demonstrating compliance or noncompliance with sulfur dioxide standards.

4. For emission units not subject to nitrogen oxides continuous monitoring requirements, the test methods for nitrogen oxides emissions shall be EPA Methods 7, 7A, or 7E, as described at 40 C.F.R. Part 60, Appendix A-4 adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. Four grab samples at 15 minute intervals (±2 min.) per run shall be required for EPA Methods 7 and 7A. For emission units that are subject to continuous monitoring requirements under 42 U.S.C. sections 7661 – 7661f or 40 C.F.R. Part 75, emissions of compliance with nitrogen oxides emission limits shall be determined demonstrated based on a 30-day rolling average, except as specifically provided by 40 C.F.R. Parts 60 or 76.  40 C.F.R. Parts 60, 75, and 76 are adopted and incorporated by reference at Rule 62-204.800, F.A.C.

5. No change.

(f) through (g) No change.

(2) New Emissions Units Emissions Limits.

(a) through (d) No change.

(3)3. For the purposes of this rule, nameplate generating capacity means the manufacturer's capacity rating of electrical generating output (expressed in MWe) as designed.

Rulemaking Authority 403.061 FS. Law Implemented 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(5), Amended 6-29-93, Formerly 17-296.405, Amended 11-23-94, 1-1-96, 3-13-96, 3-2-99,___________.

62-296.406 Fossil Fuel Steam Generators with Less Than 250 Million Btu Per Hour Heat Input, New and Existing Emissions Units.

The following standards apply to new and existing emissions units, except for emissions units that would otherwise be exempt from permitting pursuant to subsection 62-210.300(3), F.A.C., and emissions units that would otherwise be considered insignificant pursuant to subparagraph 62-213.300(2)(a)1. or paragraph 62-213.430(6)(b), F.A.C. These standards apply unless otherwise specified by rule, or by order or permit issued by the Department prior to July 15, 1989.

(1) Visible Emissions – shall not exceed 20 percent opacity except for either one six-minute period per one-hour period during which opacity shall not exceed 27 twenty-seven  percent (27%), or one two-minute period per hour during which opacity shall not exceed 40 percent. The option selected shall be specified in the emissions unit’s construction and operation permits. An opacity of 30 percent shall be allowed for emissions units rated at 241 million Btu per hour heat input for which a valid Department operating permit was issued prior to October 1, 1972 in Escambia County, while burning fuel oil in conjunction with waste material derived from waste streams previously discharged into underground wells.

(2) through (3) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(6), Amended 6-29-93, Formerly 17-296.406, Amended 11-23-94, 3-13-96, 3-2-99,___________.

62-296.408 Nitric Acid Plants.

These limits are applicable to new and existing emissions units producing weak nitric acid (50 to 70 percent) by pressure or atmospheric pressure process.

(1) Visible emissions – shall not exceed 10 percent opacity.

(2) Nitrogen Oxides – emissions shall not exceed 3 pounds per ton of acid produced (100 percent basis).

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA DEP Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test methods for nitrogen oxides emissions shall be EPA Methods 7, 7A, 7B, 7C, or 7D, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be as specified in EPA Method 7. Four grab samples at 15 minute intervals (±2 minutes) per run required.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(8), 17-296.408, Amended 11-23-94, 1-1-96,___________.

62-296.409 Sulfur Recovery Plants.

These limits are applicable to plants recovering sulfur from crude oil gas.

(1) New Plants recovering sulfur from crude oil gas – emissions shall not exceed 0.004 pounds of sulfur dioxide per pound of sulfur input to the recovery system or 0.004 pounds of sulfur dioxide per pound of sulfur removed from an oil well.

(2) Existing Plants (for which a valid Department Construction permit was issued prior to July 1, 1973) recovering sulfur from crude oil gas – emissions shall not exceed 0.08 pounds of sulfur dioxide per pound of sulfur input to the recovery system or 0.08 pounds of sulfur dioxide per pound of sulfur removed from crude oil or gas processed.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for sulfur dioxide shall be EPA Method 6, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 0.71 dry standard cubic feet. Two 20-minute samples (+ or - five minutes) per run required.

(b) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(9), 17-296.409, Amended 11-23-94, 1-1-96,___________.

62-296.410 Carbonaceous Fuel Burning Equipment.

(1) Emissions units for which a valid Department operation or construction permit was issued prior to July 1, 1974.

(a) Burners with a capacity less than 30 million Btu per hour heat input – Visible emissions shall not exceed with a density of Number 1 on the Ringelmann Chart (20 percent opacity) except that visible emissions not exceeding 27 percent opacity shall be allowed for one six-minute period in any one-hour period. with a density of Number 2 (40 percent opacity) are permissible for not more than two minutes in any one hour.

(b) Burners with a capacity equal to or greater than 30 million Btu per hour heat input.

1. Visible Eemissions – shall not exceed Visible emissions with a density of Number 1.5 on the Ringelmann Chart (30 percent opacity) except that visible emissions not exceeding 33 percent opacity shall be allowed for one six-minute period in any one-hour period. a density of Ringelmann Number 2 (40 percent opacity) is permissible for not more than two minutes in any one hour.

2. Particulate Matter – shall not exceed 0.3 pounds per million Btu of heat input of carbonaceous fuel plus 0.1 pounds per million Btu heat input of fossil fuel.

(2) New Emissions Units.

(a) Burners of capacity less than 30 million Btu per hour total heat input – Visible emissions shall not exceed Ringelmann Number 1 (20 percent opacity) except that visible emissions not exceeding 27 percent opacity shall be allowed for one-six minute period in any one-hour period. a density of Ringelmann Number 2 (40 percent opacity) is permissible for not more than two minutes in any one hour.

(b) Burners of capacity equal to or greater than 30 million Btu per hour total heat input.

1. Visible Emissions – shall not exceed Number 1.5 on the Ringelmann Chart (30 percent opacity) except that visible emissions not exceeding 33 percent opacity shall be allowed for one six-minute period in any one-hour period. a density of Ringelmann Number 2 (40 percent opacity) is permissible for not more than two minutes in any one hour.

2. Particulate Matter – shall not exceed 0.2 pounds per million Btu of heat input of carbonaceous fuel plus 0.1 pounds per million Btu heat input of fossil fuel.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA DEP Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(10), 17-296.410, Amended 11-23-94, 1-1-96,___________.

62-296.412 Dry Cleaning Facilities.

(1) through (4) No change.

(5) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) No change.

(b) The concentration of organic compounds in the filter residue, per paragraph 62-296.412(2)(1)(c), F.A.C., shall be determined by the procedure specified in EPA-340/1-80-007,“RACT Compliance Guidance for Carbon Adsorbers on Perchloroethylene Drycleaners,” adopted and incorporated by reference at subparagraph 62-297.440(2)(e)2., F.A.C using ASTM 322-67, 1972.

(c) The mass reduction of organic compounds from solvent stills shall be determined using EPA Method 21, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(d) through (f) No change.

Rulemaking Authority 403.061, 403.8055 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(12), 17-296.412, Amended 11-23-94, 4-18-95, 1-1-96, 3-13-96, 6-25-96, 10-7-96, 3-11-10,___________.

62-296.414 Concrete Batching Plants.

The following requirements apply to new and existing emissions units producing concrete and concrete products by batching or mixing cement and other materials. This rule also applies to facilities processing cement and other materials for the purposes of producing concrete, and to equipment used to mix cement and soil for onsite soil augmentation or stabilization.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this subsection shall comply with the following requirements.

(a) The reference test method for visible emissions shall be EPA Method 9, as described at 40 CFR, Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

(b) through (d) No change.

(4) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(14), 17-296.414, Amended 11-23-94, 1-1-96, 11-13-97, 1-10-07,___________.

62-296.415 Soil Thermal Treatment Facilities.

This rule prescribes air pollution control requirements for soil thermal treatment facilities. Soil thermal treatment facilities are only authorized to treat petroleum contaminated soil as defined in Chapter 62-775, F.A.C., Soil Thermal Treatment Facilities. The following requirements apply to all new, modified, and existing soil thermal treatment facilities. All facilities shall comply with these requirements by December 1, 1992.

(1) Volatile Organic Compounds (VOC).

(a) No change.

(b) The average carbon monoxide (CO) emissions shall not exceed 100 parts per million (ppm) by volume, dry basis, during all 60-consecutive-minute periods of plant operation. The average CO emissions is the arithmetic mean of all CO concentration measurements during any consecutive 60 minutes of plant operation that were recorded by the continuous emissions monitor required pursuant to subsection 62-296.415(6) Rule 62-297.500, F.A.C.

(c) through (d) No change.

(2) through (4) No change.

(5) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) The test method for carbon monoxide shall be EPA Method 10, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(d) No change.

(6) Continuous Emissions Monitoring Requirements. Any facility subject to this rule shall be equipped with instruments to continuously monitor and record the temperature and the carbon monoxide concentration of the flue gases leaving the high temperature zone, but before any dilution air is mixed with the flue gases. The temperature monitor shall be certified by the manufacturer to be accurate to within 1% of the temperature being measured. The temperature monitoring system shall be calibrated at least annually by the procedure recommended by the manufacturer. The calibration shall be at a minimum of three temperatures and over a range from 10% below to 10% above the designed flue gas hot zone temperature of the soil thermal treatment facility. Calibration records shall be kept for a minimum of three years. The carbon monoxide monitor shall be certified by the manufacturer to be accurate to within 10% of the carbon monoxide concentration by volume, mean value, or 5% of the applicable standard of 100 ppm, whichever is greater, as determined by EPA Test Method 10, as described at in 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference in subsection 62-204.800(7), F.A.C. The carbon monoxide continuous emission monitoring device shall be certified, calibrated, and operated according to Performance Specification 4 of 40 C.F.R. Part 60, Appendix B, adopted and incorporated by reference in subsection 62-204.800(7), F.A.C., excluding Section 5.2, Calibration Drift Test Period, of Performance Specification 2. Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–New 11-17-92, Formerly 17-296.415, Amended 11-23-94, 1-1-96, 3-13-96,___________.

62-296.416 Waste-to-Energy Facilities.

(1) through (2) No change.

(3) Mercury Emissions Limiting Standards. Waste-to-energy facilities subject to the requirements of this rule shall comply with the mercury emission limiting standards of paragraph 62-296.416(3)(a) or (b), F.A.C., depending on whether the facility chooses to control mercury emissions through the use of post-combustion control equipment designed to remove mercury from flue gases or mercury waste separation, respectively. Facilities choosing to control mercury emissions through the use of mercury control equipment must also comply with the flue gas temperature standard of subsection 62-296.416(4), F.A.C.

(a) through (c) No change.

(d) Mercury Emissions Test Method and Procedures. All mercury emissions tests performed pursuant to the requirements of this rule shall comply with the following provisions.

1. The test method for mercury shall be EPA Method 29, as described at 40 C.F.R. Part 60, Appendix A-8, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

2. No change.

(4) through (5) No change.

(6) Review of Standards. The Department shall review the mercury emission limits contained in subsection 62-296.416(3), F.A.C., and make recommendations to the Environmental Regulation Commission on revising the mercury emission limits no later than July 1, 1998. The review shall include an examination of available mercury emissions data and advances in mercury control technologies and mercury source separation techniques.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–New 10-5-93, Formerly 17-296.416, Amended 11-23-94, 1-1-96, 10-20-96,___________.

62-296.501 Can Coating.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)1., 17-296.501, Amended 11-23-94, 1-1-96,___________.

62-296.502 Coil Coating.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)2., 17-296.502, Amended 11-23-94, 1-1-96,___________.

62-296.503 Paper Coating.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)3., 17-296.503, Amended 11-23-94, 1-1-96,___________.

62-296.504 Fabric and Vinyl Coating.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)4., 17-296.504, Amended 11-23-94, 1-1-96,___________.

62-296.505 Metal Furniture Coating.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7,, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)5., 17-296.505, Amended 11-23-94, 1-1-96,___________.

62-296.506 Surface Coating of Large Appliances.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)6., 17-296.506, Amended 11-23-94, 1-1-96,___________.

62-296.507 Magnet Wire Coating.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)7., 17-296.507, Amended 11-23-94, 1-1-96,___________.

62-296.508 Petroleum Liquid Storage.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) No change.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)8., 17-296.508, Amended 11-23-94, 1-1-96, 10-6-08,___________.

62-296.510 Bulk Gasoline Terminals.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Vapor Control Emissions. The test methods for volatile organic compounds shall be EPA Methods 2A, and 2B, as described at 40 C.F.R. Part 60, Appendix A-1; and EPA Methods 25A and 25B, as described at 40 C.F.R. Part 60, Appendix A-7; adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. sSub-subpargraph 62-297.440(2)(b)1.a., F.A.C., shall also apply.

(b) Equipment Vapor-Leak Detection. The test methods for volatile organic compounds shall be EPA Methods 21, as described at 40 C.F.R. Part 60, Appendix A-7; and EPA Method 27, as described at 40 C.F.R. Part 60, Appendix A-8; adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. sSub-subparagraph 62-297.440(2)(b)2.a., F.A.C., shall also apply.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)10., 17-296.510, Amended 11-23-94, 1-1-96,___________.

62-296.511 Solvent Metal Cleaning.

(1) through (4) No change.

(5) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for volatile organic compound emissions from the specified equipment shall be EPA Method 21, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for non-halogenated organic solvent emissions from a destructive add-on control device shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

(c) The test method for organic solvent emissions from a non-destructive add-on control device shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(d) No change.

Rulemaking Authority 403.061, 403.8055 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)12., 17-296.511, Amended 11-23-94, 1-1-96, 6-25-96, 10-7-96,___________.

62-296.512 Cutback Asphalt.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(b) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)13., 17-296.512, Amended 11-23-94, 1-1-96,___________.

62-296.513 Surface Coating of Miscellaneous Metal Parts and Products.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference. incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)14., 17-296.513, Amended 11-23-94, 1-1-96,___________.

62-296.514 Surface Coating of Flat Wood Paneling.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference. incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)15., 17-296.514, Amended 11-23-94, 1-1-96,___________.

62-296.515 Graphic Arts Systems.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) Low Solvent Technology. The test method for volatile organic compounds shall be EPA Method 24 or, 24A, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or EPA 450/3-84-019, Procedures for Certifying Quantity of Volatile Organic Compounds Emitted by Paint, Ink, and Other Coatings (Dept. of State link), hereby adopted and incorporated by reference incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) Add-on Control Device.

1. Destructive. The test method for volatile organic compounds shall be EPA Method 25, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., or Attachment 3 of EPA 450/ 2-78-041, Alternate Test Method for Direct Measurement of Total Gaseous Organic Compounds Using a Flame Ionization Analyzer, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

2. Non-destructive. The test method for volatile organic compounds shall be EPA VOC Capture Efficiency Test Procedures as described at Rule 62-297.450 subsection 62-297.440(7), F.A.C. The sampling time for each capture efficiency test run shall be at least 8 hours, unless otherwise approved by the Department pursuant to Rule 62-297.620, F.A.C.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)16., 17-296.515, Amended 11-23-94, 1-1-96,___________.

62-296.516 Petroleum Liquid Storage Tanks with External Floating Roofs.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for volatile organic compounds shall be EPA Method 21, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C.; and as described in section 5.2 on p. 5-3 of EPA 450/2-78-047, adopted and incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)17., 17-296.516, Amended 11-23-94, 1-1-96,___________.

62-296.570 Reasonably Available Control Technology (RACT) - Requirements for Major VOC- and NOx-Emitting Facilities.

(1) through (3) No change.

(4) RACT Emission Limiting Standards.

(a) Emissions Testing Compliance Dates and Monitoring.

1. through 2. No change.

3. For units that are not equipped with a continuous emission monitoring system (CEMS) for NOx or VOCs, compliance with the emission limits established in this rule shall be demonstrated by annual emissions testing is required in accordance with applicable EPA Reference Methods from as described in 40 C.F.R. Part 60, Appendices A-1 through A-8, adopted and incorporated by reference at Rule 62-204.800 Rule 62-297.401, F.A.C., or other methods approved by the Department in accordance with the requirements of Rule 62-297.620, F.A.C., except as otherwise provided in paragraph 62-296.570(4)(b), F.A.C. If required, such annual emission testing shall be conducted during each federal fiscal year (October 1 – September 30). Annual emissions compliance testing while firing oil is unnecessary for units operating on oil for less than 400 hours in the current federal fiscal year.

4. No change.

(b) Emission Limiting Standards.

1. through 8. No change.

9. Emissions of NOx from any other external combustion emissions unit subject to the requirements of this rule, and not covered in subparagraph 62-296.570(4)(b)1. through 8., F.A.C., shall not exceed 0.50 lb/million BTU. Emissions Compliance shall be determined demonstrated annually in accordance with the applicable EPA Method from 40 C.F.R. Part 60, Appendices A-1 through A-8, adopted and incorporated by reference at Rule 62-204.800 Rule 62-297.401, F.A.C., or other method approved by the Department in accordance with the requirements of Rule 62-297.620, F.A.C.

10. through 11. No change.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.031, 403.061, 403.087 FS. History–New 2-2-93, Amended 4-17-94, Formerly 17-296.570, Amended 11-23-94, 1-1-96, 3-2-99,___________.

62-296.600 Reasonably Available Control Technology (RACT) - Lead.

(1) through (5) No change.

(6) Emissions Determination Compliance Demonstration. The owner or operator of any facility subject to an emissions limiting standard pursuant to Rules 62-296.602 through 62-296.605, F.A.C., shall determine emissions demonstrate compliance with such limit by the initial compliance date established in the permit required pursuant to this rule, or in accordance with the terms of any construction permit addressing the requirements of this rule, and every five years thereafter unless a more frequent schedule is specified in the permit. Emissions Compliance shall be determined demonstrated as follows:

(a) Compliance with lLead emissions standards shall be determined demonstrated by EPA Method 12, as described at 40 C.F.R. Part 60, Appendix A-5, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) Compliance with oOpacity standards shall be determined demonstrated by EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–New 8-8-94, Formerly 17-296.600, Amended 1-1-96, 3-13-96,___________.

62-296.602 Primary Lead-Acid Battery Manufacturing Operations.

(1) through (2) No change.

(3) Attainment Demonstration. As part of the initial application for the permit required pursuant to subsection 62-296.600(3), F.A.C., the owner or operator of a facility subject to the requirements of this rule shall demonstrate to the Department that, after the application of RACT, the facility shall not cause or contribute to a violation of the national ambient air quality standard for lead as set forth in Rule 62-204.240, F.A.C. The demonstration shall be made using air quality models as provided in 40 C.F.R. Part 51, Appendix W, adopted and incorporated by reference at Rule 62-204.800 subsection 62-204.220(2), F.A.C., and shall address both stack and fugitive emissions.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–New 8-8-94, Formerly 17-296.602, Amended 1-1-96, 3-13-96,___________.

62-296.603 Secondary Lead Smelting Operations.

(1) through (2) No change.

(3) Attainment Demonstration. As part of the initial application for the permit required pursuant to subsection 62-296.600(3), F.A.C., the owner or operator of a facility subject to the requirements of this rule shall demonstrate to the Department that, after the application of RACT, the facility shall not cause or contribute to a violation of the national ambient air quality standard for lead as set forth in Rule 62-272.300, F.A.C. The demonstration shall be made using air quality models as provided in 40 C.F.R. Part 51, Appendix W, adopted and incorporated by reference at Rule 62-204.800 Rule 62-210.500, F.A.C., and shall address both stack and fugitive emissions.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–New 8-8-94, Formerly 17-296.603, Amended 1-1-96,___________.

62-296.604 Electric Arc Furnace Equipped Secondary Steel Manufacturing Operations.

(1) through (2) No change.

(3) Attainment Demonstration. As part of the initial application for the permit required pursuant to subsection 62-296.600(3), F.A.C., the owner or operator of a facility subject to the requirements of this rule shall demonstrate to the Department that, after the application of RACT, the facility shall not cause or contribute to a violation of the national ambient air quality standard for lead as set forth in Rule 62-272.300, F.A.C. The demonstration shall be made using air quality models as provided in 40 C.F.R. Part 51, Appendix W, adopted and incorporated by reference at Rule 62-204.800 Rule 62-210.500, F.A.C., and shall address both stack and fugitive emissions.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–New 8-8-94, Formerly 17-296.604, Amended 1-1-96,___________.

62-296.701 Portland Cement Plants.

(1) through (3) No change.

(4) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) For emissions units subject to the visible emissions standard in subsection 62-296.701(2), F.A.C., the test method shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) A transmissometer shall be used to determine the opacity of compliance with the visible emissions regulated by the standard in subsection 62-296.701(3), F.A.C. The transmissometer shall be calibrated in accordance with 40 C.F.R. Part 60, Appendix B, Rule 62-297.520, F.A.C., and 40 C.F.R. 60.13, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

(c) The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(d) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)1., Amended 6-29-93, Formerly 17-296.701, Amended 11-23-94, 1-1-96,___________.

62-296.702 Fossil Fuel Steam Generators.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. A transmissometer may be used and calibrated in accordance with 40 C.F.R. Part 60, Appendix B, Rule 62-297.520, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 17, as described at 40 C.F.R. Part 60, Appendix A-6, EPA Method 5B as described at 40 C.F.R. Part 60, Appendix A-3;, or EPA Method 5F, as described at 40 C.F.R. Part 60, Appendix A-3; adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet. EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated by reference at Rule 62-204.800, F.A.C., may be used with the filter temperature at no more than 320 degrees Fahrenheit. For EPA Method 17, stack temperature shall be less than 375 degrees Fahrenheit. The owner or operator may use EPA Method 5 to demonstrate compliance. EPA Method 3 or 3A with Orsat analysis, as described at 40 C.F.R. Part 60, Appendix A-2, adopted and incorporated by reference at Rule 62-204.800, F.A.C., shall be used when oxygen based F factor computed according to EPA Method 19, as described at 40 C.F.R. Part 60, Appendix A-7, adopted and incorporated by reference at Rule 62-204.800, F.A.C., is used in lieu of heat input. Use Acetone wash with Method 5 or 17.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)2., 17-296.702, Amended 11-23-94, 1-1-96,___________.

62-296.703 Carbonaceous Fuel Burners.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet. For EPA Method 5, the filter temperature may not exceed 320 degrees Fahrenheit.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)3., 17-296.703, Amended 11-23-94, 1-1-96,___________.

62-296.704 Asphalt Concrete Plants.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5 or EPA Method 5A, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)4., 17-296.704, Amended 11-23-94, 1-1-96,___________.

62-296.705 Phosphate Processing Operations.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5 or EPA Method 5A, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)5., 17-296.705, Amended 11-23-94, 1-1-96,___________.

62-296.706 Glass Manufacturing Process.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5 or EPA Method 5A, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)6., 17-296.706, Amended 11-23-94, 1-1-96,___________.

62-296.707 Electric Arc Furnaces.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5 or EPA Method 5D, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)7., 17-296.707, Amended 11-23-94, 1-1-96,___________.

62-296.708 Sweat or Pot Furnaces.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)8., 17-296.708, Amended 11-23-94, 1-1-96,___________.

62-296.709 Lime Kilns.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)9., 17-296.709, Amended 11-23-94, 1-1-96,___________.

62-296.710 Smelt Dissolving Tanks.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)10., 17-296.710, Amended 11-23-94, 1-1-96,___________.

62-296.711 Materials Handling, Sizing, Screening, Crushing and Grinding Operations.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) through (d) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)11., 17-296.711, Amended 11-23-94, 1-1-96,___________.

62-296.712 Miscellaneous Manufacturing Process Operations.

(1) through (2) No change.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated and adopted by reference at Rule 62-204.800 in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) through (d) No change.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)12., 17-296.712, Amended 11-23-94, 1-1-96,___________.

NAME OF PERSON ORIGINATING PROPOSED RULE: Cindy Phillips, Division of Air Resource Management

NAME OF AGENCY HEAD WHO APPROVED THE PROPOSED RULE: Herschel T. Vinyard Jr., Secretary

DATE PROPOSED RULE APPROVED BY AGENCY HEAD: April 1, 2014

DATE NOTICE OF PROPOSED RULE DEVELOPMENT PUBLISHED IN FAR: September 5, 2013