- 25-4.0345 Customer Premises Equipment and Inside Wire.

(1) Definitions: For purposes of this chapter, the definition to the following terms apply:

(a) “Customer Premises Equipment (CPE).” Includes terminal equipment intended for use on the customer’s premises such as pay telephones, telephone sets, teletypewriters, data terminal equipment, mobile telephone terminal equipment, private branch exchange equipment, key system equipment, dialers and other supplemental equipment. CPE does not include “911” public safety answering point equipment (ALI, ANI, ACD equipment) or telecommunications devices required by hearing or speech impaired subscribers.

(b) “Demarcation Point.” The point of physical interconnection (connecting block, terminal strip, jack, protector, optical network interface, or remote isolation device) between the telephone network and the customer’s premises wiring. Unless otherwise ordered by the Commission for good cause shown, the location of this point is:

1. Single Line/Single Customer Building – Either at the point of physical entry to the building or a junction point as close as practicable to the point of entry.

2. Single Line/Multi Customer Building – Within the customer’s premises at a point easily accessed by the customer.

3. Multi Line Systems/Single or Multi Customer Building – At a point within the same room and within 25 feet of the FCC registered terminal equipment or cross connect field.

4. Temporary Accommodations Subscriber Premises with Inadequate Grounding (e.g., some mobile homes, trailers, houseboats, construction modules) – On a permanent stake, pole, or structure with a suitable safety ground.

(c) “Complex Equipment Wire” The premises wiring owned by the local exchange company which may be used as station wiring and to connect off-premises extensions and is beyond the normal demarcation points.

(d) “Inside Wire” All wire or cable other than complex equipment wire located on the customer’s side of the demarcation point.

(e) “Customer Premises” The discrete real property owned, leased, or controlled by a customer for the customer’s own business or residential purposes.

(2) The provision and maintenance of CPE and inside wire, but not complex equipment wire, is deregulated for intrastate purposes.

(3) Network facilities up to and including the demarcation point are part of the telephone network, provided and maintained by the telecommunications company under tariff.

(4) CPE Network Responsibility. No CPE may harm the network by introducing signals that interfere or affect other subscribers or network operations.

Rulemaking Authority 350.127(2) FS. Law Implemented 364.01(4), 364.03 FS. History–New 12-13-82, Amended 9-30-85, Formerly 25-4.345, Amended 4-16-90, 3-10-96, 2-1-99.