- 62-296.512 Cutback Asphalt.

(1) Applicability. The emission limiting standard or control technology set forth in subsection 62-296.512(2), F.A.C., shall apply to the manufacture and use of cutback asphalts for paving or maintaining roads, streets, highways, and parking lots.

(2) Control Standards. No person shall cause, allow, or permit the manufacture, mixing, storage, use, or application of cutback asphalts except where:

(a) Long-life storage of liquid asphalt is necessary, or

(b) Stockpile storage of cold mixed asphaltic concrete patching material is necessary, or

(c) The use or application at ambient temperature less than 50 degrees Fahrenheit (10 degrees Celsius) as determined by the nearest National Weather Service Station is necessary, or

(d) The cutback asphalt is to be used solely as a penetrating prime coat, or

(e) The cutback asphalt is to be used in a sand seal coat, or

(f) The cutback asphalt is to be used as a tack coat in the routine maintenance of public roads, or the minor betterment of public roads.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for particulate emissions shall be EPA Method 5, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated by reference at Rule 62-204.800, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(b) Test procedures shall meet all applicable requirements of Chapter 62-297, F.A.C.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(1)(f)13., 17-296.512, Amended 11-23-94, 1-1-96, 7-10-14.