- 62-296.706 Glass Manufacturing Process.

(1) Applicability. The emission limitations set forth in Rule 62-296.706, F.A.C., shall apply to glass melting furnaces producing container glass.

(2) Emission Limitations. No owner or operator of a glass melting furnace shall cause, permit, or allow emissions of particulate matter in excess of the following standards:

(a) Gas fired furnaces – 1.3 pounds per ton of glass produced.

(b) Oil fired furnaces – 1.5 pounds per ton of glass produced.

(c) Visible emissions – 20 percent opacity.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5 or EPA Method 5A, as described at 40 C.F.R. Part 60, Appendix A-3, adopted and incorporated by reference at Rule 62-204.800, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) Test procedures shall meet all applicable requirements of Chapter 62-297, F.A.C.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)6., 17-296.706, Amended 11-23-94, 1-1-96, 7-10-14.