31-13.004 Building Sewers and Connections.

(1) No unauthorized person shall uncover, make any connections with or opening into, use, alter, or disturb any public sewer or appurtenance thereof without first obtaining a written permit from the Executive Director.

(2) All costs and expenses incidental to the installation and connection of the building sewer shall be borne by the owner. The owner shall indemnify the District for any loss or damage that may directly or indirectly be occasioned by the installation of the building sewer.

(3) Old building sewers may be used in connection with new buildings only when they are found, on examination and test by the Executive Director, to meet all requirements of this Rule.

(4) The size, slope, alignment, and materials of construction of a building sewer, and the methods to be used in excavating, placing of the pipe, jointing, connecting to the public sewer, testing, and backfilling the trench shall conform to the requirements of applicable building and plumbing codes and rules of the District. In the absence of code provisions or in amplification thereof, the materials and procedures set forth in appropriate specifications of the latest edition of the ASTM and WPCF Manual of Practice shall apply, incorporated herein by reference.

(5) Whenever possible, the building sewer shall be brought to the building at an elevation below the ground, or if applicable, basement floor. In all buildings in which any building drain is too low to permit gravity flow to the public sewer, sanitary sewage carried by such building drain shall be lifted by an approved means and discharged to the building sewer.

(6) No person shall make connections of roof downspouts, foundation drains, areaway drains, or other sources of surface runoff or groundwater to a building sewer or building drain which in turn is connected directly or indirectly to a public sanitary sewer.

(7) All connections to the public sewer shall be made watertight and verified by proper testing. Any deviation from the prescribed procedures and materials must be approved by the Executive Director before installation.

(8) The applicant for the building sewer permit shall notify the Executive Director when the building sewer is ready for inspection and connection to the public sewer. The connection and testing shall be made under the supervision of the District’s representatives.

(9) All excavations for building sewer installation shall be adequately guarded with barricades and lights so as to protect the public from hazard. Streets, sidewalks, parkways, and other public property disturbed in the course of the work shall be restored in a manner satisfactory to the District which would be reasonable under the circumstances.

Specific Authority Chapter 78-559, Section 6(19). Law Implemented Chapter 71-822, Sections 6(2), (10); Section 12 as amended by Chapter 78-559. History–New 5-5-85, Formerly 31-13.04.