- 64E-4.001 Registration.

(1) Except as otherwise specifically exempted, all persons who receive, possess, acquire, transfer, own, manufacture, or use lasers which emit or may emit laser radiation shall register within 30 days after acquisition of a laser product and comply with these rules. Nothing in these rules shall be interpreted as limiting the intentional exposure of patients to laser radiation for the purpose of treatment or use commensurate with the licensed practitioner’s use of the healing arts.

(2) A separate registration is required for each location with a laser device, on DH Form 1605, 09/14, “Laser Device Registration Form,” which is herein incorporated by reference and which is available from the internet at https://www.flrules.org/Gateway/reference.asp?No=Ref-04839 or www.FloridaHealth.gov/radiation. Laser manufacturers must register on DH Form 1604, 09/14, “Manufacturer’s Laser Device Registration Form,” which is herein incorporated by reference and which is available from the internet at https://www.flrules.org/Gateway/reference.asp?No=Ref-04838 or www.FloridaHealth.gov/radiation. Subsequent registrations for each facility or mobile laser facility are required whenever additional laser devices are installed or previously registered laser devices are disposed of, altered, destroyed or moved to new locations. Subsequent registrations for each manufacturer are required only when the description of the laser devices manufactured by the manufacturer changes.

(3)(a) Laser products certified by a manufacturer to be compliant with the Federal laser product performance standard of 21 C.F.R. Part 1040 in effect at the date of manufacture shall be maintained in compliance with such requirements. Certified laser products which have been modified shall comply with these rules.

(b) Uncertified lasers shall meet the requirements of these rules.

(4) If any conflict arises between the requirements of these rules and the federal laser product performance standard, the requirements of the federal standard shall apply.

(5) Completed registration forms or other correspondence or reports required by Chapter 64E-4, F.A.C., to be sent in writing to the Department can be mailed to Florida Department of Health, Bureau of Radiation Control, 4052 Bald Cypress Way, Bin #C-21, Tallahassee, Florida 32399-1741; or faxed to (850)487-0435; or scanned and emailed to RadiationControl@FlHealth.gov in PDF, JPG, or TIF format.

Rulemaking Authority 501.122(2) FS. Law Implemented 501.122(2) FS. History–New 9-6-84, Amended 5-7-96, 12-12-96, Formerly 10D-89.101, Amended 4-20-15.