40B-8.041 Minimum Surfacewater Levels and Flows for Lower Suwannee River, Little Fanning Spring, Fanning Spring, and Manatee Spring.

The Governing Board hereby establishes the following minimum surfacewater flows and levels. The Governing Board finds that the following minimum surfacewater flows and levels are the limit at which further withdrawals would be significantly harmful to the water resources or ecology.

(1) The minimum surfacewater flow for Manatee Spring is the flow that will maintain 90 percent of the historic flow regime for the Manatee Spring discharge as defined in “Technical Report – MFL Establishment for the Lower Suwannee River and Estuary, Little Fanning, Fanning, and Manatee Springs, October 2005.”

(2) In addition to subsection (1) above, the minimum surface water flow for Manatee Spring for the period of November 1 through April 30 is 130 cubic feet per second for a flow duration frequency of 50 percent.

(3) The minimum surfacewater flow for Fanning and Little Fanning Spring is the flow that will maintain 90 percent of the historic flow regime for the Fanning Spring discharge as defined in “Technical Report – MFL Establishment for the Lower Suwannee River and Estuary, Little Fanning, Fanning, and Manatee Springs, October 2005.”

(4) The minimum surface water level for Fanning Spring for the period of November 1 through April 30 is 2.71 feet (NGVD) for a stage duration frequency of 85 percent.

(5) The minimum surfacewater flow for the Lower Suwannee River below the Wilcox gauge is 6,600 cubic feet per second for a flow duration frequency of 50 percent for the period May 1 through October 31. For the period November 1 through April 30, the minimum surfacewater flow is 7,600 cubic feet per second for a flow duration frequency of 50 percent.

Specific Authority 373.044, 373.113 FS. Law Implemented 373.042, 373.0421, 373.103, 373.415 FS. History–New 8-7-06.