40C-4.301 Conditions for Issuance of Permits.

(1) In order to obtain a standard, individual, or conceptual approval permit under this chapter or Chapter 40C-40, F.A.C., an applicant must provide reasonable assurance that the construction, alteration, operation, maintenance, removal or abandonment of a surface water management system:

(a) Will not cause adverse water quantity impacts to receiving waters and adjacent lands;

(b) Will not cause adverse flooding to on-site or off-site property;

(c) Will not cause adverse impacts to existing surface water storage and conveyance capabilities;

(d) Will not adversely impact the value of functions provided to fish and wildlife and listed species by wetlands and other surface waters;

(e) Will not adversely affect the quality of receiving waters such that the water quality standards set forth in Chapters 62-3, 62-4, 62-302, 62-520, 62-522, and 62-550, F.A.C., including any antidegradation provisions of paragraphs 62-4.242(1)(a) and (b), subsections 62-4.242(2) and (3), and Rule 62-302.300, F.A.C., and any special standards for Outstanding Florida Waters and Outstanding National Resource Waters set forth in subsections 62-4.242(2) and (3), F.A.C., will be violated;

(f) Will not cause adverse secondary impacts to the water resources;

(g) Will not adversely impact the maintenance of surface or ground water levels or surface water flows established in Chapter 40C-8, F.A.C.;

(h) Will not cause adverse impacts to a work of the District established pursuant to Section 373.086, F.S.;

(i) Will be capable, based on generally accepted engineering and scientific principles, of being performed and of functioning as proposed;

(j) Will be conducted by an entity with the financial, legal and administrative capability of ensuring that the activity will be undertaken in accordance with the terms and conditions of the permit, if issued; and

(k) Will comply with any applicable special basin or geographic area criteria established in Chapter 40C-41, F.A.C.

(2) If the applicant is unable to meet water quality standards because existing ambient water quality does not meet standards, the applicant must comply with the requirements set forth in subsection 12.2.4.5 of the Applicant’s Handbook: Management and Storage of Surface Waters.

(3) The standards and criteria, including the mitigation provisions and the provisions for elimination or reduction of impacts, contained in the Applicant’s Handbook: Management and Storage of Surface Waters adopted by reference in Rule 40C-4.091, F.A.C., shall determine whether the reasonable assurances required by subsection 40C-4.301(1) and Rule 40C-4.302, F.A.C., have been provided.

Specific Authority 373.016, 373.044, 373.113, 373.171 FS. Law Implemented 373.016, 373.042, 373.409, 373.413, 373.414, 373.416, 373.426 FS. History–New 1-31-77, Formerly 16I-4.10, 40C-4.10, Amended 2-3-81, 12-7-83, Formerly 40C-4.301, 40C-4.0301, Amended 9-25-91, 9-16-92, 6-7-93, 10-3-95, 10-11-01.