- 40C-21.631 Phase II Severe Water Shortage.

The following restrictions shall apply when a Phase II water shortage is declared by the District.

(1) Essential/Domestic/Utility/Commercial.

(a) Essential Use.

1. The use of water for firefighting and health or medical purposes will not be restricted.

2. Fire hydrant flushing will be undertaken only on an emergency basis.

3. Sanitary sewer line flushing and testing will not be restricted except on a voluntary basis.

(b) Household and Domestic Type Use.

1. Residential type domestic use should be voluntarily reduced to 50 gallons per person per day.

2. Domestic type use in industrial and commercial establishments should be voluntarily reduced.

(c) Water Utility Use.

1. Initial pressure at the point of use (meter) will be reduced by 15% where it is operationally feasible to do so. Prior to the reduction of pressure, the utility will notify the appropriate firefighting agencies and make arrangements for direct communication when additional pressure is required.

2. New water line flushing and disinfection will be restricted to the hours of 9:00 p.m. to 7:00 a.m., seven days per week.

3. As appropriate the utility should institute additional voluntary conservation measures such as reclaiming of backwash water, improving and accelerating leak detection surveys and repair programs, installing and calibrating meters, and stabilizing and equalizing system pressures.

(d) Power Production Use. Water used for power production should be voluntarily reduced.

(e) Commercial and Industrial Process Use.

1. Commercial car washes will be restricted as follows:

a. For washes servicing passenger vehicles and mobile equipment weighing less than 10,000 pounds:

(I) Use in excess of 75 gallons per wash will be prohibited, and

(II) Use equal to or less than 75 gallons per wash should be voluntarily reduced.

b. For washes servicing mobile equipment weighing 10,000 pounds or more:

(I) Use in excess of 150 gallons per wash will be prohibited, and

(II) Use equal to or less than 150 gallons per wash should be voluntarily reduced.

2. Water use for commercial and industrial processes should be voluntarily reduced.

(2) Agriculture.

(a) Agricultural Use.

1. Overhead irrigation, including portable volume guns, will be restricted to the hours of 2:00 p.m. to 10:00 a.m.

2. Low volume irrigation hours will not be restricted.

3. Flood and seepage irrigation will not discharge tailwaters from the property.

4. All irrigation systems will be operated in a manner that will maximize the percentage of water withdrawn and held which is placed in the root zone of the crop and will minimize the amount of water which is withdrawn and released or lost to the user but is not immediately available for other users.

5. Users having access to more than one source class will maximize the use of the lesser or least restricted source class.

(b) Livestock Use. Livestock water use should be voluntarily reduced.

(c) Aquacultural Use. Aquacultural water use should be voluntarily reduced.

(d) Soil Flooding.

1. Soil flooding for pest control or soil preservation will be prohibited.

2. Soil flooding to permit harvesting of sod should be voluntarily reduced.

(e) Freeze Protection. Water use for freeze protection will be restricted to situations in which official weather forecasting services predict temperatures likely to cause permanent damage to crops and when the wet bulb temperature drops to 34º Fahrenheit or lower.

(3) Nursery/Landscape Irrigation/Recreation.

(a) Nursery Use.

1. Low volume irrigation uses and low volume hand watering should be voluntarily reduced.

2. Overhead irrigation uses will be restricted as follows:

a. Inside – 8:00 a.m. to 8:00 p.m., seven days per week.

b. Outside – 7:00 p.m. to 7:00 a.m., on odd numbered days.

3. Flood irrigation systems will be restricted to 6 days per month.

4. Applications of fertilizers, herbicides and pesticides should be coordinated with irrigation schedules.

(b) Landscape Irrigation – New Installation.

1. For installations which have been in place for less than 30 days, and:

a. Less than 5 acres in size, water use for irrigation shall be restricted to the hours of 2:00 a.m. to 8:00 a.m., Monday, Wednesday and Friday.

b. 5 acres or greater in size, water use for irrigation shall be restricted to the hours of 12:01 a.m. to 8:00 a.m., Monday, Wednesday, Thursday and Friday.

2. Low volume hand watering of new landscaping should be voluntarily reduced.

3. Cleaning and adjusting of new irrigation systems will be restricted to 10 minutes per zone on a one time basis.

(c) Landscape Irrigation – Existing Installation.

1. For existing installations less than 5 acres in size, water use for irrigation will be restricted to the hours of 4:00 a.m. to 8:00 a.m., two days per week.

a. Installations with odd addresses will be permitted to irrigate on Wednesday and Saturday.

b. Installations with even addresses or no address will be permitted to irrigate on Thursday and Sunday.

2. For existing installations 5 acres or greater in size, water use for irrigation will be restricted to the hours of 12:01 a.m. to 8:00 a.m., two days per week.

a. Installations with odd addresses will be permitted to irrigate on Wednesday and Saturday.

b. Installations with even addresses or no address will be permitted to irrigate on Thursday and Sunday.

3. Applications of fertilizers, herbicides and pesticides should be coordinated with irrigation schedules.

(d) Recreation Area Use. Existing and new recreation area water use will be restricted as provided in paragraphs (3)(b) and (c) respectively.

(e) Golf Course Use.

1. Irrigation of greens and tees should be voluntarily reduced and will be accomplished during non-daylight hours.

2. Irrigation of fairways, roughs and nonplaying areas on the first nine holes of the course will be restricted to the hours of 12:01 a.m. to 8:00 a.m. on Wednesday and Saturday.

3. Irrigation of fairways, roughs and nonplaying areas on the last nine holes of the course will be restricted to the hours of 12:01 a.m. to 8:00 a.m. on Thursday and Sunday.

4. Applications of fertilizers, herbicides and pesticides should be coordinated with irrigation schedules.

(f) Water Based Recreation Use.

1. Water based recreation water use should be voluntarily reduced.

2. Draining of facilities into sewers or onto impervious surfaces will be prohibited.

3. Existing facilities will not be refilled except for makeup water.

(4) Miscellaneous.

(a) Heating and Air Conditioning Use.

1. The use of water for heating will be restricted to that amount of water necessary to maintain a maximum temperature of 68 degrees Fahrenheit.

2. The use of water for air conditioning will be restricted to that amount of water necessary to maintain a minimum temperature of 78 degrees Fahrenheit.

(b) Dewatering Use. Discharge of fresh water to tide from dewatering will be prohibited.

(c) Other Outside Uses.

1. Washing or cleaning streets, driveways, sidewalks, or other impervious areas with water will be prohibited.

2. Mobile equipment washing with water will be restricted to the hours and days prescribed for existing landscape irrigation in paragraph (3)(c) using only low volume methods.

3. Outside pressure cleaning will be restricted to only low volume methods, seven days per week.

(d) Diversion and Impoundment into non-District Facilities. Water used for diversion and impoundment into non-District facilities should be voluntarily reduced.

(e) Aesthetic Use.

1. Outside aesthetic uses of water will be prohibited.

2. Inside aesthetic uses of water should be voluntarily reduced.

Rulemaking Authority 373.044, 373.113 FS. Law Implemented 373.175, 373.246 FS. History–New 1-1-84.