40C-42.0225 Exemptions From Permitting for Stormwater Management Systems.

The following types of stormwater management systems are exempt from the notice and permit requirements of this chapter:

(1) Systems designed to accommodate only one single family dwelling unit, duplex, triplex, or quadruplex, provided the single unit, duplex, triplex or quadruplex is not part of a larger common plan of development or sale.

(2) Systems which are designed to serve single family residential projects, including duplexes, triplexes and quadruplexes, of less than 10 acres total land area and which have less than 2 acres impervious surface and if the systems:

(a) Comply with all regulations or ordinances applicable to stormwater management adopted by a city or county;

(b) Are not part of a larger common plan of development or sale; and

(c) Discharge into a stormwater management system exempted or permitted by the District under this chapter which has sufficient capacity and treatment capability as specified in this chapter and is owned, maintained, or operated by a city, county, special district with drainage responsibility, or water management district; however, this exemption does not authorize discharge to a system without the system owner’s prior written consent.

(3) Systems that qualify for a noticed general permit pursuant to Chapter 40C-400, F.A.C., and which comply with the requirements of such noticed general permit.

(4) Construction of communication tower sites with one-half acre or less of impervious or semi-impervious surface such as access roads, buildings, and equipment pads. The design of above-grade access roads must allow for conveyance of normal surface water flows.

(5) The construction of the following minor roadway safety projects provided that the capacity of existing swales, ditches, or other stormwater management systems is not reduced:

(a) Sidewalks,

(b) Turnlanes less than 0.25 miles in length and other intersection improvements, or

(c) Road widening and shoulder paving projects which do not result in the creation of additional traffic lanes.

(6) Recreational paths which do not allow motorized vehicles powered by internal combustion engines, except for maintenance and emergency vehicles.

Specific Authority 373.044, 373.113, 373.414, 373.418 FS. Law Implemented 373.413, 373.414, 373.416, 373.418, 403.813(2) FS. History–New 9-25-91, Amended 3-21-93, 10-3-95, 10-11-01.