40D-2.041 Permits Required.

(1) Unless expressly exempted by law or District rule, a Water Use Permit must be obtained from the District prior to withdrawal of water if any of the following thresholds are exceeded:

(a) Total withdrawal capacity from any source or combined sources is greater than or equal to 1,000,000 gallons per day (gpd).

(b) Annual average withdrawal from any source or combined sources is greater than or equal to 100,000 gpd.

(c) Withdrawal is from a well having an outside diameter of 6 inches or more at the surface.

(d) Withdrawal is from a surface water body and the outside diameter of the withdrawal pipe or the sum of the outside diameters of the withdrawal pipes is 4 inches or greater.

(e) In addition to the thresholds set forth in paragraphs (1)(a) through (1)(d) above, a permit is required within the Most Impacted Area (MIA) as set forth in Section 7.2.8.A. and B., and Figure 7.2-2, Basis of Review for Water Use Permit Applications, when withdrawal is from wells having a cumulative outside diameter greater than 6 inches at the surface any of which wells is constructed after April 11, 1994. This paragraph (e) shall not apply to any proposed well less than 6 inches in diameter at the surface when it is of the same diameter or smaller than a well it replaces and an application to plug the replaced well in accordance with Rule 40D-3.531, F.A.C., is filed with the application to construct the replacement well in accordance with Rule 40D-3.041, F.A.C.

(2) Water Use Permits must receive approval of the Governing Board if the annual average daily withdrawal equals or exceeds 500,000 gpd.

(3) Water Use Permits with an annual average daily withdrawal less than 500,000 gpd may be issued by District staff as general permits.

(4) The District shall require a water use permit when evidence indicates the withdrawal is likely to cause significant adverse impacts to existing water or land uses or the water resource.

Specific Authority 373.044, 373.113, 373.118, 373.149, 373.171, 373.216, 373.249 FS. Law Implemented 373.219, 373.223, 373.224, 373.226 FS. History–Readopted 10-5-74, Amended 12-31-74, 10-24-76, 9-4-77, 10-16-78, Formerly 16J-2.04(1), (2), (4), (5), Amended 9-1-84, 11-4-84, 10-1-89, 2-10-93, 4-11-94.