40D-8.626 Minimum Aquifer Levels in Hillsborough County North of State Road 60, and Pasco and Pinellas Counties.

Salt Water Intrusion Minimum Aquifer Levels. Salt Water Intrusion within this area has not been regional in nature, but is of localized concern in some coastal areas. The District has chosen the North Pinellas, Southwest Pasco and Northwest Hillsborough Counties region as the priority area for the establishment of Salt Water Intrusion Minimum Aquifer Levels (“SWIMAL”) to prevent regional seawater intrusion because of the availability of data and the potential for future intrusion. The SWIMALs for the Upper Floridan aquifer shall be established as follows:

(1) The location of the SWIMAL sites serves to monitor the area where early signs of regional salt water intrusion can best be detected.

(2) The SWIMALs shall be established by setting levels in coastward transects of wells whose potentiometric levels define the Long-term average hydraulic gradient of the Upper Floridan aquifer potentiometric surface.

(3) The Salt Water Intrusion Minimum Aquifer Levels are as set forth in the following table. Aquifer levels are deemed to be below the SWIMAL when the Long-term average elevation is below the SWIMAL.

TABLE 8-4 SALT WATER INTRUSION

MINIMUM AQUIFER LEVELS

|  | Well Name | Saltwater Intrusion Minimum |
| --- | --- | --- |
|  |  | Aquifer Level (feet NGVD) |
|  | Sheldon Road Deep (RMP-16D) | 7.7 |
|  | RMP-13D | 16.4 |
|  | RMP-8D1 | 26.8 |
|  | Tarpon Rd. Deep | 9.9 |
|  | SWI-18S | 14.8 |
|  | 201-M | 13.7 |
|  | EW 2S Deep | 10.3 |

Specific Authority 373.044, 373.113, 373.171 FS. Law Implemented 373.036, 373.0361, 373.0395, 373.042, 373.0421 FS. History–New 8-7-00.