- 62-600.430 Additional Treatment ‒ Water Quality-Based Effluent Limitations (WQBELs).

(1) Surface water discharge.

(a) In addition to the minimum treatment standards (TBELs) specified in Rule 62-600.420, F.A.C., facilities shall provide additional treatment when necessary to meet water quality standards for receiving surface waters pursuant to Chapter 62-302, F.A.C.

(b) The WQBELs shall be determined by the Department in accordance with Chapters 62-304 and 62-650, F.A.C., and shall be based upon the characteristics of the discharge, the receiving water characteristics, and the criteria and standards of Chapters 62-4 and 62-302, F.A.C., and this chapter. Requests for zones of mixing and any previous approved zones of mixing will be taken into consideration when determining WQBELs. No zone of mixing, as contained in Rule 62-4.244, F.A.C., shall be provided for any parameters for which the permittee fails or declines to provide the necessary characteristics of the discharge. WQBELs shall be met after disinfection.

(2) Ground water discharge.

(a) In addition to the TBELs specified in Rule 62-600.420, F.A.C., facilities shall provide additional treatment when needed to satisfy water quality standards for receiving ground waters.

(b) Such limitations shall be established based on the provisions of Chapters 62-304, 62-528, and 62-650, and subsection 62-610.850(2), F.A.C.

Rulemaking Authority 403.051, 403.061, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, Formerly 17-600.430, Amended 2-8-16.