40E-22.242 Minimum Flows.

(1) The following minimum monthly flows shall be maintained:

(a) The monthly discharge at Structure 48 shall not be less than:

|  | MONTH | MINIMUM FLOW |
| --- | --- | --- |
|  |  | (in acre-feet) |
|  | January | 292 |
|  | February | 329 |
|  | March | 365 |
|  | April | 219 |
|  | May | 401 |
|  | June | 1,168 |
|  | July | 1,533 |
|  | August | 1,533 |
|  | September | 1,680 |
|  | October | 2,118 |
|  | November | 694 |
|  | December | 292 |

(b) The monthly discharge at Structure 49 shall not be less than:

|  | MONTH | MINIMUM FLOW |
| --- | --- | --- |
|  |  | (in acre-feet) |
|  | January | 439 |
|  | February | 319 |
|  | March | 319 |
|  | April | 280 |
|  | May | 439 |
|  | June | 1,636 |
|  | July | 998 |
|  | August | 1,636 |
|  | September | 1,277 |
|  | October | 1,636 |
|  | November | 430 |
|  | December | 240 |

(c) The monthly discharge at Structure 50 shall not be less than:

|  | MONTH | MINIMUM FLOW |
| --- | --- | --- |
|  |  | (in acre-feet) |
|  | January | 480 |
|  | February | 400 |
|  | March | 560 |
|  | April | 520 |
|  | May | 440 |
|  | June | 1,279 |
|  | July | 1,639 |
|  | August | 1,399 |
|  | September | 1,199 |
|  | October | 1,679 |
|  | November | 1,439 |
|  | December | 399 |

(2) If Canal 23, 24, or 25 is at or below the following levels the minimum flow requirement for that canal will be suspended until the canal exceeds these levels:

(a) Dry Season (October 15 through May14)

|  | CANAL | LEVEL |
| --- | --- | --- |
|  |  | (feet above mean sea level) |
|  | C-23 above Structure 97 | 23.2 |
|  | C-24 above Structure 49 | 21.2 |
|  | C-25 above Structure 99 | 21.2 |

(b) Wet Season (May 15 through October 14)

|  | CANAL | LEVEL |
| --- | --- | --- |
|  |  | (feet above mean sea level) |
|  | C-23 above Structure 97 | 22.2 |
|  | C-24 above Structure 49 | 20.2 |
|  | C-25 above Structure 99 | 20.2 |

Specific Authority 373.044, 373.113 FS. Law Implemented 373.042, 373.086, 373.103(4) FS. History–New 9-3-81, Formerly 16K-31.03, 40E-21.242.