40E-24.201 Year-Round Landscape Irrigation Measures.

(1) The year-round landscape irrigation measures contained in this chapter are applicable to all users as defined in subsection 40E-24.101(9), F.A.C., including permitted and exempt users under Chapter 40E-2, F.A.C., unless indicated otherwise herein. These measures apply to all water sources, except that landscape irrigation accomplished using reclaimed water is subject to only paragraph (5)(a) of this section. In addition to the requirements of this section, all permitted users under Chapter 40E-2, F.A.C., are required to maintain compliance with all CUP conditions and terms, including those designed to require the implementation of water conservation practices.

(2) Any restrictions or other measures declared pursuant to Chapter 40E-21, F.A.C., or related Board or Executive Director orders which are more restrictive than a measure contained within this chapter, shall supersede this rule for the duration of the applicable water shortage declaration.

(3) It shall be the duty of each water user to keep informed as to the landscape irrigation measures presented within this chapter, which affect each particular water use.

(4) In addition to the specific measures enumerated below, all wasteful and unnecessary water use as defined in subsection 40E-24.101(10), F.A.C., is prohibited.

(5) The following requirements or exceptions shall apply to all users unless specified otherwise herein:

(a) Landscape irrigation shall be prohibited daily between the hours of 10:00 a.m. and 4:00 p.m., except as otherwise provided herein.

(b) Even addresses as defined in subsection 40E-24.101(4), F.A.C., may accomplish necessary landscape irrigation only on Tuesday, and/or Thursday and/or Sunday.

(c) Odd addresses as defined in subsection 40E-24.101(7), F.A.C., and rights-of-way or other locations without an address may accomplish necessary landscape irrigation only on Monday and/or Wednesday and/or Saturday.

(d) Landscape irrigation systems may be operated during restricted days and/or times for cleaning and maintenance purposes with an attendant on site in the area being tested. Landscape irrigation systems may routinely be operated for such purposes no more than once per week, and the run time for any one test should not exceed 10 minutes.

(e) Landscape irrigation for the purpose of watering in insecticides, fungicides and herbicides, where such watering-in is required by the manufacturer, or by federal, state or local law, shall be allowed under the following conditions:

1. Such watering-in shall be limited to one application in the absence of specific alternative instructions from the manufacturer; and

2. Such watering-in shall be accomplished during normally allowable watering days and times unless a professional licensed applicator has posted a temporary pesticide sign containing the date of application and the date(s) of needed watering-in activity.

(f) Any plant material may be watered using low-volume hand watering methods without regard to the watering days or times allowed pursuant to this section.

Specific Authority 373.044, 373.113, 373.171 FS. Law Implemented 373.042, 373.0421, 373.171, 373.223 FS. History–New 6-12-03.