40E-63.458 Limiting Conditions for the Optional Discharge Monitoring Program.

For those applicants proposing to implement the optional discharge monitoring program, the program will be incorporated by reference into the Individual Permit if the following conditions, (1) through (11), are met. The conditions shall be attached to the Individual Permit:

(1) Those conditions listed under Rule 40E-63.454, F.A.C.;

(2) The approved discharge monitoring plan shall be incorporated by reference and made part of this permit;

(3) Within 30 days of the permit issuance date, the permittee shall contact the District to verify that installation of the monitoring equipment is complete and to schedule an inspection;

(4) The permittee shall implement the discharge monitoring plan in accordance with the permit and shall submit to the District any proposed modification of the plan by submitting an application to modify the permit for review and approval prior to implementation.

(5) The location of sample collection shall be such that water sampled is representative of all water that discharges off site through the structure being monitored.

(6) All water quality sample collection, preservation, handling, transport, and chain-of-custody documentation shall be conducted in accordance with an approved Comprehensive Quality Assurance Plan as specified in the approved discharge monitoring plan. All laboratory analyses shall be conducted by a laboratory with proper certification for the specified parameter (e.g. phosphorus);

(7) In the event that water quality automatic sampling equipment becomes inoperable for any reason, grab samples shall be temporarily taken on a daily basis during flow events and composited for a maximum of 14 days for total phosphorus analysis. Reasonable effort must be made to render the automatic sampling equipment operable within 14 days;

(8) Monitoring conditions may be reduced or adjusted upon submission of data and/or studies that provide the basis for such, reasonably demonstrating that equivalent data will be obtained with the reduction or adjustment in monitoring;

(9) The District will provide at least one week notice to the permittee of the intent to conduct a quality assurance field audit of the sampling collection procedures;

(10) The water quantity and quality data shall be submitted to the District in a timely manner and in a consistent electronic format. Water quantity data shall be submitted to the District in proper electronic format on a monthly basis. Water quality data shall be submitted to the District in accordance with timeframes as specified in Special Limiting Conditions of the permit; and

(11) All flow quantity discharged from the property shall be calculated using a method proposed by a Florida-registered Professional Engineer in a Calibration Report approved by the District. A Calibration Report shall be required for each pump, culvert or other discharge structure. Each Calibration Report shall contain, at a minimum: data collection methodology, instrumentation and procedures; the actual field data collected; the basis for the full operating range represented by the data; the methodology for development of the calibration equation; operational information needed to calculate flow with a temporary backup methodology to be used if the primary equipment becomes inoperable; and the final calibration equation and primary method for calculating the flow. Any modification to the approved calibration shall require an application to modify the existing permit application.

Specific Authority 373.044, 373.083, 373.085, 373.086, 373.113, 373.4592 FS. Law Implemented 373.085, 373.4592 FS. History–New 1-24-02.