- 62-560.400 Scope of Drinking Water Public Notification Rules.

The following sections set forth the requirements that a supplier of water shall meet when public notification is required. In addition to the requirements described in this part, the following requirements and appendices are adopted and incorporated herein by reference:

(1) In the July 1, 2015, edition of 40 C.F.R. §141.202 (http://www.flrules.org/Gateway/reference.asp?No=Ref-07215), the Tier 1 public notice requirements pertaining to violation of the Surface Water Treatment Rule (SWTR), Interim Enhanced Surface Water Treatment Rule (IESWTR), or Long Term 1 Enhanced Surface Water Treatment Rule (LT1ESWTR) treatment technique requirement resulting from a single exceedance of the maximum allowable turbidity limit, where the primacy agency determines after consultation that a Tier 1 notice is required or where consultation does not take place within 24 hours after the system learns of the violation.

(2) In the July 1, 2015, edition of 40 C.F.R. §141.202 (http://www.flrules.org/Gateway/reference.asp?No=Ref-07215), the Tier 1 public notice requirements pertaining to detection of E. coli, enterococci, or coliphage in source water samples required under 40 C.F.R. §§141.402(a) and 141.402(b) (July 1, 2014), which are incorporated by reference in subsection 62-550.828(1), F.A.C.

(3) In the July 1, 2015, edition of 40 C.F.R. §141.203 (http://www.flrules.org/Gateway/reference.asp?No=Ref-07220), the Tier 2 public notice requirements pertaining to failure to take corrective action or failure to maintain at least 4-log treatment of viruses before or at the first customer under 40 C.F.R. §141.403(a) (July 1, 2014), which is incorporated by reference in subsection 62-550.828(1), F.A.C.

(4) In the July 1, 2015, edition of 40 C.F.R. §141.203(b)(3) (http://www.flrules.org/Gateway/reference.asp?No=Ref-07218), the consultation requirements pertaining to violation of the SWTR, IESWTR, or LT1ESWTR treatment technique requirement resulting from a single exceedance of the maximum allowable turbidity limit.

(5) The July 1, 2015, edition of 40 C.F.R. §141.211 (http://www.flrules.org/Gateway/reference.asp?No=Ref-07227).

(6) The July 1, 2015, edition of Appendix A to Subpart Q of 40 C.F.R. Part 141, “National Primary Drinking Water Regulation Violations and Other Situations Requiring Public Notice,” (http://www.flrules.org/Gateway/reference.asp?No=Ref-07229).

(7) The July 1, 2015, edition of Appendix B to Subpart Q of 40 C.F.R. Part 141, “Standard Health Effects Language for Public Notification,” (http://www.flrules.org/Gateway/reference.asp?No=Ref-07230).

(8) The July 1, 2015, edition of Appendix C to Subpart Q of 40 C.F.R. Part 141, “List of Acronyms Used in Public Notification Regulation,” (http://www.flrules.org/Gateway/reference.asp?No=Ref-07231).

Rulemaking Authority 403.8055, 403.861(9) FS. Law Implemented 403.857 FS. History–New 1-18-89, Amended 1-3-91, 1-1-93, Formerly 17-560.400, Amended 9-7-94, 11-27-01, 1-17-05, 12-30-11, 8-5-16.