- 40C-8.031 Minimum Surface Water Levels and Flows and Groundwater Levels.

(1) The following minimum groundwater levels and minimum mean annual spring flows are established:

| - Spring Name | - County | Head (ft NGVD) | Discharge (cfs) |
| --- | --- | --- | --- |
| - Messant Spring | - Lake | 32 | 12 |
| Miami Springs | Seminole | 27 | 4 |
| Palm Springs | Seminole | 27 | 7 |
| Rock Springs | Orange | 31 | 53 |
| Sanlando Springs | Seminole | 28 | 15 |
| Seminole Springs | Lake | 34 | 34 |
| Starbuck Springs | Seminole | 31 | 13 |
| Wekiwa Springs | Orange | 24 | 62 |

(2) The following minimum surface water levels and flows are established:

| System Name | County | Minimum Level | Level (ft NGVD) | Flow (cfs) | Hydroperiod Category | Duration (days) | Return Interval (years) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Black Water Creek at The SR 44 Bridge | Lake | Infrequent High | 27.0 | 340 | – | 7 | 5 |
|  |  | Frequent High | 25.8 | 145 | – | 30 | 2 |
|  |  | Average | 24.3 | 33 | – | 180 | 1.7 |
|  |  | Frequent Low | 22.8 | 2.5 | – | 90 | 15 |
|  |  | Phase 1 Restriction | 22.7 | 2.0 | – | – | – |
|  |  | Phase 2 Restriction | 22.5 | 1.0 | – | – | – |
|  |  | Phase 3 Restriction | 22.4 | 0.6 | – | – | – |
|  |  | Phase 4 Restriction | 22.3 | 0.3 | – | – | – |
|  |  | Infrequent Low | 21.9 | 0.0 | – | 7 | 100 |
| St. Johns River 1.5 miles downstream of Lake Washington weir | Brevard | Frequent High | 15.3 | 1450 | Seasonally Flooded | – | – |
|  |  | Average | 12.7 | 240 | Typically Saturated | – | – |
|  |  | Frequent Low | 11.3 | 28 | Semipermanently Flooded | – | – |
| St. Johns River at SR 44 near DeLand | Volusia | Frequent High | 1.9 | 4600 | – | 30 | 3 |
|  |  | Average | 0.8 | 2050 | – | 180 | 1.5 |
|  |  | Frequent Low | 0.3 | 1100 | – | 120 | 5 |
| Taylor Creek 1.7 miles downstream of structure S-164 |  | Frequent High | – | 95 | Seasonally Flooded | – | – |
|  |  | Average | – | 17 | Typically Saturated | – | – |
|  |  | Frequent Low | – | 0.5 | Semipermanently Flooded | – | – |
| St. Johns River at SR 50 near Christmas | Orange and Brevard | Frequent High | 8.1 | 1950 | – | 30 | 2 |
|  |  | Average | 5.9 | 580 | – | 180 | 1.5 |
|  |  | Frequent Low | 4.2 | 140 | – | 120 | 5 |
|  |  | Infrequent Low | 2.7 | 43 | – | 60 | 50 |
| Wekiva River at the SR 46 Bridge | Seminole and Lake | Infrequent High | 9.0 | 880 | – | 7 | 5 |
|  |  | Frequent High | 8.0 | 410 | – | 30 | 2 |
|  |  | Average | 7.6 | 240 | – | 180 | 1.7 |
|  |  | Frequent Low | 7.2 | 200 | – | 90 | 3 |
|  |  | Phase 1 Restriction | 7.0 | 190 | – | – | – |
|  |  | Phase 2 Restriction | 6.9 | 180 | – | – | – |
|  |  | Phase 3 Restriction | 6.7 | 160 | – | – | – |
|  |  | Phase 4 Restriction | 6.5 | 150 | – | – | – |
|  |  | Infrequent Low | 6.1 | 120 | – | 7 | 100 |

(3) The following minimum surface water flows are established for Blue Spring in Volusia County:

| - Time Period | - Minimum Long Term Mean Flow |
| --- | --- |
| December 3, 2006 through March 31, 2009 | 133 cfs |
| April 1, 2009 through March 31, 2014 | 137 cfs |
| April 1, 2014 through March 31, 2019 | 142 cfs |
| - April 1, 2019 through March 31, 2024 | 148 cfs |
| After March 31, 2024 | 157 cfs |

(4) The following minimum surface water levels are established:

| System Name | County | Minimum Level | Level (ft NGVD) | Level (ft NAVD) | Hydroperiod Category | Duration (days) | Return Interval (years) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (a) Apshawa North | Lake | Frequent High | 85.0 | 83.9 | Seasonally Flooded | – | – |
|  |  | Average | 83.3 | 82.2 | Typically Saturated | – | – |
|  |  | Frequent Low | 81.3 | 80.2 | Semipermanently Flooded | – | – |
| (b) Apshawa South | Lake | Frequent High | 86.0 | 85.7 | Seasonally Flooded | – | – |
|  |  | Average | 84.7 | 83.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 83.2 | 82.1 | Semipermanently Flooded | – | – |
| (c) Argenta | Putnam | Frequent High | 50.1 | 49.0 | Seasonally Flooded | – | – |
|  |  | Average | 47.7 | 46.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 46.3 | 45.3 | Semipermanently Flooded | – | – |
| (d) Ashby | Volusia | Frequent High | 12.3 | 11.3 | – | 60 | 2 |
|  |  | Frequent Low | 11.1 | 10.2 | – | 120 | 5 |
| (e) Banana | Putnam | Frequent High | 37.7 | 36.8 | – | 30 | 3 |
|  |  | Average | 36.7 | 35.8 | – | 180 | 1.7 |
|  |  | Frequent Low | 33.7 | 32.8 | – | 120 | 5 |
| (f) Bell | Putnam | Frequent High | 42.5 | 44.5 | Temporarily Flooded | – | – |
|  |  | Average | 40.5 | 36.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.7 | 37.9 | Semipermanently Flooded | – | – |
| (g) Big | Volusia | Frequent High | 26.1 | 25.1 | Seasonally Flooded | – | – |
|  |  | Average | 25.0 | 24.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 23.7 | 22.7 | Semipermanently Flooded | – | – |
| (h) Bird Pond | Putnam | Frequent High | 41.8 | 40.8 | Seasonally Flooded | – | – |
|  |  | Average | 39.5 | 38.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.1 | 37.1 | Semipermanently Flooded | – | – |
| (i) Blue Pond | Clay | Frequent High | 174.1 | 173.1 | Temporarily Flooded | – | – |
|  |  | Average | 173.3 | 172.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 171.7 | 170.7 | Semipermanently Flooded | – | – |
| (j) Boggy Marsh | Lake | Frequent High | 117.3 | 116.4 | Seasonally Flooded | – | – |
|  |  | Average | 115.9 | 115.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 114.5 | 113.6 | Semipermanently Flooded | – | – |
| (k) Bowers | Marion | Frequent High | 57.1 | 56.1 | Temporarily Flooded | – | – |
|  |  | Average | 54.0 | 53.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 52.7 | 51.7 | Semipermanently Flooded | – | – |
| (l) Brantley | Seminole | Frequent High | 46.3 | 45.3 | Seasonally Flooded | – | – |
|  |  | Average | 45.6 | 44.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 44.1 | 43.1 | Semipermanently Flooded | – | – |
| (m) Brooklyn | Clay | Frequent High | 114.6 | 113.5 | Temporarily Flooded | – | – |
|  |  | Average | 108.0 | 106.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 101.0 | 99.9 | Semipermanently Flooded | – | – |
| (n) Broward | Putnam | Frequent High | 40.0 | 39.1 | Temporarily Flooded | – | – |
|  |  | Average | 38.2 | 35.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.5 | 35.6 | Semipermanently Flooded | – | – |
| (o) Burkett | Orange | Frequent High | 53.5 | 52.5 | Seasonally Flooded | – | – |
|  |  | Average | 52.6 | 51.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.2 | 50.2 | Semipermanently Flooded | – | – |
| (p) Charles | Marion | Frequent High | 40.2 | 39.3 | Seasonally Flooded | – | – |
|  |  | Average | 39.3 | 38.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.9 | 36.9 | Semipermanently Flooded | – | – |
| (q) Cherry | Lake | Frequent High | 96.0 | 94.9 | Seasonally Flooded | – | – |
|  |  | Average | 94.9 | 93.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 93.4 | 92.4 | Semipermanently Flooded | – | – |
| (r) Clear | Putnam | Frequent High | 37.4 | 36.4 | Temporarily Flooded | – | – |
|  |  | Average | 36.4 | 35.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.9 | 34.0 | Semipermanently Flooded | – | – |
| (s) Colby | Volusia | Frequent High | 27.6 | 26.6 | – | 30 | 3 |
|  |  | Frequent Low | 22.9 | 21.8 | – | 120 | 3 |
| (t) Como | Putnam | Frequent High | 37.7 | 36.8 | – | 30 | 3 |
|  |  | Average | 36.7 | 35.8 | – | 180 | 1.7 |
|  |  | Frequent Low | 33.7 | 32.8 | – | 120 | 5 |
| (u) Como, Little Lake | Putnam | Frequent High | 37.7 | 36.8 | – | 30 | 3 |
|  |  | Average | 34.6 | 33.7 | – | 180 | 1.7 |
|  |  | Frequent Low | 32.6 | 31.7 | – | 120 | 5 |
| (v) Coon Pond | Volusia | Frequent High | 35.7 | 34.8 | Seasonally Flooded | – | – |
|  |  | Average | 34.6 | 33.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.1 | 32.2 | Semipermanently Flooded | – | – |
| (w) Cowpen | Putnam | Infrequent High | 92.0 | 90.8 |  | 30 | 25 |
|  |  | Average | 85.2 | 84.0 |  | 180 | 1.7 |
|  |  | Frequent Low | 83.5 | 82.3 |  | 120 | 2.7 |
| (x) Cow Pond | Volusia | Frequent High | 40.5 | 39.5 | Seasonally Flooded | – | – |
|  |  | Average | 39.8 | 38.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.6 | 36.6 | Semipermanently Flooded | – | – |
| (y) Crystal/Baker | Putnam | Frequent High | 35.5 | 34.6 | Seasonally Flooded | – | – |
|  |  | Average | 33.9 | 33.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.0 | 32.1 | Semipermanently Flooded | – | – |
| (z) Daugharty | Volusia | Frequent High | 44.8 | 43.8 | Temporarily Flooded | – | – |
|  |  | Average | 42.6 | 41.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 41.2 | 40.1 | Semipermanently Flooded | – | – |
| (aa) Davis | Volusia | Frequent High | 36.2 | 35.2 | Seasonally Flooded | – | – |
|  |  | Average | 35.4 | 34.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.0 | 33.0 | Semipermanently Flooded | – | – |
| (bb) Deep | Putnam | Frequent High | 35.0 | 34.1 | Seasonally Flooded | – | – |
|  |  | Average | 33.1 | 32.2 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.2 | 31.3 | Semipermanently Flooded | – | – |
| (cc) Dias | Volusia | Frequent High | 34.6 | 33.5 | Seasonally Flooded | – | – |
|  |  | Average | 33.5 | 33.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.2 | 31.6 | Semipermanently Flooded | – | – |
| (dd) Disston | Flagler | Frequent High | 13.8 | 12.8 | Seasonally Flooded | – | – |
|  |  | Average | 13.2 | 12.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 12.5 | 11.5 | Semipermanently Flooded | – | – |
| (ee) Dorr | Lake | Frequent High | 43.5 | 42.5 | Seasonally Flooded | – | – |
|  |  | Average | 43.1 | 42.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 42.1 | 41.0 | Semipermanently Flooded | – | – |
| (ff) Dream Pond | Putnam | Frequent High | 49.0 | 48.0 | Seasonally Flooded | – | – |
|  |  | Average | 47.5 | 46.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 46.0 | 45.1 | Semipermanently Flooded | – | – |
| (gg) Drudy | Volusia | Frequent High | 42.1 | 41.0 | Seasonally Flooded | – | – |
|  |  | Average | 40.6 | 39.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 39.1 | 38.1 | Semipermanently Flooded | – | – |
| (hh) Echo | Putnam | Frequent High | 38.8 | 37.9 | Seasonally Flooded | – | – |
|  |  | Average | 36.7 | 35.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 35.2 | 34.3 | Semipermanently Flooded | – | – |
| (ii) Emma | Lake | Frequent High | 94.1 | 93.0 | Seasonally Flooded | – | – |
|  |  | Average | 92.5 | 91.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 91.1 | 90.0 | Semipermanently Flooded | – | – |
| (jj) Emporia | Volusia | Frequent High | 38.9 | 37.8 | Seasonally Flooded | – | – |
|  |  | Average | 35.8 | 34.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.3 | 31.6 | Semipermanently Flooded | – | – |
| (kk) Estella | Putnam | Frequent High | 38.6 | 37.7 | Seasonally Flooded | – | – |
|  |  | Average | 37.2 | 36.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.5 | 35.5 | Semipermanently Flooded | – | – |
| (ll) Fox | Brevard | Frequent High | 16.7 | 15.4 | Temporarily Flooded | – | – |
|  |  | Average | 15.3 | 14.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 13.8 | 12.6 | Semipermanently Flooded | – | – |
| (mm) Geneva | Clay | Frequent High | 103.0 | 101.9 | Seasonally Flooded | – | – |
|  |  | Average | 101.0 | 99.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 98.5 | 97.4 | Semipermanently Flooded | – | – |
| (nn) Georges Lake | Putnam | Frequent High | 98.4 | 97.4 | Seasonally Flooded | – | – |
|  |  | Average | 97.8 | 96.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 97.0 | 96.1 | Semipermanently Flooded | – | – |
| (oo) Gertie | Volusia | Frequent High | 27.5 | 26.5 | Temporarily Flooded | – | – |
|  |  | Average | 25.6 | 24.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 23.3 | 22.4 | Semipermanently Flooded | – | – |
| (pp) Gore | Flagler | Frequent High | 21.1 | 20.0 | – | 30 | 3 |
|  |  | Average | 20.6 | 19.6 | – | 180 | 1.5 |
|  |  | Frequent Low | 19.2 | 18.2 | – | 120 | 5 |
| (qq) Grandin | Putnam | Frequent High | 81.5 | 80.7 | – | 30 | 2 |
|  |  | Frequent Low | 78.6 | 77.8 | – | 120 | 5 |
| (rr) Halfmoon | Marion | Frequent High | 49.7 | 48.6 | Seasonally Flooded | – | – |
|  |  | Average | 47.9 | 46.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 46.5 | 45.3 | Semipermanently Flooded | – | – |
| (ss) Helen | Volusia | Frequent High | 46.1 | 45.0 | Temporarily Flooded | – | – |
|  |  | Average | 44.2 | 43.2 | Typically Saturated | – | – |
|  |  | Frequent Low | 43.6 | 42.6 | Semipermanently Flooded | – | – |
| (tt) Hires | Volusia | Frequent High | 41.0 | 40.0 | Seasonally Flooded | – | – |
|  |  | Average | 39.5 | 38.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.0 | 37.0 | Semipermanently Flooded | – | – |
| (uu) Hokey | Volusia | Frequent High | 35.4 | 34.4 | Seasonally Flooded | – | – |
|  |  | Average | 33.7 | 32.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.3 | 31.3 | Semipermanently Flooded | – | – |
| (vv) Hopkins Prairie | Marion | Frequent High | 25.8 | 24.5 | Seasonally Flooded | – | – |
|  |  | Average | 23.4 | 22.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 22.0 | 20.7 | Semipermanently Flooded | – | – |
| (ww) Howell | Putnam | Frequent High | 34.5 | 33.5 | Seasonally Flooded | – | – |
|  |  | Average | 33.6 | 32.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 31.8 | 30.9 | Semipermanently Flooded | – | – |
| (xx) Howell | Seminole | Frequent High | 53.7 | 52.7 | Seasonally Flooded |  |  |
|  |  | Average | 52.9 | 51.9 | Typically Saturated |  |  |
|  |  | Frequent Low | 51.5 | 50.5 | Semipermanently Flooded |  |  |
| (yy) Indian | Volusia | Frequent High | 36.2 | 35.2 |  | 30 | 3 |
|  |  | Average | 35.0 | 34.0 |  | 180 | 1.7 |
|  |  | Frequent Low | 32.8 | 31.8 |  | 120 | 5 |
| (zz) Irma | Orange | Frequent High | 55.1 | 54.1 | Seasonally Flooded | – | – |
|  |  | Average | 54.8 | 53.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 53.4 | 52.4 | Semipermanently Flooded | – | – |
| (aaa) Kerr | Marion | Infrequent High | 25.6 | 24.6 |  | 30 | 25 |
|  |  | Frequent High | 23.3 | 22.3 |  | 14 | 2 |
|  |  | Average | 21.7 | 20.7 |  | 180 | 1.7 |
|  |  | Frequent Low | 20.4 | 19.4 |  | 120 | 5 |
| (bbb) Lizzie | Putnam | Frequent High | 43.9 | 43.0 | Seasonally Flooded | – | – |
|  |  | Average | 42.7 | 41.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 41.7 | 40.8 | Semipermanently Flooded | – | – |
| (ccc) Louisa | Lake | Frequent High | 96.5 | 95.6 | Seasonally Flooded | – | – |
|  |  | Average | 95.4 | 94.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 94.0 | 93.0 | Semipermanently Flooded | – | – |
| (ddd) Lower Lake Louise | Volusia | Frequent High | 31.8 | 30.8 | Seasonally Flooded | – | – |
|  |  | Average | 31.2 | 30.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 29.7 | 28.7 | Semipermanently Flooded | – | – |
| (eee) Lucy | Lake | Frequent High | 94.1 | 93.0 | Seasonally Flooded | – | – |
|  |  | Average | 92.5 | 91.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 91.1 | 90.0 | Semipermanently Flooded | – | – |
| (fff) Magnolia | Clay | Frequent High | 124.7 | 123.6 | Seasonally Flooded | – | – |
|  |  | Average | 124.2 | 123.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 121.4 | 120.3 | Semipermanently Flooded | – | – |
| (ggg) Mall, Little Lake | Putnam | Frequent High | 38.7 | 37.7 | Seasonally Flooded | – | – |
|  |  | Average | 36.8 | 35.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 35.2 | 34.2 | Semipermanently Flooded | – | – |
| (hhh) Margaret | Putnam | Frequent High | 35.2 | 34.2 | Seasonally Flooded | – | – |
|  |  | Average | 34.5 | 33.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.5 | 31.5 | Semipermanently Flooded | – | – |
| (iii) Martha | Orange | Frequent High | 53.5 | 52.5 | Seasonally Flooded | – | – |
|  |  | Average | 52.6 | 51.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.2 | 50.2 | Semipermanently Flooded | – | – |
| (jjj) Marvin | Putnam | Frequent High | 38.6 | 37.6 | Seasonally Flooded | – | – |
|  |  | Average | 37.3 | 36.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.3 | 35.3 | Semipermanently Flooded | – | – |
| (kkk) McGrady | Putnam | Frequent High | 41.5 | 40.6 | Seasonally Flooded | – | – |
|  |  | Average | 39.9 | 38.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.8 | 36.8 | Semipermanently Flooded | – | – |
| (lll) McKasel | Putnam | Frequent High | 36.7 | 35.7 | Seasonally Flooded | – | – |
|  |  | Average | 35.5 | 34.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.1 | 33.1 | Semipermanently Flooded | – | – |
| (mmm) Melrose | Putnam | Frequent High | 104.7 | 103.6 |  | 30 | 3 |
|  |  | Average | 104.2 | 103.1 |  | 180 | 1.7 |
|  |  | Frequent Low | 103.7 | 102.6 |  | 120 | 10 |
| (nnn) Mills | Seminole | Frequent High | 42.5 | 41.1 | Seasonally Flooded | – | – |
|  |  | Average | 41.4 | 40.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 39.9 | 38.9 | Semipermanently Flooded | – | – |
| (ooo) Minneola | Lake | Frequent High | 96.0 | 95.1 | Seasonally Flooded | – | – |
|  |  | Average | 95.3 | 94.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 93.9 | 93.0 | Semipermanently Flooded | – | – |
| (ppp) Monroe | Seminole and Volusia | Frequent High | 2.8 | 1.9 | – | 30 | 2 |
|  |  | Average | 1.2 | 0.3 | – | 180 | 1.5 |
|  |  | Frequent Low | 0.5 | -0.4 | – | 120 | 5 |
| (qqq) Nettles/English | Putnam | Frequent High | 44.3 | 43.3 | Seasonally Flooded | – | – |
|  |  | Average | 42.7 | 41.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 41.7 | 40.7 | Semipermanently Flooded | – | – |
| (rrr) Nicotoon | Marion | Frequent High | 54.7 | 53.7 | Seasonally Flooded | – | – |
|  |  | Average | 53.3 | 52.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.9 | 50.9 | Semipermanently Flooded | – | – |
| (sss) Norris | Lake | Frequent High | 29.7 | 28.7 |  | 30 | 1.7 |
|  |  | Frequent Low | 27.7 | 26.7 |  | 120 | 7.6 |
|  |  |  |  |  |  | – | – |
| (ttt) North Como Park | Putnam | Frequent High | 41.3 | 40.4 | Seasonally Flooded | – | – |
|  |  | Average | 39.7 | 38.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.5 | 37.6 | Semipermanently Flooded | – | – |
| (uuu) North Talmadge | Volusia | Frequent High | 55.6 | 54.5 | Seasonally Flooded | – | – |
|  |  | Average | 54.4 | 53.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 52.9 | 51.9 | Semipermanently Flooded | – | – |
| (vvv) Omega | Putnam | Frequent High | 57.4 | 56.3 | Temporarily Flooded | – | – |
|  |  | Average | 56.1 | 55.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 54.0 | 52.9 | Semipermanently Flooded | – | – |
| (www) Orio | Putnam | Frequent High | 37.1 | 35.5 | Seasonally Flooded | – | – |
|  |  | Average | 35.6 | 34.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.7 | 33.7 | Semipermanently Flooded | – | – |
| (xxx) Pam | Putnam | Frequent High | 39.3 | 38.2 | Seasonally Flooded | – | – |
|  |  | Average | 37.5 | 36.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.1 | 35.0 | Semipermanently Flooded | – | – |
| (yyy) Pearl | Orange | Frequent High | 53.5 | 52.5 | Seasonally Flooded | – | – |
|  |  | Average | 52.6 | 51.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.2 | 50.2 | Semipermanently Flooded | – | – |
| (zzz) Pierson | Volusia | Frequent High | 34.4 | 33.3 | Seasonally Flooded | – | – |
|  |  | Average | 33.8 | 32.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.4 | 31.3 | Semipermanently Flooded | – | – |
| (aaaa) Pine Island | Lake | Frequent High | 107.7 | 106.6 | Seasonally Flooded | – | – |
|  |  | Average | 106.8 | 105.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 105.4 | 104.3 | Semipermanently Flooded | – | – |
| (bbbb) Prevatt | Orange | Frequent High | 56.0 | 54.9 | Seasonally Flooded | – | – |
|  |  | Average | 53.0 | 52.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 50.9 | 49.9 | Semipermanently Flooded | – | – |
| (cccc) Prior | Putnam | Frequent High | 42.3 | 41.3 | Seasonally Flooded | – | – |
|  |  | Average | 40.0 | 39.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 39.0 | 38.0 | Semipermanently Flooded | – | – |
| (dddd) Purdom | Volusia | Frequent High | 36.3 | 35.3 |  | 30 | 2 |
|  |  | Average | 35.4 | 34.3 |  | 180 | 1.7 |
|  |  | Frequent Low | 34.4 | 33.3 |  | 120 | 5 |
| (eeee) Sand | Putnam | Frequent High | 40.9 | 39.4 | Seasonally Flooded | – | – |
|  |  | Average | 39.0 | 38.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.6 | 35.7 | Semipermanently Flooded | – | – |
| (ffff) Sand Hill | Clay | Frequent High | 132.0 | 130.9 | Seasonally Flooded | – | – |
|  |  | Average | 131.6 | 130.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 129.5 | 128.4 | Semipermanently Flooded | – | – |
| (gggg) Savannah | Volusia | Frequent High | 31.1 | 30.2 | Seasonally Flooded | – | – |
|  |  | Average | 29.5 | 28.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 28.0 | 27.0 | Semipermanently Flooded | – | – |
| (hhhh) Scoggin | Volusia | Frequent High | 35.0 | 34.0 | Seasonally Flooded | – | – |
|  |  | Average | 34.1 | 33.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.7 | 31.7 | Semipermanently Flooded | – | – |
| (iiii) Shaw | Volusia | Frequent High | 36.7 | 35.7 | – | 30 | 3 |
|  |  | Average | 35.4 | 34.4 | – | 180 | 1.7 |
|  |  | Frequent Low | 33.7 | 32.7 | – | 120 | 3 |
| (jjjj) Silver | Putnam | Frequent High | 36.8 | 35.8 | Seasonally Flooded | – | – |
|  |  | Average | 35.1 | 34.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.7 | 32.7 | Semipermanently Flooded | – | – |
| (kkkk) Smith | Marion | Frequent High | 54.6 | 53.6 | Temporarily Flooded | – | – |
|  |  | Average | 51.4 | 50.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 50.0 | 49.0 | Semipermanently Flooded | – | – |
| (llll) South | Brevard | Frequent High | 16.7 | 15.5 | Temporarily Flooded | – | – |
|  |  | Average | 15.3 | 14.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 13.8 | 12.6 | Semipermanently Flooded | – | – |
| (mmmm) South Como Park | Putnam | Frequent High | 38.1 | 37.2 | Seasonally Flooded | – | – |
|  |  | Average | 36.7 | 35.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 35.3 | 34.4 | Semipermanently Flooded | – | – |
| (nnnn) Star | Putnam | Frequent High | 77.5 | 76.5 | Seasonally Flooded | – | – |
|  |  | Average | 75.4 | 74.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 74.0 | 73.0 | Semipermanently Flooded | – | – |
| (oooo) Stella | Putnam | Frequent High | 39.4 | 38.4 | Seasonally Flooded | – | – |
|  |  | Average | 38.6 | 37.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.2 | 36.1 | Semipermanently Flooded | – | – |
| (pppp) Sunset | Lake | Frequent High | 85.9 | 84.9 | Temporarily Flooded | – | – |
|  |  | Average | 83.5 | 82.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 81.0 | 79.9 | Semipermanently Flooded | – | – |
| (qqqq) Swan | Putnam | Frequent High | 93.0 | 91.9 | Temporarily Flooded | – | – |
|  |  | Average | 90.3 | 89.2 | Typically Saturated | – | – |
| (rrrr) Sylvan | Seminole | Frequent High | 40.4 | 39.5 | Seasonally Flooded | – | – |
|  |  | Average | 38.9 | 38.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.5 | 36.5 | Semipermanently Flooded | – | – |
| (ssss) Tarhoe | Putnam | Frequent High | 36.6 | 35.7 |  | 30 | 3 |
|  |  | Average | 35.1 | 34.2 |  | 180 | 1.7 |
|  |  | Frequent Low | 33.0 | 32.0 |  | 120 | 5 |
| (tttt) Three Island Lakes | Volusia | Frequent High | 23.7 | 22.7 | – | 30 | 5 |
|  |  | Frequent Low | 19.4 | 18.4 | – | 120 | 10 |
| (uuuu) Trone | Putnam | Frequent High | 37.1 | 36.2 | – | 30 | 3 |
|  |  | Average | 35.4 | 34.5 | – | 180 | 1.7 |
|  |  | Frequent Low | 32.8 | 31.9 | – | 120 | 5 |
| (vvvv) Trout | Volusia | Frequent High | 23.3 | 22.3 | Seasonally Flooded | – | – |
|  |  | Average | 20.9 | 20.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 17.7 | 16.8 | Semipermanently Flooded | – | – |
| (wwww) Tuscawilla | Alachua | Frequent High | 77.6 | 76.4 | Seasonally Flooded | – | – |
|  |  | Average | 74.6 | 73.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 73.2 | 72.0 | Semipermanently Flooded | – | – |
| (xxxx) Upper Lake Louise | Volusia | Frequent High | 35.3 | 34.4 | Seasonally Flooded | – | – |
|  |  | Average | 34.6 | 33.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.2 | 32.2 | Semipermanently Flooded | – | – |
| (yyyy) Washington | Brevard | Frequent High | 15.6 | 14.2 | Seasonally Flooded | – | – |
|  |  | Average | 14.2 | 12.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 12.8 | 11.4 | Semipermanently Flooded | – | – |
| (zzzz) Wauberg | Alachua | Frequent High | 67.4 | 66.4 | Seasonally Flooded | – | – |
|  |  | Average | 67.1 | 66.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 65.6 | 64.7 | Semipermanently Flooded | – | – |
| (aaaaa) Weir | Marion | Frequent High | 57.2 | 56.2 | Seasonally Flooded | – | – |
|  |  | Average | 56.4 | 55.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 54.9 | 53.9 | Semipermanently Flooded | – | – |
| (bbbbb) Winnemisett | Volusia | Frequent High | 59.5 | 58.5 | Seasonally Flooded | – | – |
|  |  | Average | 57.8 | 56.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 56.0 | 55.0 | Semipermanently Flooded | – | – |
| (ccccc) Winona | Volusia | Frequent High | 36.1 | 35.1 | Seasonally Flooded | – | – |
|  |  | Average | 33.5 | 32.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.0 | 31.0 | Semipermanently Flooded | – | – |

(5) The following minimum levels are established for Blue Cypress Water Management Area (BCWMA):

(a) The minimum average level, calculated as the long term mean of BCWMA water levels, is 24.0 feet NGVD (22.5 feet NAVD). Water levels shall be at or above this level at least 75% of time over the long term.

(b) The minimum frequent low is 23.0 feet NGVD (21.5 feet NAVD). The daily BCWMA water level shall not fall to this level or below more often than once every 2.5 years over the long term.

(c) The minimum infrequent low is 22.5 feet NGVD (21.0 feet NAVD). The BCWMA water level shall not fall to this level or below for 60 continuous days more frequently than once every 10 years over the long term.

(6) Ground or surface water withdrawals or surface water works must not cause the infrequent high or frequent high surface water flows and levels to occur less frequently or for at lesser duration than stated. Ground or surface water withdrawals or surface water works must not cause the minimum average, frequent low, or infrequent low surface water levels and flows to occur more frequently or for longer durations than stated.

Rulemaking Authority 373.044, 373.113 FS. Law Implemented 373.042, 373.0421, 373.103, 373.415 FS. History–New 9-16-92, Amended 8-17-94, 6-8-95, 1-17-96, 8-20-96, 10-20-96, 11-4-98, 6-27-00, 2-13-01, 3-19-02, 5-12-03, 11-10-03, 1-12-04, 2-1-06, 12-3-06, 5-10-07, 5-24-07, 1-11-10, 8-22-13, 4-3-14, 11-25-14, 12-31-14, 1-31-16 (4)(aaa), 1-31-16 (4)(ssss), 2-1-17 (4)(w).