- 40C-8.031 Minimum Surface Water Levels and Flows and Groundwater Levels.

(1) The following minimum groundwater levels and minimum mean annual spring flows are established:

| - Spring Name | - County | Head (ft NGVD) | Discharge (cfs) |
| --- | --- | --- | --- |
| - Messant Spring | - Lake | 32 | 12 |
| Miami Springs | Seminole | 27 | 4 |
| Palm Springs | Seminole | 27 | 7 |
| Rock Springs | Orange | 31 | 53 |
| Sanlando Springs | Seminole | 28 | 15 |
| Seminole Springs | Lake | 34 | 34 |
| Starbuck Springs | Seminole | 31 | 13 |
| Wekiwa Springs | Orange | 24 | 62 |

(2) The following minimum surface water levels and flows are established:

| System Name | County | Minimum Level | Level (ft NGVD) | Flow (cfs) | Hydroperiod Category | Duration (days) | Return Interval (years) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Black Water Creek at The SR 44 Bridge | Lake | Infrequent High | 27.0 | 340 | – | 7 | 5 |
|  |  | Frequent High | 25.8 | 145 | – | 30 | 2 |
|  |  | Average | 24.3 | 33 | – | 180 | 1.7 |
|  |  | Frequent Low | 22.8 | 2.5 | – | 90 | 15 |
|  |  | Phase 1 Restriction | 22.7 | 2.0 | – | – | – |
|  |  | Phase 2 Restriction | 22.5 | 1.0 | – | – | – |
|  |  | Phase 3 Restriction | 22.4 | 0.6 | – | – | – |
|  |  | Phase 4 Restriction | 22.3 | 0.3 | – | – | – |
|  |  | Infrequent Low | 21.9 | 0.0 | – | 7 | 100 |
| St. Johns River 1.5 miles downstream of Lake Washington weir | Brevard | Frequent High | 15.3 | 1450 | Seasonally Flooded | – | – |
|  |  | Average | 12.7 | 240 | Typically Saturated | – | – |
|  |  | Frequent Low | 11.3 | 28 | Semipermanently Flooded | – | – |
| St. Johns River at SR 44 near DeLand | Volusia | Frequent High | 1.9 | 4600 | – | 30 | 3 |
|  |  | Average | 0.8 | 2050 | – | 180 | 1.5 |
|  |  | Frequent Low | 0.3 | 1100 | – | 120 | 5 |
| Taylor Creek 1.7 miles downstream of structure S-164 |  | Frequent High | – | 95 | Seasonally Flooded | – | – |
|  |  | Average | – | 17 | Typically Saturated | – | – |
|  |  | Frequent Low | – | 0.5 | Semipermanently Flooded | – | – |
| St. Johns River at SR 50 near Christmas | Orange and Brevard | Frequent High | 8.1 | 1950 | – | 30 | 2 |
|  |  | Average | 5.9 | 580 | – | 180 | 1.5 |
|  |  | Frequent Low | 4.2 | 140 | – | 120 | 5 |
|  |  | Infrequent Low | 2.7 | 43 | – | 60 | 50 |
| Wekiva River at the SR 46 Bridge | Seminole and Lake | Infrequent High | 9.0 | 880 | – | 7 | 5 |
|  |  | Frequent High | 8.0 | 410 | – | 30 | 2 |
|  |  | Average | 7.6 | 240 | – | 180 | 1.7 |
|  |  | Frequent Low | 7.2 | 200 | – | 90 | 3 |
|  |  | Phase 1 Restriction | 7.0 | 190 | – | – | – |
|  |  | Phase 2 Restriction | 6.9 | 180 | – | – | – |
|  |  | Phase 3 Restriction | 6.7 | 160 | – | – | – |
|  |  | Phase 4 Restriction | 6.5 | 150 | – | – | – |
|  |  | Infrequent Low | 6.1 | 120 | – | 7 | 100 |

(3) The following minimum surface water flows are established for Blue Spring in Volusia County:

| - Time Period | - Minimum Long Term Mean Flow |
| --- | --- |
| December 3, 2006 through March 31, 2009 | 133 cfs |
| April 1, 2009 through March 31, 2014 | 137 cfs |
| April 1, 2014 through March 31, 2019 | 142 cfs |
| - April 1, 2019 through March 31, 2024 | 148 cfs |
| After March 31, 2024 | 157 cfs |

(4) The following minimum surface water levels are established:

| System Name | County | Minimum Level | Level (ft NGVD) | Level (ft NAVD) | Hydroperiod Category | Duration (days) | Return Interval (years) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (a) Apshawa North | Lake | Frequent High | 85.0 | 83.9 | Seasonally Flooded | – | – |
|  |  | Average | 83.3 | 82.2 | Typically Saturated | – | – |
|  |  | Frequent Low | 81.3 | 80.2 | Semipermanently Flooded | – | – |
| (b) Apshawa South | Lake | Frequent High | 86.0 | 85.7 | Seasonally Flooded | – | – |
|  |  | Average | 84.7 | 83.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 83.2 | 82.1 | Semipermanently Flooded | – | – |
| (c) Argenta | Putnam | Frequent High | 50.1 | 49.0 | Seasonally Flooded | – | – |
|  |  | Average | 47.7 | 46.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 46.3 | 45.3 | Semipermanently Flooded | – | – |
| (d) Ashby | Volusia | Frequent High | 12.3 | 11.3 | – | 60 | 2 |
|  |  | Frequent Low | 11.1 | 10.2 | – | 120 | 5 |
| (e) Banana | Putnam | Frequent High | 37.7 | 36.8 | – | 30 | 3 |
|  |  | Average | 36.7 | 35.8 | – | 180 | 1.7 |
|  |  | Frequent Low | 33.7 | 32.8 | – | 120 | 5 |
| (f) Bell | Putnam | Frequent High | 42.5 | 44.5 | Temporarily Flooded | – | – |
|  |  | Average | 40.5 | 36.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.7 | 37.9 | Semipermanently Flooded | – | – |
| (g) Big | Volusia | Frequent High | 26.1 | 25.1 | Seasonally Flooded | – | – |
|  |  | Average | 25.0 | 24.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 23.7 | 22.7 | Semipermanently Flooded | – | – |
| (h) Bird Pond | Putnam | Frequent High | 41.8 | 40.8 | Seasonally Flooded | – | – |
|  |  | Average | 39.5 | 38.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.1 | 37.1 | Semipermanently Flooded | – | – |
| (i) Blue Pond | Clay | Frequent High | 174.1 | 173.1 | Temporarily Flooded | – | – |
|  |  | Average | 173.3 | 172.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 171.7 | 170.7 | Semipermanently Flooded | – | – |
| (j) Boggy Marsh | Lake | Frequent High | 117.3 | 116.4 | Seasonally Flooded | – | – |
|  |  | Average | 115.9 | 115.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 114.5 | 113.6 | Semipermanently Flooded | – | – |
| (k) Bowers | Marion | Frequent High | 57.1 | 56.1 | Temporarily Flooded | – | – |
|  |  | Average | 54.0 | 53.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 52.7 | 51.7 | Semipermanently Flooded | – | – |
| (l) Brantley | Seminole | Frequent High | 46.3 | 45.3 | Seasonally Flooded | – | – |
|  |  | Average | 45.6 | 44.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 44.1 | 43.1 | Semipermanently Flooded | – | – |
| (m) Brooklyn | Clay | Frequent High | 114.6 | 113.5 | Temporarily Flooded | – | – |
|  |  | Average | 108.0 | 106.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 101.0 | 99.9 | Semipermanently Flooded | – | – |
| (n) Broward | Putnam | Frequent High | 40.0 | 39.1 | Temporarily Flooded | – | – |
|  |  | Average | 38.2 | 35.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.5 | 35.6 | Semipermanently Flooded | – | – |
| (o) Burkett | Orange | Frequent High | 53.5 | 52.5 | Seasonally Flooded | – | – |
|  |  | Average | 52.6 | 51.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.2 | 50.2 | Semipermanently Flooded | – | – |
| (p) Charles | Marion | Frequent High | 40.2 | 39.3 | Seasonally Flooded | – | – |
|  |  | Average | 39.3 | 38.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.9 | 36.9 | Semipermanently Flooded | – | – |
| (q) Cherry | Lake | Frequent High | 96.0 | 94.9 | Seasonally Flooded | – | – |
|  |  | Average | 94.9 | 93.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 93.4 | 92.4 | Semipermanently Flooded | – | – |
| (r) Clear | Putnam | Frequent High | 37.4 | 36.4 | Temporarily Flooded | – | – |
|  |  | Average | 36.4 | 35.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.9 | 34.0 | Semipermanently Flooded | – | – |
| (s) Colby | Volusia | Frequent High | 27.6 | 26.6 | – | 30 | 3 |
|  |  | Frequent Low | 22.9 | 21.8 | – | 120 | 3 |
| (t) Como | Putnam | Frequent High | 37.7 | 36.8 | – | 30 | 3 |
|  |  | Average | 36.7 | 35.8 | – | 180 | 1.7 |
|  |  | Frequent Low | 33.7 | 32.8 | – | 120 | 5 |
| (u) Como, Little Lake | Putnam | Frequent High | 37.7 | 36.8 | – | 30 | 3 |
|  |  | Average | 34.6 | 33.7 | – | 180 | 1.7 |
|  |  | Frequent Low | 32.6 | 31.7 | – | 120 | 5 |
| (v) Coon Pond | Volusia | Frequent High | 35.7 | 34.8 | Seasonally Flooded | – | – |
|  |  | Average | 34.6 | 33.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.1 | 32.2 | Semipermanently Flooded | – | – |
| (w) Cowpen | Putnam | Infrequent High | 92.0 | 90.8 |  | 30 | 25 |
|  |  | Average | 85.2 | 84.0 |  | 180 | 1.7 |
|  |  | Frequent Low | 83.5 | 82.3 |  | 120 | 2.7 |
| (x) Cow Pond | Volusia | Frequent High | 40.5 | 39.5 | Seasonally Flooded | – | – |
|  |  | Average | 39.8 | 38.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.6 | 36.6 | Semipermanently Flooded | – | – |
| (y) Crystal/Baker | Putnam | Frequent High | 35.5 | 34.6 | Seasonally Flooded | – | – |
|  |  | Average | 33.9 | 33.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.0 | 32.1 | Semipermanently Flooded | – | – |
| (z) Daugharty | Volusia | Frequent High | 44.8 | 43.8 | Temporarily Flooded | – | – |
|  |  | Average | 42.6 | 41.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 41.2 | 40.1 | Semipermanently Flooded | – | – |
| (aa) Davis | Volusia | Frequent High | 36.2 | 35.2 | Seasonally Flooded | – | – |
|  |  | Average | 35.4 | 34.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.0 | 33.0 | Semipermanently Flooded | – | – |
| (bb) Deep | Putnam | Frequent High | 35.0 | 34.1 | Seasonally Flooded | – | – |
|  |  | Average | 33.1 | 32.2 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.2 | 31.3 | Semipermanently Flooded | – | – |
| (cc) Dias | Volusia | Frequent High | 34.6 | 33.5 | Seasonally Flooded | – | – |
|  |  | Average | 33.5 | 33.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.2 | 31.6 | Semipermanently Flooded | – | – |
| (dd) Disston | Flagler | Frequent High | 13.8 | 12.8 | Seasonally Flooded | – | – |
|  |  | Average | 13.2 | 12.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 12.5 | 11.5 | Semipermanently Flooded | – | – |
| (ee) Dorr | Lake | Frequent High | 43.5 | 42.5 | Seasonally Flooded | – | – |
|  |  | Average | 43.1 | 42.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 42.1 | 41.0 | Semipermanently Flooded | – | – |
| (ff) Dream Pond | Putnam | Frequent High | 49.0 | 48.0 | Seasonally Flooded | – | – |
|  |  | Average | 47.5 | 46.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 46.0 | 45.1 | Semipermanently Flooded | – | – |
| (gg) Drudy | Volusia | Frequent High | 42.1 | 41.0 | Seasonally Flooded | – | – |
|  |  | Average | 40.6 | 39.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 39.1 | 38.1 | Semipermanently Flooded | – | – |
| (hh) Echo | Putnam | Frequent High | 38.8 | 37.9 | Seasonally Flooded | – | – |
|  |  | Average | 36.7 | 35.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 35.2 | 34.3 | Semipermanently Flooded | – | – |
| (ii) Emma | Lake | Frequent High | 94.1 | 93.0 | Seasonally Flooded | – | – |
|  |  | Average | 92.5 | 91.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 91.1 | 90.0 | Semipermanently Flooded | – | – |
| (jj) Emporia | Volusia | Frequent High | 38.9 | 37.8 | Seasonally Flooded | – | – |
|  |  | Average | 35.8 | 34.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.3 | 31.6 | Semipermanently Flooded | – | – |
| (kk) Estella | Putnam | Frequent High | 38.6 | 37.7 | Seasonally Flooded | – | – |
|  |  | Average | 37.2 | 36.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.5 | 35.5 | Semipermanently Flooded | – | – |
| (ll) Fox | Brevard | Frequent High | 16.7 | 15.4 | Temporarily Flooded | – | – |
|  |  | Average | 15.3 | 14.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 13.8 | 12.6 | Semipermanently Flooded | – | – |
| (mm) Geneva | Clay | Frequent High | 103.0 | 101.9 | Seasonally Flooded | – | – |
|  |  | Average | 101.0 | 99.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 98.5 | 97.4 | Semipermanently Flooded | – | – |
| (nn) Georges Lake | Putnam | Frequent High | 98.4 | 97.4 | Seasonally Flooded | – | – |
|  |  | Average | 97.8 | 96.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 97.0 | 96.1 | Semipermanently Flooded | – | – |
| (oo) Gertie | Volusia | Frequent High | 27.5 | 26.5 | Temporarily Flooded | – | – |
|  |  | Average | 25.6 | 24.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 23.3 | 22.4 | Semipermanently Flooded | – | – |
| (pp) Gore | Flagler | Frequent High | 21.1 | 20.0 | – | 30 | 3 |
|  |  | Average | 20.6 | 19.6 | – | 180 | 1.5 |
|  |  | Frequent Low | 19.2 | 18.2 | – | 120 | 5 |
| (qq) Grandin | Putnam | Frequent High | 81.5 | 80.7 | – | 30 | 2 |
|  |  | Frequent Low | 78.6 | 77.8 | – | 120 | 5 |
| (rr) Halfmoon | Marion | Frequent High | 49.7 | 48.6 | Seasonally Flooded | – | – |
|  |  | Average | 47.9 | 46.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 46.5 | 45.3 | Semipermanently Flooded | – | – |
| (ss) Helen | Volusia | Frequent High | 46.1 | 45.0 | Temporarily Flooded | – | – |
|  |  | Average | 44.2 | 43.2 | Typically Saturated | – | – |
|  |  | Frequent Low | 43.6 | 42.6 | Semipermanently Flooded | – | – |
| (tt) Hires | Volusia | Frequent High | 41.0 | 40.0 | Seasonally Flooded | – | – |
|  |  | Average | 39.5 | 38.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.0 | 37.0 | Semipermanently Flooded | – | – |
| (uu) Hokey | Volusia | Frequent High | 35.4 | 34.4 | Seasonally Flooded | – | – |
|  |  | Average | 33.7 | 32.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.3 | 31.3 | Semipermanently Flooded | – | – |
| (vv) Hopkins Prairie | Marion | Frequent High | 25.8 | 24.5 | Seasonally Flooded | – | – |
|  |  | Average | 23.4 | 22.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 22.0 | 20.7 | Semipermanently Flooded | – | – |
| (ww) Howell | Putnam | Frequent High | 34.5 | 33.5 | Seasonally Flooded | – | – |
|  |  | Average | 33.6 | 32.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 31.8 | 30.9 | Semipermanently Flooded | – | – |
| (xx) Howell | Seminole | Frequent High | 53.7 | 52.7 | Seasonally Flooded |  |  |
|  |  | Average | 52.9 | 51.9 | Typically Saturated |  |  |
|  |  | Frequent Low | 51.5 | 50.5 | Semipermanently Flooded |  |  |
| (yy) Indian | Volusia | Frequent High | 36.2 | 35.2 |  | 30 | 3 |
|  |  | Average | 35.0 | 34.0 |  | 180 | 1.7 |
|  |  | Frequent Low | 32.8 | 31.8 |  | 120 | 5 |
| (zz) Irma | Orange | Frequent High | 55.1 | 54.1 | Seasonally Flooded | – | – |
|  |  | Average | 54.8 | 53.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 53.4 | 52.4 | Semipermanently Flooded | – | – |
| (aaa) Kerr | Marion | Infrequent High | 25.6 | 24.6 |  | 30 | 25 |
|  |  | Frequent High | 23.3 | 22.3 |  | 14 | 2 |
|  |  | Average | 21.7 | 20.7 |  | 180 | 1.7 |
|  |  | Frequent Low | 20.4 | 19.4 |  | 120 | 5 |
| (bbb) Lizzie | Putnam | Frequent High | 43.9 | 43.0 | Seasonally Flooded | – | – |
|  |  | Average | 42.7 | 41.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 41.7 | 40.8 | Semipermanently Flooded | – | – |
| (ccc) Louisa | Lake | Frequent High | 96.5 | 95.6 | Seasonally Flooded | – | – |
|  |  | Average | 95.4 | 94.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 94.0 | 93.0 | Semipermanently Flooded | – | – |
| (ddd) Lower Lake Louise | Volusia | Frequent High | 31.8 | 30.8 | Seasonally Flooded | – | – |
|  |  | Average | 31.2 | 30.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 29.7 | 28.7 | Semipermanently Flooded | – | – |
| (eee) Lucy | Lake | Frequent High | 94.1 | 93.0 | Seasonally Flooded | – | – |
|  |  | Average | 92.5 | 91.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 91.1 | 90.0 | Semipermanently Flooded | – | – |
| (fff) Magnolia | Clay | Frequent High | 124.7 | 123.6 | Seasonally Flooded | – | – |
|  |  | Average | 124.2 | 123.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 121.4 | 120.3 | Semipermanently Flooded | – | – |
| (ggg) Mall, Little Lake | Putnam | Frequent High | 38.7 | 37.7 | Seasonally Flooded | – | – |
|  |  | Average | 36.8 | 35.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 35.2 | 34.2 | Semipermanently Flooded | – | – |
| (hhh) Margaret | Putnam | Frequent High | 35.2 | 34.2 | Seasonally Flooded | – | – |
|  |  | Average | 34.5 | 33.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.5 | 31.5 | Semipermanently Flooded | – | – |
| (iii) Martha | Orange | Frequent High | 53.5 | 52.5 | Seasonally Flooded | – | – |
|  |  | Average | 52.6 | 51.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.2 | 50.2 | Semipermanently Flooded | – | – |
| (jjj) Marvin | Putnam | Frequent High | 38.6 | 37.6 | Seasonally Flooded | – | – |
|  |  | Average | 37.3 | 36.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.3 | 35.3 | Semipermanently Flooded | – | – |
| (kkk) McGrady | Putnam | Frequent High | 41.5 | 40.6 | Seasonally Flooded | – | – |
|  |  | Average | 39.9 | 38.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.8 | 36.8 | Semipermanently Flooded | – | – |
| (lll) McKasel | Putnam | Frequent High | 36.7 | 35.7 | Seasonally Flooded | – | – |
|  |  | Average | 35.5 | 34.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.1 | 33.1 | Semipermanently Flooded | – | – |
| (mmm) Melrose | Putnam | Frequent High | 104.7 | 103.6 |  | 30 | 3 |
|  |  | Average | 104.2 | 103.1 |  | 180 | 1.7 |
|  |  | Frequent Low | 103.7 | 102.6 |  | 120 | 10 |
| (nnn) Mills | Seminole | Frequent High | 42.5 | 41.1 | Seasonally Flooded | – | – |
|  |  | Average | 41.4 | 40.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 39.9 | 38.9 | Semipermanently Flooded | – | – |
| (ooo) Minneola | Lake | Frequent High | 96.0 | 95.1 | Seasonally Flooded | – | – |
|  |  | Average | 95.3 | 94.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 93.9 | 93.0 | Semipermanently Flooded | – | – |
| (ppp) Monroe | Seminole and Volusia | Frequent High | 2.8 | 1.9 | – | 30 | 2 |
|  |  | Average | 1.2 | 0.3 | – | 180 | 1.5 |
|  |  | Frequent Low | 0.5 | -0.4 | – | 120 | 5 |
| (qqq) Nettles/English | Putnam | Frequent High | 44.3 | 43.3 | Seasonally Flooded | – | – |
|  |  | Average | 42.7 | 41.9 | Typically Saturated | – | – |
|  |  | Frequent Low | 41.7 | 40.7 | Semipermanently Flooded | – | – |
| (rrr) Nicotoon | Marion | Frequent High | 54.7 | 53.7 | Seasonally Flooded | – | – |
|  |  | Average | 53.3 | 52.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.9 | 50.9 | Semipermanently Flooded | – | – |
| (sss) Norris | Lake | Frequent High | 29.7 | 28.7 |  | 30 | 1.7 |
|  |  | Frequent Low | 27.7 | 26.7 |  | 120 | 7.6 |
|  |  |  |  |  |  | – | – |
| (ttt) North Como Park | Putnam | Frequent High | 41.3 | 40.4 | Seasonally Flooded | – | – |
|  |  | Average | 39.7 | 38.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 38.5 | 37.6 | Semipermanently Flooded | – | – |
| (uuu) North Talmadge | Volusia | Frequent High | 55.6 | 54.5 | Seasonally Flooded | – | – |
|  |  | Average | 54.4 | 53.3 | Typically Saturated | – | – |
|  |  | Frequent Low | 52.9 | 51.9 | Semipermanently Flooded | – | – |
| (vvv) Omega | Putnam | Frequent High | 57.4 | 56.3 | Temporarily Flooded | – | – |
|  |  | Average | 56.1 | 55.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 54.0 | 52.9 | Semipermanently Flooded | – | – |
| (www) Orio | Putnam | Frequent High | 37.1 | 35.5 | Seasonally Flooded | – | – |
|  |  | Average | 35.6 | 34.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 34.7 | 33.7 | Semipermanently Flooded | – | – |
| (xxx) Pam | Putnam | Frequent High | 39.3 | 38.2 | Seasonally Flooded | – | – |
|  |  | Average | 37.5 | 36.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.1 | 35.0 | Semipermanently Flooded | – | – |
| (yyy) Pearl | Orange | Frequent High | 53.5 | 52.5 | Seasonally Flooded | – | – |
|  |  | Average | 52.6 | 51.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 51.2 | 50.2 | Semipermanently Flooded | – | – |
| (zzz) Pierson | Volusia | Frequent High | 34.4 | 33.3 | Seasonally Flooded | – | – |
|  |  | Average | 33.8 | 32.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.4 | 31.3 | Semipermanently Flooded | – | – |
| (aaaa) Pine Island | Lake | Frequent High | 107.7 | 106.6 | Seasonally Flooded | – | – |
|  |  | Average | 106.8 | 105.7 | Typically Saturated | – | – |
|  |  | Frequent Low | 105.4 | 104.3 | Semipermanently Flooded | – | – |
| (bbbb) Prevatt | Orange | Frequent High | 56.0 | 54.9 | Seasonally Flooded | – | – |
|  |  | Average | 53.0 | 52.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 50.9 | 49.9 | Semipermanently Flooded | – | – |
| (cccc) Prior | Putnam | Frequent High | 42.3 | 41.3 | Seasonally Flooded | – | – |
|  |  | Average | 40.0 | 39.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 39.0 | 38.0 | Semipermanently Flooded | – | – |
| (dddd) Purdom | Volusia | Frequent High | 36.3 | 35.3 |  | 30 | 2 |
|  |  | Average | 35.4 | 34.3 |  | 180 | 1.7 |
|  |  | Frequent Low | 34.4 | 33.3 |  | 120 | 5 |
| (eeee) Sand | Putnam | Frequent High | 40.9 | 39.4 | Seasonally Flooded | – | – |
|  |  | Average | 39.0 | 38.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 36.6 | 35.7 | Semipermanently Flooded | – | – |
| (ffff) Sand Hill | Clay | Frequent High | 132.0 | 130.9 | Seasonally Flooded | – | – |
|  |  | Average | 131.6 | 130.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 129.5 | 128.4 | Semipermanently Flooded | – | – |
| (gggg) Savannah | Volusia | Frequent High | 31.1 | 30.2 | Seasonally Flooded | – | – |
|  |  | Average | 29.5 | 28.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 28.0 | 27.0 | Semipermanently Flooded | – | – |
| (hhhh) Scoggin | Volusia | Frequent High | 35.0 | 34.0 | Seasonally Flooded | – | – |
|  |  | Average | 34.1 | 33.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.7 | 31.7 | Semipermanently Flooded | – | – |
| (iiii) Shaw | Volusia | Frequent High | 36.7 | 35.7 | – | 30 | 3 |
|  |  | Average | 35.4 | 34.4 | – | 180 | 1.7 |
|  |  | Frequent Low | 33.7 | 32.7 | – | 120 | 3 |
| (jjjj) Silver | Putnam | Frequent High | 36.8 | 35.8 | Seasonally Flooded | – | – |
|  |  | Average | 35.1 | 34.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.7 | 32.7 | Semipermanently Flooded | – | – |
| (kkkk) Smith | Marion | Frequent High | 54.6 | 53.6 | Temporarily Flooded | – | – |
|  |  | Average | 51.4 | 50.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 50.0 | 49.0 | Semipermanently Flooded | – | – |
| (llll) South | Brevard | Frequent High | 16.7 | 15.5 | Temporarily Flooded | – | – |
|  |  | Average | 15.3 | 14.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 13.8 | 12.6 | Semipermanently Flooded | – | – |
| (mmmm) South Como Park | Putnam | Frequent High | 38.1 | 37.2 | Seasonally Flooded | – | – |
|  |  | Average | 36.7 | 35.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 35.3 | 34.4 | Semipermanently Flooded | – | – |
| (nnnn) Star | Putnam | Frequent High | 77.5 | 76.5 | Seasonally Flooded | – | – |
|  |  | Average | 75.4 | 74.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 74.0 | 73.0 | Semipermanently Flooded | – | – |
| (oooo) Stella | Putnam | Frequent High | 39.4 | 38.4 | Seasonally Flooded | – | – |
|  |  | Average | 38.6 | 37.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.2 | 36.1 | Semipermanently Flooded | – | – |
| (pppp) Sunset | Lake | Frequent High | 85.9 | 84.9 | Temporarily Flooded | – | – |
|  |  | Average | 83.5 | 82.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 81.0 | 79.9 | Semipermanently Flooded | – | – |
| (qqqq) Swan | Putnam | Frequent High | 93.0 | 91.9 | Temporarily Flooded | – | – |
|  |  | Average | 90.3 | 89.2 | Typically Saturated | – | – |
| (rrrr) Sylvan | Seminole | Frequent High | 40.4 | 39.5 | Seasonally Flooded | – | – |
|  |  | Average | 38.9 | 38.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 37.5 | 36.5 | Semipermanently Flooded | – | – |
| (ssss) Tarhoe | Putnam | Frequent High | 36.6 | 35.7 |  | 30 | 3 |
|  |  | Average | 35.1 | 34.2 |  | 180 | 1.7 |
|  |  | Frequent Low | 33.0 | 32.0 |  | 120 | 5 |
| (tttt) Three Island Lakes | Volusia | Frequent High | 23.7 | 22.7 | – | 30 | 5 |
|  |  | Frequent Low | 19.4 | 18.4 | – | 120 | 10 |
| (uuuu) Trone | Putnam | Frequent High | 37.1 | 36.2 | – | 30 | 3 |
|  |  | Average | 35.4 | 34.5 | – | 180 | 1.7 |
|  |  | Frequent Low | 32.8 | 31.9 | – | 120 | 5 |
| (vvvv) Trout | Volusia | Frequent High | 23.3 | 22.3 | Seasonally Flooded | – | – |
|  |  | Average | 20.9 | 20.0 | Typically Saturated | – | – |
|  |  | Frequent Low | 17.7 | 16.8 | Semipermanently Flooded | – | – |
| (wwww) Tuscawilla | Alachua | Frequent High | 77.6 | 76.4 | Seasonally Flooded | – | – |
|  |  | Average | 74.6 | 73.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 73.2 | 72.0 | Semipermanently Flooded | – | – |
| (xxxx) Upper Lake Louise | Volusia | Frequent High | 35.3 | 34.4 | Seasonally Flooded | – | – |
|  |  | Average | 34.6 | 33.6 | Typically Saturated | – | – |
|  |  | Frequent Low | 33.2 | 32.2 | Semipermanently Flooded | – | – |
| (yyyy) Washington | Brevard | Frequent High | 15.6 | 14.2 | Seasonally Flooded | – | – |
|  |  | Average | 14.2 | 12.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 12.8 | 11.4 | Semipermanently Flooded | – | – |
| (zzzz) Wauberg | Alachua | Frequent High | 67.4 | 66.4 | Seasonally Flooded | – | – |
|  |  | Average | 67.1 | 66.1 | Typically Saturated | – | – |
|  |  | Frequent Low | 65.6 | 64.7 | Semipermanently Flooded | – | – |
| (aaaaa) Weir | Marion | Frequent High | 57.2 | 56.2 | Seasonally Flooded | – | – |
|  |  | Average | 56.4 | 55.4 | Typically Saturated | – | – |
|  |  | Frequent Low | 54.9 | 53.9 | Semipermanently Flooded | – | – |
| (bbbbb) Winnemisett | Volusia | Frequent High | 59.5 | 58.5 | Seasonally Flooded | – | – |
|  |  | Average | 57.8 | 56.8 | Typically Saturated | – | – |
|  |  | Frequent Low | 56.0 | 55.0 | Semipermanently Flooded | – | – |
| (ccccc) Winona | Volusia | Frequent High | 36.1 | 35.1 | Seasonally Flooded | – | – |
|  |  | Average | 33.5 | 32.5 | Typically Saturated | – | – |
|  |  | Frequent Low | 32.0 | 31.0 | Semipermanently Flooded | – | – |

(5) The following minimum levels are established for Blue Cypress Water Management Area (BCWMA):

(a) The minimum average level, calculated as the long term mean of BCWMA water levels, is 24.0 feet NGVD (22.5 feet NAVD). Water levels shall be at or above this level at least 75% of time over the long term.

(b) The minimum frequent low is 23.0 feet NGVD (21.5 feet NAVD). The daily BCWMA water level shall not fall to this level or below more often than once every 2.5 years over the long term.

(c) The minimum infrequent low is 22.5 feet NGVD (21.0 feet NAVD). The BCWMA water level shall not fall to this level or below for 60 continuous days more frequently than once every 10 years over the long term.

(6)(a) The minimum spring flow for De Leon Springs in Volusia County is 25.6 Cubic feet per second (CFS).

(b) The minimum spring flow for De Leon Springs is a mean annual flow based on the baseline flow-time series data from 1965 to 2015 under the 2010-pumping condition (Baseline Flow), which data is incorporated by reference and available at http://www.flrules.org/Gateway/reference.asp?No=Ref-07783, and upon request from the St. Johns River Water Management District, 4049 Reid Street, Palatka, FL 32177-2529. The minimum spring flow for De Leon Springs is equal to the Baseline Flow as defined above.

(7) Ground or surface water withdrawals or surface water works must not cause the infrequent high or frequent high surface water flows and levels to occur less frequently or for at lesser duration than stated. Ground or surface water withdrawals or surface water works must not cause the minimum average, frequent low, or infrequent low surface water levels and flows to occur more frequently or for longer durations than stated.

(8) Determining Ongoing Status – The purpose of this subsection is to provide the approach to determine whether the flow(s) and/or level(s) of a specific MFL water body is/are below or projected to fall below the rule-specified MFL criteria (along with the associated evaluations necessary to make such a determination). This status assessment is independent from and not a determination of consumptive use permit compliance or environmental resource permit compliance. Permit compliance is a regulatory function that is not considered to be within the scope of this subsection.

(a) A screening level analysis, which incorporates change in rainfall trend and uncertainty in MFLs, will be performed approximately every five years to monitor the status of an adopted MFL, as well as when permit applications are considered that may impact an MFL. If the screening level analysis shows that the MFL is being met based on the rainfall-adjusted flows or levels, then no further actions are required beyond continued monitoring. If the analysis shows that the MFL is not being met, or is trending toward not being met based on the rainfall-adjusted flows and levels, the District will conduct a cause and effect analysis to independently evaluate the potential impacts of various stressors on the MFL water body being assessed. Factors other than consumptive uses of water (e.g., long-term drought) can cause the flow or level of a surface watercourse, aquifer, surface water, or spring to drop below an adopted minimum flow or level. Factors to be considered in the determination of causation include, but are not limited to:

1. Rainfall or other climatic variables;

2. Consumptive use;

3. Land use changes or development;

4. Surface water drainage;

5. Geology/hydromorphology (e.g., sinkhole formation);

6. Water levels/flows in other appropriate water resources (e.g., nearby wells, lakes, streams, wetlands); and,

7. Ecological assessment information.

(b) The types of tools used in the causation analysis include, but are not limited to:

1. Double-mass analyses;

2. Rainfall/flow statistical analysis or flow regression;

3. Stage/duration/frequency analysis;

4. Modeling (regional, groundwater, ecological or water budget models); and,

5. Ecological tools.

Based on the causation analysis, the District will assess existing MFL criteria and any associated recovery and prevention strategies to determine the effectiveness of the strategies in recovering from or preventing significant harm to the water body.

(c) This subsection shall not apply within the Central Florida Water Initiative Area, as defined in paragraph 373.0465(2)(a), F.S. (2016).

Rulemaking Authority 373.044, 373.113 FS. Law Implemented 373.042, 373.0421, 373.103, 373.415 FS. History–New 9-16-92, Amended 8-17-94, 6-8-95, 1-17-96, 8-20-96, 10-20-96, 11-4-98, 6-27-00, 2-13-01, 3-19-02, 5-12-03, 11-10-03, 1-12-04, 2-1-06, 12-3-06, 5-10-07, 5-24-07, 1-11-10, 8-22-13, 4-3-14, 11-25-14, 12-31-14, 1-31-16 (4)(aaa), 1-31-16 (4)(ssss), 2-1-17 (4)(w), 2-1-17 (6), (8).