- 62-304.406 Aucilla River Basin TMDLs.

(1) Wacissa River. The nutrient Total Maximum Daily Load (TMDL) for the Wacissa River is an in-stream monthly arithmetic mean concentration of 0.20 mg/L nitrate, and is allocated as follows:

(a) The wasteload allocation (WLA) for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Separate Storm Sewer System (MS4) Permitting Program is a 39% reduction of nitrate based on average concentrations from the 2005-2015 period;

(c) The Load Allocation (LA) for nonpoint sources is a 39% reduction of nitrate based on average loads from the 2005-2015 period; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for nitrate has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(2) Wacissa Springs. The nutrient TMDL for Wacissa Springs is a monthly arithmetic mean of 0.24 mg/L nitrate at the spring vent, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 38% reduction of nitrate based on average concentrations from the 2005-2015 period;

(c) The Load Allocation (LA) for nonpoint sources is a 38% reduction of nitrate based on average loads from the 2005-2015 period; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for nitrate has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 5-30-17.