61C-5.011 Alterations to Electric and Hydraulic Elevators and Escalators.

(1) In addition to the alterations set forth in Rule 1003.3 and Rule 1006.3, ASME A17.1, 1996, the following alterations require, in addition to a construction permit, that inspections and tests be performed to determine conformance with the ASME A17.1, 1996, rules cited below:

ALTERATIONS	Electric Elevators	Hydraulic Elevators

(a) Addition of elevator to existing hoistway (new1201.1b1201.1b

installation)

(b) Brake (replacement of existing drive machine brake208-210.8–

by a new brake)

(c) Buffer (addition of oil buffer)1202.21203.2b

(d) Driving machine (replacement of)1202.9a1203.3

(e) Freight elevator converted to passenger service1202.8a1203.2h

(f) Rope, replacement in size or number of ropes1200.4d1200.4d

(g) Sheave, driving machine (replacement in size)1202.9a1202.9a

(2) The following alterations require, in addition to a construction permit, that inspections be performed to determine conformance with the ASME A17.1, 1996, rule cited below:

ALTERATIONS	Electric Elevators	Hydraulic Elevators

(a) Access Switch (addition of)1201.11d1203.1k

(b) Automatic transfer device (addition of)12051205

(c) Car, door or gate (addition of car door or gate1202.51203.2e

electric contacts)

(d) Car enclosure1202.51203.2e

(e) Car leveling device (addition of) and (trucking1202.12b1203.8b

device)

(f) Control1202.12e1203.8f

(g) Control equipment1202.121203.8

(h) Controller (existing controller w/new) (excluding1202.12d1203.8e

dispatching device)

(i) Counterweight (change of)1202.31203.2c

(j) Increase in travel (or decrease)1202.10a1203.4a

(k) Door, hoistway (replacement of all hoistway doors)1201.101203.1j

(l) Escalator, relocation of1207–

(m) Escalator, skirt switches (addition of safety device)805–

(n) Freight elevator permitted to carry passengers1202.8c1203.2j

(o) Guide rails (change in type or size)1202.11203.2a

(p) Hoistway door, power operation of (addition of)1201.121203.1m

(q) Hoistway door locking device (addition of)1201.111203.1k

(r) Operation, change in type of1202.12f1203.8g

(s) Platform, car (complete replacement of)1202.4a1203.2d

(t) Roller guide shoe, counterweight and car (addition1202.31202.3

of)

(u) Rope equalizer (addition of)1202.14c1203.9c

(v) Rope fastening device, auxiliary (addition of)1202.14d1202.14d

(w) Tank (replacement of) (with different capacity)– 1203.6

(x) Top of car operating device (addition of)1202.12a1203.8a

Specific Authority 399.02 FS. Law Implemented 399.02 FS. History–New 2-11-92, Formerly 7C-5.011, Amended 8-1-96, 10-4-00.