- 61C-5.001 Safety Standards.

(1) Applicability. The safety standards adopted herein shall apply to all elevators covered by Chapter 399, F.S., as follows:

(a) The installation, alteration, relocation, and change in classification must comply with Chapter 399, F.S., as well as the edition of the Florida Building Code or the Elevator Safety Code, in effect on the date of receipt of application for the permit to install, alter, relocate, or change in classification.

(b) The maintenance, repair and replacement must comply with the maintenance, repair and replacement requirements of the most recent adopted codes and the codes under which the elevator was installed, altered, relocated, or changed classification, as applicable.

(c) Inspections and periodic test witnessing must be performed in accordance with the inspection and test requirements of the most recent adopted codes.

(2) The following safety standards for elevators, escalators, dumbwaiters, and moving walks are hereby adopted and incorporated by reference:

(a) ASME A17.1-2007, Safety Code for Elevators and Escalators, including ASME A17.1a-2008, Addenda to ASME A17.1-2007, and A17.1b-2009, Addenda to ASME A17.1-2007, effective March 15, 2012;

(b) ASME A17.1-2013, Safety Code for Elevators and Escalators, effective December 31, 2017. The following parts of ASME A17.1-2013, Safety Code for Elevator and Escalators, are hereby excluded and not incorporated herein by reference:

1. Parts 5.3, 8.10.5.2, and 8.11.5.2 relating to private residence elevators.

2. Parts 5.4, 8.6.7.4 and 8.10.5.2 relating to private residence inclined elevators.

3. Parts 5.8, 8.6.7.8, 8.7.5.8, 8.10.5.8, and 8.11.5.8 relating to marine elevators.

4. Part 5.9, 8.6.7.9, and 8.7.5.9 relating to mine elevators.

5. Part 5.11, 8.6.7.11, 8.10.5.14, and 8.11.5.14 relating to wind turbine tower elevators.

6. Part 5.12, 8.6.7.12, 8.7.5.10, 8.10.5.15, and 8.11.5.15 relating to outside emergency elevators.

(c) ASME A17.3-1996, Safety Code for Existing Elevators and Escalators, effective March 15, 2012.

(3) The following safety standard for stairway chairlifts and inclined or vertical wheelchair lifts is hereby adopted and incorporated by reference:

ASME A18.1-2008, Safety Standard for Platform Lifts and Stairway Chairlifts, effective March 15, 2012.

(4) “ASME” is the recognized abbreviation for the American Society of Mechanical Engineers, and generally the abbreviation is used in these rules in identifying the publications of the association, which are copyrighted. It would be a violation of federal copyright law to post the materials incorporated in this rule on the Internet for public viewing. Accordingly, the public may purchase a copy of any ASME publication by writing ASME, P.O. Box 2300, Fairfield, NJ 07007-2300, or at www.asme.org. In addition, copies of the incorporated materials are available for viewing during regular business hours at the Florida Department of Business and Professional Regulation, Division of Hotels and Restaurants, located at 2601 Blair Stone Road, Tallahassee, Florida 32399, and the Florida Department of State located at R.A. Gray Building, 500 South Bronough Street, Tallahassee, FL 32399-0250.

Rulemaking Authority 399.02, 399.10 FS. Law Implemented 399.02, 399.03 FS. History–Amended 10-20-63, 4-20-64, 11-17-73, 12-20-73, Revised 3-22-74, Amended 12-18-74, 8-21-79, 8-1-82, 9-19-84, Formerly 7C-5.01, Amended 11-1-87, 10-31-88, 6-12-89, 9-10-89, 10-3-90, 5-12-91, 6-23-91, 8-9-91, 8-27-92, Formerly 7C-5.001, Amended 2-2-94, 8-1-96, 1-1-98, 10-4-00, 4-2-08, 3-15-12, 8-21-12, 1-11-18.