- 5J-20.050 Installation of Unvented Room Heaters.

The following exceptions to the requirements of Section 10.22 of NFPA 54 as incorporated by reference in rule 5J-20.002, F.A.C., are adopted with regard to the installation of unvented room heaters:

(1) One listed, wall-mounted, unvented room heater, equipped with an oxygen depletion safety shutoff system may be installed in a bathroom, provided that the input rating shall not exceed 6,000 BTU (1760 W) per hour and combustion and ventilation air are provided as specified by Section 10.1.2 of NFPA 54 as incorporated by reference in rule 5J-20.002, F.A.C.

(2) One listed wall-mounted, unvented room heater equipped with an oxygen depletion safety shutoff system may be installed in a bedroom, provided that the input rating shall not exceed 10,000 BTU (2930 W) per hour and combustion and ventilation air are provided as specified by Section 10.1.2 of NFPA 54 as incorporated by reference in rule 5J-20.002, F.A.C.

Rulemaking Authority 527.06 FS. Law Implemented 527.06 FS. History–New 1-24-95, Amended 1-29-06, 8-2-07, Formerly 5F-11.050, Amended 6-27-18.