61G15-34.005 Design of Heat and Energy Transfer Systems.

(1) Heat and Energy Transfer Systems are those systems that are designed to transfer heat or energy from one fluid to another, as part of an industrial, commercial, or cogeneration process. Items to be included in the design of these systems are fluid type and characteristics, fluid flow quantities, fluid pressure head, pump type, fan type, heat exchanger type, piping specification, ductwork specification, and process type.

(2) Mechanical documents applicable to heat and energy transfer systems shall at a minimum include the following:

(a) Equipment schedule for each piece of mechanical equipment including fluid type and characteristics, system pressure head and flow requirements, and electrical or fuel requirements.

(b) Floor plans, site plans, and building and systems elevations.

(c) Process flow diagrams with pipe or ductwork sizes.

(d) System piping or ductwork layout.

(e) Specific system design requirements to allow independent project review.

(f) List of NFPA, ASHRAE, ASME, ANSI or other applicable design standards and requirements.

(g) Instrumentation and Control Diagrams and sequence of operation.

(h) Required fire protection systems and devices.

Specific Authority 471.008, 471.033(2) FS. Law Implemented 471.033 FS. History–New 11-16-94.