- 62B-33.0086 Electronic Submittals.

An electronic application (“Application for a Permit for Construction Seaward of the CCCL or 50-Foot Setback” – DEP Form 73-100), adopted and incorporated by reference in subsection 62B-33.008(1), F.A.C., must be submitted to the Department via email at CCCL@dep.state.fl.us, together with the appropriate fee listed in rule 62B-33.0085, F.A.C., to obtain a CCCL permit. Information saved on an electronic storage device can be mailed to 2600 Blair Stone Road, MS 3522, Tallahassee, Florida 32399-2400. If the applicant provides a written statement that the electronic submittal requirements pose a substantial technical or financial hardship, the electronic submittal requirements may be waived, and the documents may be submitted to the above address. When submitting the electronic information, the applicant shall use the following standards:

(1) Submit the “Application for a Permit for Construction Seaward of the CCCL or 50-Foot Setback” (DEP Form 73-100), adopted and incorporated by reference in subsection 62B-33.008(1), F.A.C., in a searchable electronic format. The Department encourages property owners to complete their application online at the link below and submit the completed application via email following the instructions given on the website; https://floridadep.gov/water/coastal-construction-control-line/content/coastal-construction-control-line-cccl-forms;

(2) Submit geotechnical data in electronic file format;

(3) Submit Geographic Information System (GIS) data (.shp, .dwg or .dxf or other GIS compatible format) that has been projected into the appropriate Florida State Plane coordinate system. Metadata shall be included using the Federal Geographic Data Committee (FGDC) standard. Metadata may be submitted in .xml format if included as part of the shapefile, in a .txt file if sent separately, or other convertible file;

(4) Use electronic signatures, pursuant to chapter 668, F.S.;

(5) Use electronic professional certifications, pursuant to section 471.025, F.S.;

(6) Appendices or attachments shall be indexed to the CCCL Application; and

(7) Submit the application and cover letter by email; or to a web page set up by the Department to receive electronic applications; or on an electronic storage device mailed to the Department of Environmental Protection, 2600 Blair Stone Road, Mail Station 3522, Tallahassee, Florida 32399-2400.

Rulemaking Authority 20.255(8), 161.052(11), 161.053(20), 668 FS. Law Implemented 20.255(8), 161.052(2), 161.053(2), (4), 403.061(44), 471.025, 668 FS. History–New 11-28-18.