- 14-86.004 Permit Application Procedure.

(1) An applicant shall submit a Drainage Connection Permit, Form 850-040-06 (12/18), incorporated herein by reference https://www.flrules.org/Gateway/reference.asp?No=Ref-10197 and available for electronic submittal through https://osp.fdot.gov.

(2) Each completed Drainage Connection Permit package shall be accompanied by:

(a) A location map, included in the construction plans, sufficient to show the location of the improvement and any drainage connection to the Department’s right-of-way, and shall include the state highway number, county, city, and section, range, and township.

(b) A grading plan drawn to scale showing pre-improvement and post-improvement site conditions including all pervious and impervious surfaces, land contours, spot elevations, and all drainage facilities of the Department and of the adjacent property. The bench mark datum for the plans shall be noted on the plans. Contour information shall extend 50 feet beyond the property boundaries or be sufficient to clearly define the portion of the watershed which drains through the property to the Department’s right-of-way.

(c) Photographs which accurately depict pre-improvement and present conditions.

(d) Soil borings and water table data and, where percolation or infiltration is utilized in the design, appropriate percolation test methodology and results.

(e) Computations as required by subsection 14-86.003(2), F.A.C.

(f) The Drainage Connection Certification, Part 2 of the permit must be certified by a Licensed Professional that the complete set of plans and computations comply with either paragraph 14-86.003(2)(a) or 14-86.003(2)(b), F.A.C.

(3) Improvements which otherwise meet the criteria of subparagraphs 14-86.003(3)(f)1. and 14-86.003(3)(f)4., F.A.C., but which create or alter a drainage connection to the Department’s right-of-way, will not require submittal of the information required by paragraphs 14-86.004(2)(d) through (f), F.A.C., but will otherwise require the submittal of all other required information.

(4) The Department recognizes that regulatory and permitting programs exist or may be developed by local units of government, and state or federal agencies which may overlap with the requirements of this rule chapter. In order to avoid duplication the Department will:

(a) In lieu of the requirements in rule 14-86.003 and subsection 14-86.004(2), F.A.C., accept a permit that accomplishes the purposes of this rule chapter so long as the permit is issued by a governmental entity with specific stormwater management authority and is based on requirements equal to or more stringent than those in rule 14-86.003, F.A.C.; or

(b) Accept any form, plans, specifications, drawings, calculations, or other data developed to support an application for a permit required by a governmental entity, pursuant to any rule which establishes requirements equal to or more stringent than rule 14-86.003, F.A.C.

(5) Once approved by the Department, the drainage connection application and supporting documents become the Drainage Connection Permit.

Rulemaking Authority 334.044(2), (15) FS. Law Implemented 334.044(15) FS. History–New 11-12-86, Amended 1-20-09, 2-6-19.